3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220029

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2260 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 21wk49 Implementation , at step 9-10 there is an extra DRB3 configuration. this change is introduced as part of R5s211507 but this CR is withdrawn . When DRB2 is released as part of step 13-14 , DRB3 is present along with SRB2 which is satisfying the core spec requirement 38.331 section 5.3.1.1 Aconfiguration with SRB2 without DRB or with DRB without SRB2 is not supported(i.e., SRB2 and at least one DRB must be configured in the same RRCReconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection). For IAB-MT, a configuration with SRB2 withoutDRB is supported.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extra Drb3 configuration has to be removed from steps 9-10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_TC_8_1_2_1_1_NR5GC_TestBody	2

# Corrections required

## Correction to function f_TC_8_1_2_1_1_NR5GC_TestBody

| Function name | f_TC_8_1_2_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | In 21wk49 Implementation , at step 9-10 there is an extra DRB3 configuration. this change is introduced as part of R5s211507 but this CR is withdrawn . When DRB2 is released as part of step 13-14 , DRB3 is present along with SRB2 which is satisfying the core spec requirement 38.331 section 5.3.1.1 Aconfiguration with SRB2 without DRB or with DRB without SRB2 is not supported(i.e., SRB2 and at least one DRB must be configured in the same RRCReconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection). For IAB-MT, a configuration with SRB2 withoutDRB is supported. |
| Summary of change | Extra Drb3 configuration has to be removed from steps 9-10 |
| TTCN module | develop\NR5GC\8_1_2\RRC_Reconfiguration_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 R5-217812 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var NR_RadioBearerList_Type v_SS_Config; // @siclog "Step 1" siclog@ // SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 2-4" siclog@ // UE transmits an RRCSetupRequest message // SS transmits an RRCSetup message // The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { // @sic R5s200571 Ch. 4 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Non-IMS DRBs configured"); } // @siclog "Step 5" siclog@ // SS transmits a SecurityModeCommand message to activate AS security. v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedMsg.SecurityProtection.NasCount); f_NR_Security_Set(v_SecurityParams); // Read PDCP SQN for SRB1 and calculate Ciphering Activation times v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // send SecurityModeCommand on SRB1. Follow on flag is set to TRUE to ensure the RRCConnectionReconfiguration message is sent on the same TTI and therefore SRB.send(cas_NR_SRB1_RrcPdu_REQ_FollowOnFlag(nr_Cell1, -, // cs_TimingInfo_Now is needed for FollowOn = true to work cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm)))); // Sent before it sends SecurityModeComplete message. // @siclog "Step 6" siclog@ // Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_CellGroupConfig.mac_CellGroupConfig := omit; v_CellGroupConfig.physicalCellGroupConfig := omit; v_CellGroupConfig.spCellConfig := omit; f_ReconfigureDRBs( nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}, // @sic Baseline Moving sic@, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, -, false, -, -, v_SecurityParams); // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.1.1: Step 7. SecurityModeComplete Received from UE"); // @siclog "Step 8" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Steps 9/10 - Void" siclog@ // Must release and re-configure the SS for SRB2 and DRB1 to re-establish the RLC f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2)}); f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBId))}); // DRB1 reconfigured during reconfiguration f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_SRB2_Config}); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, { cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBId, cs_38508_PDCP_Config),cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, tsc_NR_DRB3, cs_38508_PDCP_Config)}); // @sic R5s211507 sic@ // Build RLC-BearerConfig for SRB2 and DRB v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (nr_Cell1, {v_DRBId}, true); // There should be 2 entries and both of them need to update the re-establishRLC flag v_CellGroupConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_CellGroupConfig.rlc_BearerToAddModList[1].reestablishRLC := true_; // @siclog "Steps 11-12" siclog@ // The SS transmits an RRCReconfiguration message to modify SRB and DRB configuration. // Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1, v_RadioBearerConfig, {v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier v_CellGroupConfig, omit, tsc_NR_RbId_SRB1); // @sic R5s190388 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @sic R5s190936 sic@ // Build NAS command v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand (int2oct(valueof(v_DRBInfo.PDUSessionId), 1)); // @sic R5s190936, R5s200564 sic@ // Build RRC command v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId}); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity}); if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); } // @siclog "Step 12A" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, Internet_DNN))), dedicatedBearer, v_DRBId, "Step 12A"); // @sic R5-197057 R5s200341 Ch. 1 R5s200571 Ch. 4 sic@ // @siclog "Step 12A1/12A4" siclog@ //The SS transmits an OPEN UE TEST LOOP Message. The UE transmits an OPEN UE TEST LOOP COMPLETE Message. // The SS transmits DEACTIVATE TEST MODE message. The UE transmits DEACTIVATE TEST MODE COMPLETE message. if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); } // @siclog "Steps 12B - 12C" siclog@ // Cause the UE to request establishment of PDU session. // The PDU session establishment procedure as specified in TS 38.508-1 subclause 4.5A.2 takes place. f_NR5GC_TriggerSinglePDUSession(nr_Cell1, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5s211349 sic@ // @siclog "Steps 13-14" siclog@ // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND // Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1, v_RadioBearerConfig, {v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }, tsc_NR_RbId_SRB1, -, false); // @sic R5s190388 sic@ // @siclog "Step 15 - Void" siclog@ // @siclog "Step 16" siclog@ // The UE transmits an ULInformationTransfer message and a UL NAS TRASPORT containing PDU SESSION RELEASE COMPLETE. interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) {} // The UE transmits an ULInformationTransfer message and a UL NAS TRASPORT containing PDU SESSION RELEASE COMPLETE. []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{} } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, int2oct(valueof(v_DRBInfo.PDUSessionId), 1))) { // @sic R5s200564 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15"); } }//end of f_TC_8_1_2_1_1_NR5GC_TestBody

After Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 R5-217812 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var NR_RadioBearerList_Type v_SS_Config; // @siclog "Step 1" siclog@ // SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 2-4" siclog@ // UE transmits an RRCSetupRequest message // SS transmits an RRCSetup message // The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { // @sic R5s200571 Ch. 4 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Non-IMS DRBs configured"); } // @siclog "Step 5" siclog@ // SS transmits a SecurityModeCommand message to activate AS security. v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedMsg.SecurityProtection.NasCount); f_NR_Security_Set(v_SecurityParams); // Read PDCP SQN for SRB1 and calculate Ciphering Activation times v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // send SecurityModeCommand on SRB1. Follow on flag is set to TRUE to ensure the RRCConnectionReconfiguration message is sent on the same TTI and therefore SRB.send(cas_NR_SRB1_RrcPdu_REQ_FollowOnFlag(nr_Cell1, -, // cs_TimingInfo_Now is needed for FollowOn = true to work cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm)))); // Sent before it sends SecurityModeComplete message. // @siclog "Step 6" siclog@ // Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_CellGroupConfig.mac_CellGroupConfig := omit; v_CellGroupConfig.physicalCellGroupConfig := omit; v_CellGroupConfig.spCellConfig := omit; f_ReconfigureDRBs( nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}, // @sic Baseline Moving sic@, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, -, false, -, -, v_SecurityParams); // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.1.1: Step 7. SecurityModeComplete Received from UE"); // @siclog "Step 8" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Steps 9/10 - Void" siclog@ // Must release and re-configure the SS for SRB2 and DRB1 to re-establish the RLC f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2)}); f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBId))}); // DRB1 reconfigured during reconfiguration f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_SRB2_Config}); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, { cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBId, cs_38508_PDCP_Config)/*,cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, tsc_NR_DRB3, cs_38508_PDCP_Config)*/}); // @sic R5s211507 sic@ // Build RLC-BearerConfig for SRB2 and DRB v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (nr_Cell1, {v_DRBId}, true); // There should be 2 entries and both of them need to update the re-establishRLC flag v_CellGroupConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_CellGroupConfig.rlc_BearerToAddModList[1].reestablishRLC := true_; // @siclog "Steps 11-12" siclog@ // The SS transmits an RRCReconfiguration message to modify SRB and DRB configuration. // Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1, v_RadioBearerConfig, {v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier v_CellGroupConfig, omit, tsc_NR_RbId_SRB1); // @sic R5s190388 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @sic R5s190936 sic@ // Build NAS command v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand (int2oct(valueof(v_DRBInfo.PDUSessionId), 1)); // @sic R5s190936, R5s200564 sic@ // Build RRC command v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId}); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity}); if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); } // @siclog "Step 12A" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, Internet_DNN))), dedicatedBearer, v_DRBId, "Step 12A"); // @sic R5-197057 R5s200341 Ch. 1 R5s200571 Ch. 4 sic@ // @siclog "Step 12A1/12A4" siclog@ //The SS transmits an OPEN UE TEST LOOP Message. The UE transmits an OPEN UE TEST LOOP COMPLETE Message. // The SS transmits DEACTIVATE TEST MODE message. The UE transmits DEACTIVATE TEST MODE COMPLETE message. if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); } // @siclog "Steps 12B - 12C" siclog@ // Cause the UE to request establishment of PDU session. // The PDU session establishment procedure as specified in TS 38.508-1 subclause 4.5A.2 takes place. f_NR5GC_TriggerSinglePDUSession(nr_Cell1, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5s211349 sic@ // @siclog "Steps 13-14" siclog@ // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND // Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1, v_RadioBearerConfig, {v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }, tsc_NR_RbId_SRB1, -, false); // @sic R5s190388 sic@ // @siclog "Step 15 - Void" siclog@ // @siclog "Step 16" siclog@ // The UE transmits an ULInformationTransfer message and a UL NAS TRASPORT containing PDU SESSION RELEASE COMPLETE. interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) {} // The UE transmits an ULInformationTransfer message and a UL NAS TRASPORT containing PDU SESSION RELEASE COMPLETE. []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{} } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, int2oct(valueof(v_DRBInfo.PDUSessionId), 1))) { // @sic R5s200564 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15"); } }//end of f_TC_8_1_2_1_1_NR5GC_TestBody