3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220040

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2264 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_EmergencyCallWithRegistration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the function f_NR5GC_EmergencyCallWithRegistration, current TTCN implementaion activates a default handler by calling function a_NR5GC_AdditionalPDUSessionEstablishmentReq which can be used to handle the parallely received PDU Session Establishment/Modification REQs. This takes care of the case when the messages can come on SRB1 or SRB2 in any order. However, the TTCN does not handle the incoming rrcReconfigurationComplete message at all, because the call to to f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, -, -, -, -, -, -, false); specifically avoids that. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling to f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, -, -, -, -, -, -, false);, called to f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); Please see below for detailed change description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	4

1.1	Correction to function f_NR5GC_EmergencyCallWithRegistration	4

# Corrections required

## Correction to function f_NR5GC_EmergencyCallWithRegistration

| Function name | f_NR5GC_EmergencyCallWithRegistration |
| --- | --- |
| Reason for change | In the function f_NR5GC_EmergencyCallWithRegistration, current TTCN implementaion activates a default handler by calling function a_NR5GC_AdditionalPDUSessionEstablishmentReq which can be used to handle the parallely received PDU Session Establishment/Modification REQs. This takes care of the case when the messages can come on SRB1 or SRB2 in any order. However, the TTCN does not handle the incoming rrcReconfigurationComplete message at all, because the call to to f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, -, -, -, -, -, -, false); specifically avoids that. This needs to be addressed. |
| Summary of change | Instead of calling to f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, -, -, -, -, -, -, false);, called to f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); Please see below for detailed change description |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_EmergencyCallWithRegistration(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_PDUEstAlreadyRxd := 0; var default v_MyDefaultVar := null; // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // @sic R5-217286 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfPDUSessionEstReq(); // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); // Steps 4-7 Configure SRB2 & DRB f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, -, -, -, -, -, -, false); // @sic R5s2111489 sic@ // then check if anything's been received if (f_NR5GC_MsgInDefault_GetNumOfPDUSessionEstReq() > 0) { v_PDUEstAlreadyRxd := 1; } // don't need this anymore so deactivate it before it needs reactivating @sic R5-217286 sic@ deactivate(v_MyDefaultVar); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, -, v_PDUEstAlreadyRxd); // @sic R5-217286 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); // Emergency call now active }

After Change:

function f_NR5GC_EmergencyCallWithRegistration(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_PDUEstAlreadyRxd := 0; var default v_MyDefaultVar := null; // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // @sic R5-217286 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfPDUSessionEstReq(); // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); // Steps 4-7 Configure SRB2 & DRB //WA#11_4_4 COMMENTED OUT f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, -, -, -, -, -, -, false); // @sic R5s2111489 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); // WA#11_4_4 // then check if anything's been received if (f_NR5GC_MsgInDefault_GetNumOfPDUSessionEstReq() > 0) { v_PDUEstAlreadyRxd := 1; } // don't need this anymore so deactivate it before it needs reactivating @sic R5-217286 sic@ deactivate(v_MyDefaultVar); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, -, v_PDUEstAlreadyRxd); // @sic R5-217286 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); // Emergency call now active }