3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220106

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2308 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for TCP functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Race conditions have been observed in some test cases following IMS de-regisration, while the “StopPDN” request (issued from either EUTRA or NR5GC PTC) is processed in the IP PTC. The TTCN is getting stuck during closing of 1 TCP socket because there is a clash between the SS closing the socket and the UE trying to close it at the same time. This should be handled in the function fl_TCP_Close() but there is a mismatch happening with the TCP close indication coming from the SS, because it gets matched against the IP connection details that were passed into the function from f_TcpServer_Stop(). This has the “remote” part of the connection set to “omit” but the indication from the SS has the “remote” connection details filled in. This causes a mismatch, and the message stays in the queue and blocks the TCP close cnf that comes shortly after. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The template parameters are changed such that only the “local” connection details are checked when receiving TCP CLOSE IND. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Execution of some test cases can intermittently hang, with IMS-enabled UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Common function fl_TCP_Close() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TCP_Close() |
| --- | --- |
| Reason for change | Race conditions have been observed in some test cases following IMS de-regisration, while the “StopPDN” request (issued from either EUTRA or NR5GC PTC) is processed in the IP PTC. The TTCN is getting stuck during closing of 1 TCP socket because there is a clash between the SS closing the socket and the UE trying to close it at the same time. This should be handled in the function fl_TCP_Close() but there is a mismatch happening with the TCP close indication coming from the SS, because it gets matched against the IP connection details that were passed into the function from f_TcpServer_Stop(). This has the “remote” part of the connection set to “omit” but the indication from the SS has the “remote” connection details filled in. This causes a mismatch, and the message stays in the queue and blocks the TCP close cnf that comes shortly after. |
| Summary of change | The template parameters are changed such that only the “local” connection details are checked when receiving TCP CLOSE IND. |
| TTCN module | TCP_Functions.ttcn |
| MCC160 Comment | Accepted in principle |

Before Change

function fl_TCP_Close(template (value) IP_Connection_Type p_IP_Connection, boolean p_WaitForCNF) runs on IP_PTC { IP_SOCK_CTRL.send(cs_TCP_CLOSE_REQ(p_IP_Connection)); if (p_WaitForCNF) { alt { [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(p_IP_Connection)) { } [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(?)) { repeat; } /* @sic R5s170043: pending CNF for previous call with p_WaitForCNF==false sic@ * NOTE: in general the CNFs in case of p_WaitForCNF==false may occur at any time */ [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(p_IP_Connection)) { repeat; } /* @sic R5s150270 change 1 sic@ */ } } }

After Change

function fl_TCP_Close(template (value) IP_Connection_Type p_IP_Connection, boolean p_WaitForCNF) runs on IP_PTC { IP_SOCK_CTRL.send(cs_TCP_CLOSE_REQ(p_IP_Connection)); if (p_WaitForCNF) { alt { [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(p_IP_Connection)) { } [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(?)) { repeat; } /* @sic R5s170043: pending CNF for previous call with p_WaitForCNF==false sic@ * NOTE: in general the CNFs in case of p_WaitForCNF==false may occur at any time */ [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(cr_TCP_ConnectionId (p_IP_Connection.Local, *))) { repeat; } } } }

## MCC160 Implementation

function fl_TCP_Close(template (value) IP_Connection_Type p_IP_Connection, boolean p_WaitForCNF) runs on IP_PTC { var template (present) IP_Connection_Type v_IP_Connection := p_IP_Connection; if (not isvalue(p_IP_Connection.Remote)) { v_IP_Connection.Remote := ?; } IP_SOCK_CTRL.send(cs_TCP_CLOSE_REQ(p_IP_Connection)); if (p_WaitForCNF) { alt { [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(p_IP_Connection)) { } [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(?)) { repeat; } /* @sic R5s170043: pending CNF for previous call with p_WaitForCNF==false sic@ * NOTE: in general the CNFs in case of p_WaitForCNF==false may occur at any time */ [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(v_IP_Connection)) { repeat; } } } }