3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220115

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2316 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As definition in chapter 7.1.0 of 38523-1. The default value of retxBSR-Timer is set to sf10240 for all layer 2 test cases. In the postamble of TC7.1.1.3.3 NR5GC, the DE-REGISTRATION REQUEST (UE ORIGINATING) has more bytes than the UL grant of RAR, So the UE need to trigger more PRACH procedures to apply for enough UL grant. But the duration is 10.240s > 5s, the Timer of postamble will be expired before the whole DE-REGISTRATION REQUEST is sent out. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Shorten the timer of retxBSR-Timer from sf10240 to sf1280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

# Corrections required

| Function name | cs_NR_BSR_Config_71133 |
| --- | --- |
| Reason for change | As definition in chapter 7.1.0 of 38523-1. The default value of retxBSR-Timer is set to sf10240 for all layer 2 test cases. In the postamble of TC7.1.1.3.3 NR5GC, the DE-REGISTRATION REQUEST (UE ORIGINATING) has more bytes than the UL grant of RAR, So the UE need to trigger more PRACH procedures to apply for enough UL grant. But the duration is 10.240s > 5s, the Timer of postamble will be expired before the whole DE-REGISTRATION REQUEST is sent out. |
| Summary of change | Shorten the timer of retxBSR-Timer from sf10240 to sf1280. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted as compliance to prose. |

Before change:

template (value) BSR_Config cs_NR_BSR_Config_71133 := cs_NR_BSR_ConfigDef(infinity_, sf10240, sf512);

After change:

template (value) BSR_Config cs_NR_BSR_Config_71133 := cs_NR_BSR_ConfigDef(infinity_, sf1280, sf512)