3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220117

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2318 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for TCP function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | some scenarios (including emergency call release) the SS and UE may be attempting to close multiple TCP sockets at the same time. In the TTCN the call to f_TCP_Close() is usually blocking waiting for a confirmation from SS. Any sockets closed by the UE during this time will generate an IP_SOCKET_IND which will then not be handled and will remain in the message queue, blocking the eventually IP_SOCKET_CNF and causing test case execution to hang. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an option in f_TCP_Close() to absorb any TCP close indication coming while waiting for TCP close cnf, even for a different TCP connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS test case execution can sometimes stall |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11,4,x and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TCP_Close() |
| --- | --- |
| Reason for change | In some scenarios (including emergency call release) the SS and UE may be attempting to close multiple TCP sockets at the same time. In the TTCN the call to f_TCP_Close() is usually blocking waiting for a confirmation from SS. Any sockets closed by the UE during this time will generate an IP_SOCKET_IND which will then not be handled and will remain in the message queue, blocking the eventually IP_SOCKET_CNF and causing test case execution to hang. |
| Summary of change | Add an option in f_TCP_Close() to absorb any TCP close indication coming while waiting for TCP close cnf, even for a different TCP connection. Note : This is an extension of the correction proposed under R5s220106. In that CR the main scenario being addressed was the SS and UE closing the same TCP socket at the same time. This CR is addressing the more general case that the UE may be concurrently closing a different TCP socket, but the result is the same and will cause test case execution to hang. So the correction of R5s220106 may no longer be required following the correction in the current CR. The “before” code below is based on R5s220106 implemented. |
| TTCN module | TCP_Functions.ttcn |
| MCC160 Comment | Accepted in principle: In general it is not a proper solution just to skip the TCP_CLOSE_INDs as resources need to be released. The following changes are proposed (see below) - while waiting for confirmation for TCP_CLOSE_CNF corresponding to the TCP_CLOSE_REQ any other ASPs shall be send back to the IPMUX_PTC where they are queued - After the TCP_CLOSE_CNF corresponding to the TCP_CLOSE_REQ has been the received a "flush" command is sent to the IPMUX_PTC which resends the queued ASPs |

Before Change

function fl_TCP_Close(template (value) IP_Connection_Type p_IP_Connection, boolean p_WaitForCNF) runs on IP_PTC { IP_SOCK_CTRL.send(cs_TCP_CLOSE_REQ(p_IP_Connection)); if (p_WaitForCNF) { alt { [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(p_IP_Connection)) { } [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(?)) { repeat; } /* @sic R5s170043: pending CNF for previous call with p_WaitForCNF==false sic@ * NOTE: in general the CNFs in case of p_WaitForCNF==false may occur at any time */ [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(cr_TCP_ConnectionId (p_IP_Connection.Local, *))) { repeat; } } } }

After Change

function fl_TCP_Close(template (value) IP_Connection_Type p_IP_Connection, boolean p_WaitForCNF) runs on IP_PTC { IP_SOCK_CTRL.send(cs_TCP_CLOSE_REQ(p_IP_Connection)); if (p_WaitForCNF) { alt { [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(p_IP_Connection)) { } [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(?)) { repeat; } /* @sic R5s170043: pending CNF for previous call with p_WaitForCNF==false sic@ * NOTE: in general the CNFs in case of p_WaitForCNF==false may occur at any time */ [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(cr_TCP_ConnectionId (p_IP_Connection.Local, *))) { repeat; } [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(?)) { repeat; } } } }

## MCC160 Implementation

function fl_TCP_Close(template (value) IP_Connection_Type p_IP_Connection, boolean p_WaitForCNF) runs on IP_PTC { var template (present) IP_Connection_Type v_IP_Connection := p_IP_Connection; var IP_SOCKET_IND v_IP_SOCKET_IND; if (not isvalue(p_IP_Connection.Remote)) { /* stop server; @sic R5s220106: if client tries in parallel to close connection(s) to the server, the remote socket is not omit in any corresponsding close indication sic@ */ v_IP_Connection.Remote := ?; } IP_SOCK_CTRL.send(cs_TCP_CLOSE_REQ(p_IP_Connection)); if (p_WaitForCNF) { alt { [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_CNF(p_IP_Connection)) { IP_SOCK_CTRL_QUEUE.send(crs_IP_FLUSH_IND); } [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(v_IP_Connection)) { repeat; } /* @sic R5s150270 change 1, R5s220106: v_IP_Connection instead of p_IP_Connection sic@ */ [] IP_SOCK_CTRL.receive(cr_IP_SOCKET_IND_AnyCtrl) -> value v_IP_SOCKET_IND { IP_SOCK_CTRL_QUEUE.send(v_IP_SOCKET_IND); } } } }

type record IP_SOCKET_CTRL_IND { IP_Connection_Type ConnectionId optional, IP_CtrlIndication_Type Ind optional };

template (value) IP_SOCKET_IND crs_IP_FLUSH_IND := { CTRL := { ConnectionId := omit, Ind := omit } };

type component IP_PTC { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ var IP_PTC_DRB_RoutingInfo_Type vc_IP_PTC_DRB_RoutingInfo; /* @sic R5s200490: vc_IP_PTC_DRB_RoutingInfo instead of vc_IP_PTC_DRB_RoutingTable sic@ */ var PDN_IP_AddrServer_Type vc_IP_PTC_PDN_AddrServer; var IP_PTC_IMS_Info_Type vc_IP_PTC_IMS_Info; var PDN_MappingTable_Type vc_IP_PTC_PDN_MappingTable; /* @sic R5s150648 Change 2.1 sic@ */ var IP_PTC_WLAN_Info_Type vc_IP_PTC_WLAN_Info; // ports connected to IMS: port IP_IMS_CTRL_PORT IMS_CTRL[tsc_NoOfIMSPDNs]; /* @sic R5s201177: tsc_NoOfIMSPDNs instead of tsc_NoOfPDNs sic@ */ port IP_IMS_SERVER_PORT IMS_Server[tsc_NoOfIMSPDNs]; /* @sic R5s201177: tsc_NoOfIMSPDNs instead of tsc_NoOfPDNs sic@ */ port IP_IMS_CLIENT_PORT IMS_Client[tsc_NoOfIMSPDNs]; /* @sic R5s201177: tsc_NoOfIMSPDNs instead of tsc_NoOfPDNs sic@ */ // coordination ports: port IP_PTC_CTRL_PORT EUTRA_CTRL; port IP_PTC_CTRL_PORT UTRAN_CTRL; port IP_PTC_CTRL_PORT GERAN_CTRL; port IP_PTC_CTRL_PORT WLAN_CTRL; /* @sic R5w160107 sic@ */ port IP_PTC_CTRL_PORT IPCAN_CTRL; /* @sic R5-203077: MCX IP test model - common IPCAN port independent from test model sic@ */ port IP_PTC_CTRL_PORT NR_CTRL; // coordination ports to HTTP_PTC port IP_HTTP_CTRL_PORT HTTPCTRL; port IP_HTTP_DATA_PORT HTTPDATA; // coordination port for sending of UDP packets to the UE (e.g. for OMA Push) port IP_UDP_HANDLER_PORT UDPHANDLER; // coordination port for generic TCP handler port IP_TCP_HANDLER_PORT TCPHANDLER; // ports to IP_MUX_PTC port IP_SOCKET_PORT IP_SOCK_DATA; /* @sic R5s150900 - MCC160 implementation sic@ */ port IP_SOCKET_PORT IP_SOCK_CTRL; /* @sic R5s150900 - MCC160 implementation sic@ */ port SOCKET_PORT_IPMUX IP_SOCK_CTRL_QUEUE; // system ports port IP_CONTROL_PORT IP_CTRL; port IPSEC_CONTROL_PORT IPSEC_CTRL; };

type component IPMUX_PTC { /* @sic R5s150900 - MCC160 implementation sic@ @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ // ports to IP_PTC port SOCKET_PORT_IPMUX IP_SOCK_DATA; port SOCKET_PORT_IPMUX IP_SOCK_CTRL; port SOCKET_PORT_IPMUX IP_SOCK_CTRL_QUEUE; // system port port IP_SOCKET_PORT IP_SOCK; };

type record of IP_SOCKET_IND IP_SOCKET_IND_List;

altstep a_IPMUX_PTC_HandlerUL_Queue(inout IP_SOCKET_IND_List p_IP_SOCKET_IND_List) runs on IPMUX_PTC { var IP_SOCKET_IND v_IP_SOCKET_IND; var integer i; [] IP_SOCK_CTRL_QUEUE.receive(crs_IP_FLUSH_IND) { for (i := 0; i < lengthof(p_IP_SOCKET_IND_List); i:= i + 1) { IP_SOCK_CTRL.send(p_IP_SOCKET_IND_List[i]); } p_IP_SOCKET_IND_List := {}; } [] IP_SOCK_CTRL_QUEUE.receive(cr_IP_SOCKET_IND_AnyCtrl) -> value v_IP_SOCKET_IND { i := lengthof(p_IP_SOCKET_IND_List); p_IP_SOCKET_IND_List[i] := v_IP_SOCKET_IND; } }

function f_IPMUX_PTC_MainLoop() runs on IPMUX_PTC { var IP_SOCKET_IND_List v_IP_SOCKET_IND_List_ByRef := {}; while (true) { alt { [] a_IPMUX_PTC_HandlerDL(IP_SOCK_DATA); [] a_IPMUX_PTC_HandlerDL(IP_SOCK_CTRL); [] a_IPMUX_PTC_HandlerUL_Queue(v_IP_SOCKET_IND_List_ByRef); [] a_IPMUX_PTC_HandlerUL(); } } }

function f_IP_PTC_CreateAndMap(IP_SYSTEM p_System) return IP_PTC { var IP_PTC v_IP_PTC := IP_PTC.create("IP") alive; // @sic R5s210270 sic@ var IPMUX_PTC v_IPMUX_PTC := IPMUX_PTC.create("IPMUX") alive; // @sic R5s210270 sic@ map(v_IP_PTC:IP_CTRL, p_System:IP_CTRL); map(v_IP_PTC:IPSEC_CTRL, p_System:IPSEC_CTRL); map(v_IPMUX_PTC:IP_SOCK, p_System:IP_SOCK); connect(v_IP_PTC:IP_SOCK_DATA, v_IPMUX_PTC:IP_SOCK_DATA); connect(v_IP_PTC:IP_SOCK_CTRL, v_IPMUX_PTC:IP_SOCK_CTRL); connect(v_IP_PTC:IP_SOCK_CTRL_QUEUE, v_IPMUX_PTC:IP_SOCK_CTRL_QUEUE); v_IPMUX_PTC.start(f_IPMUX_PTC_MainLoop()); return v_IP_PTC; }