3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220183

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2333 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test cases 9.1.5.2.7 and 9.1.5.2.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following REGISTRATION REJECT steps, IMS component should be reset to allow for 2nd IMS registration following. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reset IMS component |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant IMS-enabled UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.2.7.NR5GC, 9.1.5.2.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 1 Change 1	3

2.2 Change 2	8

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

2.1 1 Change 1

| Function name | fl_TC_9_1_5_2_8_TestBody |
| --- | --- |
| Reason for change | At STEP 5 The SS transmits a REGISTRATION REJECT message including 5GMM cause value #10 (implicitly de-registered) and so an IMS enabled UE will perform IMS registration again during the step 13-27a, following the initial Registration procedure. |
| Summary of change | Reset IMS component to allow IMS registration. |
| TTCN module | Mobility_Registration_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_9_1_5_2_8_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellB); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC(v_NGC_NASCellA);/* @sic R5s190761 change 4.1 sic@ */ var NG_NAS_SecurityParams_Type v_SecurityParams; var template (omit) EAP_Message v_EAP:= cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success))); var template (omit) ABBA v_ABBA; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; /* @sic R5s190761 change 4.1 sic@ */ var template (present) NAS_KsiValue v_KeySetId:= f_NR_SecurityKSIamf_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams_save := f_NR_Security_Get(); var NG_NAS_SecurityParams_Type v_SecurityParams_save := f_NR5GC_Security_Get(); var NasCountInfo_Type v_NasCountInfo_save:= f_SS_NG_NAS_CountGet(NASCTRL); //@siclog "Step 1" siclog@ //The SS configures: - NGC Cell B as "Serving cell" - NGC Cell A as "Non-Suitable "Off" cell". f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //---need a prose to establish the RRCConnection. //@siclog "Step 2" siclog@ //The UE transmits an REGISTRATION REQUEST message indicating "mobility registration updating". v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA, omit),/* @sic R5s190761 change 4.1 sic@ */ f_NR_SecurityKSIamf_Get(), -,-,-,-,-, f_LastVisited_TAI(v_NGC_NASCellA))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 3-4" siclog@ //The SS transmits an AUTHENTICATION REQUEST message to establish a new security context. //The UE transmits an AUTHENTICATION RESPONSE message. if (px_NAS_5GC_AuthenticationType == AKA_5G) { f_NR_Authentication5G_AKA(v_NGC_NASCellB, tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected, v_PLMN);/* @sic R5s190761 change 4.1 sic@ */ } else { f_NR_AuthenticationEAP_AKA(v_NGC_NASCellB, tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected, v_PLMN);/* @sic R5s190761 change 4.1 sic@ */ } v_SecurityParams := f_NR5GC_Security_Get(); v_ABBA := cs_ABBA(v_SecurityParams.ABBA); //@siclog "Step 5" siclog@ //The SS transmits a REGISTRATION REJECT message including 5GMM cause value #10 (implicitly de-registered) prior to imitating a SMC to take into account the created during the steps 3-4 partial native 5G security context. (NOTE 1) SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause (omit, '00001010'B)))));/* @sic R5-196755 sic@ */ //@siclog "Step 6" siclog@ //SS releases the RRC connection. f_NR_RRCRelease(v_NGC_NASCellB); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init();//@sic R5s210059 sic@ //@siclog "Step 7-9" siclog@ //Steps 1-3 from the generic procedure for UE initial Registration as specified in TS 38.508-1 [4], subclause 4.5.2, Table 4.5.2.2-2 (connectivity NR) take place. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); //@siclog "Step 10" siclog@ //Check: Does the UE send a REGISTRATION REQUEST message, 5GS registration type IE set to Initial registration? if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellB, omit),/* @sic R5s190761 change 4.1 sic@ */ v_KeySetId,/* @sic R5s190761 change 4.1 sic@ */ -,omit,-,-,-,cr_NG_TAI(v_PLMN, bit2oct(v_TAC)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10 Failed"); } //@siclog "Step 11" siclog@ //The SS transmits a SECURITY MODE COMMAND message indicating the ngKSI of the partial native 5G security context assigned in the AUTHENTICATION REQUEST message sent in step 3. if (px_NAS_5GC_AuthenticationType == AKA_5G) {// only send EAP message and ABBA for for EAP authentication v_EAP := omit; v_ABBA := omit; } f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering, true); /* download security keys and reset NAS COUNT */ v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams, Initial_Secure, v_GMM_MobilityInfo, false, omit, v_EAP, v_ABBA); // send SECURITY MODE COMMAND SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend))); //@siclog "Step 12" siclog@ //The UE transmits a SECURITY MODE REJECT message. SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellB, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp;/* @sic R5s190761 change 4.1 sic@ */ if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_SECURITY_MODE_REJECT(cr_GMM_GSM_Cause(omit, '00011000'B)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); } //@siclog "13-27a1" siclog@ //Steps 5-19a1 from the generic procedure for UE initial Registration as specified in TS 38.508-1 [4], subclause 4.5.2, Table 4.5.2.2-2 (connectivity NR) take place. /* @sic R5s190761 change 4.1 sic@ */ vc_NR5GC_Global.Security := v_SecurityParams_save; vc_NR_Global.Security := v_RRC_SecurityParams_save; f_SS_NG_NAS_CountSet(NASCTRL,f_NasSecurity_IncrementCOUNT(v_NasCountInfo_save.DL), f_NasSecurity_IncrementCOUNT(v_NasCountInfo_save.UL)); f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo,v_NGC_NASCellB,v_ReceivedMsg,TESTMode_OFF,noRrcConnectionRelease); }//End of 9_1_5_2_8 test body //TC_9_1_5_2_9 : Void @sic R5-211333 sic@ }//End of Module

After Change:

function fl_TC_9_1_5_2_8_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellB); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC(v_NGC_NASCellA);/* @sic R5s190761 change 4.1 sic@ */ var NG_NAS_SecurityParams_Type v_SecurityParams; var template (omit) EAP_Message v_EAP:= cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success))); var template (omit) ABBA v_ABBA; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; /* @sic R5s190761 change 4.1 sic@ */ var template (present) NAS_KsiValue v_KeySetId:= f_NR_SecurityKSIamf_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams_save := f_NR_Security_Get(); var NG_NAS_SecurityParams_Type v_SecurityParams_save := f_NR5GC_Security_Get(); var NasCountInfo_Type v_NasCountInfo_save:= f_SS_NG_NAS_CountGet(NASCTRL); //@siclog "Step 1" siclog@ //The SS configures: - NGC Cell B as "Serving cell" - NGC Cell A as "Non-Suitable "Off" cell". f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //---need a prose to establish the RRCConnection. //@siclog "Step 2" siclog@ //The UE transmits an REGISTRATION REQUEST message indicating "mobility registration updating". v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA, omit),/* @sic R5s190761 change 4.1 sic@ */ f_NR_SecurityKSIamf_Get(), -,-,-,-,-, f_LastVisited_TAI(v_NGC_NASCellA))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 3-4" siclog@ //The SS transmits an AUTHENTICATION REQUEST message to establish a new security context. //The UE transmits an AUTHENTICATION RESPONSE message. if (px_NAS_5GC_AuthenticationType == AKA_5G) { f_NR_Authentication5G_AKA(v_NGC_NASCellB, tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected, v_PLMN);/* @sic R5s190761 change 4.1 sic@ */ } else { f_NR_AuthenticationEAP_AKA(v_NGC_NASCellB, tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected, v_PLMN);/* @sic R5s190761 change 4.1 sic@ */ } v_SecurityParams := f_NR5GC_Security_Get(); v_ABBA := cs_ABBA(v_SecurityParams.ABBA); //@siclog "Step 5" siclog@ //The SS transmits a REGISTRATION REJECT message including 5GMM cause value #10 (implicitly de-registered) prior to imitating a SMC to take into account the created during the steps 3-4 partial native 5G security context. (NOTE 1) SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause (omit, '00001010'B)))));/* @sic R5-196755 sic@ */ //@siclog "Step 6" siclog@ //SS releases the RRC connection. f_NR_RRCRelease(v_NGC_NASCellB); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init();//@sic R5s210059 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[f_PDN_PdnIndex2Integer(PDN_1)], cms_IPCAN_IMS_Reset); //@siclog "Step 7-9" siclog@ //Steps 1-3 from the generic procedure for UE initial Registration as specified in TS 38.508-1 [4], subclause 4.5.2, Table 4.5.2.2-2 (connectivity NR) take place. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); //@siclog "Step 10" siclog@ //Check: Does the UE send a REGISTRATION REQUEST message, 5GS registration type IE set to Initial registration? if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellB, omit),/* @sic R5s190761 change 4.1 sic@ */ v_KeySetId,/* @sic R5s190761 change 4.1 sic@ */ -,omit,-,-,-,cr_NG_TAI(v_PLMN, bit2oct(v_TAC)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10 Failed"); } //@siclog "Step 11" siclog@ //The SS transmits a SECURITY MODE COMMAND message indicating the ngKSI of the partial native 5G security context assigned in the AUTHENTICATION REQUEST message sent in step 3. if (px_NAS_5GC_AuthenticationType == AKA_5G) {// only send EAP message and ABBA for for EAP authentication v_EAP := omit; v_ABBA := omit; } f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering, true); /* download security keys and reset NAS COUNT */ v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams, Initial_Secure, v_GMM_MobilityInfo, false, omit, v_EAP, v_ABBA); // send SECURITY MODE COMMAND SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend))); //@siclog "Step 12" siclog@ //The UE transmits a SECURITY MODE REJECT message. SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellB, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp;/* @sic R5s190761 change 4.1 sic@ */ if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_SECURITY_MODE_REJECT(cr_GMM_GSM_Cause(omit, '00011000'B)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); } //@siclog "13-27a1" siclog@ //Steps 5-19a1 from the generic procedure for UE initial Registration as specified in TS 38.508-1 [4], subclause 4.5.2, Table 4.5.2.2-2 (connectivity NR) take place. /* @sic R5s190761 change 4.1 sic@ */ vc_NR5GC_Global.Security := v_SecurityParams_save; vc_NR_Global.Security := v_RRC_SecurityParams_save; f_SS_NG_NAS_CountSet(NASCTRL,f_NasSecurity_IncrementCOUNT(v_NasCountInfo_save.DL), f_NasSecurity_IncrementCOUNT(v_NasCountInfo_save.UL)); f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo,v_NGC_NASCellB,v_ReceivedMsg,TESTMode_OFF,noRrcConnectionRelease); }//End of 9_1_5_2_8 test body //TC_9_1_5_2_9 : Void @sic R5-211333 sic@ }//End of Module

2.2 Change 2

| Function name | fl_TC_9_1_5_2_7_TestBody |
| --- | --- |
| Reason for change | At Step 3 The SS transmits a REGISTRATION REJECT message including 5GMM cause value #9 (UE identity cannot be derived by the network) and so an IMS enabled UE will perform IMS registration again during the step 5 for initial Registration procedure. |
| Summary of change | Reset IMS component to allow new IMS registration. |
| TTCN module | Mobility_Registration_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_9_1_5_2_7_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var float v_T3512_delay := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer,25.0); //@siclog "Step 1" siclog@ //Wait for 25 seconds (expiry of T3512 periodic registration update timer, the value of 30 sec is provided during the initial registration in the Preamble). f_Delay(v_T3512_delay); //@siclog "Step 2" siclog@ //The UE transmit a REGISTRATION REQUEST message with the 5GS registration type IE indicating "periodic registration updating". v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic, f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA, omit), f_NR_SecurityKSIamf_Get(), -, -, -, -, -, f_LastVisited_TAI(v_NGC_NASCellA))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //siclog "Step 3" siclog@ //The SS transmits a REGISTRATION REJECT message including 5GMM cause value #9 (UE identity cannot be derived by the network). SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause (omit, '00001001'B))))); //siclog "Step 4" siclog@ //SS releases the RRC connection. f_NR_RRCRelease(v_NGC_NASCellA); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init();//@sic R5s210059 sic@ //siclog "Step 5" siclog@ //Check: Does the UE perform initial registration on NGC Cell A as specified in TS 38.508-1 [4], Table 4.5.2.2-2? The UE does not provide 5G-GUTI, last visited registered TAI or ngKSI. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), '111'B )){//@sic R5-198772 R5s191081 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 5"); } f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity);//@sic R5-198772 R5s191081 sic@ }//End of 9_1_5_2_7 test body

After Change:

function fl_TC_9_1_5_2_7_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var float v_T3512_delay := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer,25.0); //@siclog "Step 1" siclog@ //Wait for 25 seconds (expiry of T3512 periodic registration update timer, the value of 30 sec is provided during the initial registration in the Preamble). f_Delay(v_T3512_delay); //@siclog "Step 2" siclog@ //The UE transmit a REGISTRATION REQUEST message with the 5GS registration type IE indicating "periodic registration updating". v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic, f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA, omit), f_NR_SecurityKSIamf_Get(), -, -, -, -, -, f_LastVisited_TAI(v_NGC_NASCellA))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //siclog "Step 3" siclog@ //The SS transmits a REGISTRATION REJECT message including 5GMM cause value #9 (UE identity cannot be derived by the network). SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause (omit, '00001001'B))))); //siclog "Step 4" siclog@ //SS releases the RRC connection. f_NR_RRCRelease(v_NGC_NASCellA); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init();//@sic R5s210059 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[f_PDN_PdnIndex2Integer(PDN_1)], cms_IPCAN_IMS_Reset); //siclog "Step 5" siclog@ //Check: Does the UE perform initial registration on NGC Cell A as specified in TS 38.508-1 [4], Table 4.5.2.2-2? The UE does not provide 5G-GUTI, last visited registered TAI or ngKSI. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), '111'B )){//@sic R5-198772 R5s191081 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 5"); } f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity);//@sic R5-198772 R5s191081 sic@ }//End of 9_1_5_2_7 test body