3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220185

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2335 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NRDC test case 8.2.2.9.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - At step 1 and Step 4, there is no need to reactivate the Security Keys since there is no change in the Keys sent in the reconfiguration message. However, in the current implementation Security Keys are always reapplied in the function f_ReconfigureDRBs_NRDC() - The UE is not expected to trigger a RACH procedure at step 2 & 5 , since the RRCReconfiguation message at Step 1 & 4 does not include “ReconfigWithSync”. However, by default PRACH Data Ind is expected in the function f_ReconfigureDRBs_NRDC(). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new paramter to control SS security activation and set as false for this test case. - Pass parameter p_WaitForRachPreamble as “noRACH_Check” in calls to f_ReconfigureDRBs_NRDC is called at step 1 & 4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	8

2.3 Change 3	11

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

2.1 Change 1

| Function name | fl_TC_8_2_2_9_2_TestBody () |
| --- | --- |
| Reason for change | - At step 1 and Step 4, there is no need to reactivate the Security Keys since there is no change in the Keys sent in the reconfiguration message. However, in the current implementation Security Keys are always reapplied in the function f_ReconfigureDRBs_NRDC() - The UE is not expected to trigger a RACH procedure at step 2 & 5 , since the RRCReconfiguation message at Step 1 & 4 does not include “ReconfigWithSync”. However, by default PRACH Data Ind is expected in the function f_ReconfigureDRBs_NRDC(). |
| Summary of change | - Pass new parameter p_applySecurity as false in call to f_ReconfigureDRBs_NRDC (), see change 2. - Pass parameter p_WaitForRachPreamble as “noRACH_Check” in calls to f_ReconfigureDRBs_NRDC is called at step 1 & 4 |
| TTCN module | RRC_Reconfiguration_NRDC_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Accepted |

Before Change

function fl_TC_8_2_2_9_2_TestBody() runs on NR5GC_PTC { //@sic R5s210772, R5-214751 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) PDCP_Config v_PDCP_Config; var DRB_Identity v_DrbId; var template (value) RadioBearerConfig v_RadioBearerConfig2; var PDUSessionInfoList_Type v_PDUSessionInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) SDAP_Config v_SDAP_ConfigSCG; // @siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to MCG radio path from SCG radio path v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_DrbId := f_NR_GetDefaultDRB_SCG(); v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_MCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC( nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_MasterKeys, -, -, -, false, cr_38508_RRCReconfigurationComplete ); //@sic R5s211470 sic@ // @siclog "Step 2" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using MCG radio path in the uplink? v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 3", -, -, nr_Cell1); // @siclog "Step 4" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to SCG radio path from MCG radio path v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_SCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC(nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_SecondaryKeys, -, -, -, false, cr_38508_RRCReconfigurationComplete ); //@sic R5s211470 sic@ // @siclog "Step 5" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 6" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using SCG radio path in the uplink? f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 6", -, -, nr_Cell10); }

After Change

function fl_TC_8_2_2_9_2_TestBody() runs on NR5GC_PTC { //@sic R5s210772, R5-214751 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) PDCP_Config v_PDCP_Config; var DRB_Identity v_DrbId; var template (value) RadioBearerConfig v_RadioBearerConfig2; var PDUSessionInfoList_Type v_PDUSessionInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) SDAP_Config v_SDAP_ConfigSCG; // @siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to MCG radio path from SCG radio path v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_DrbId := f_NR_GetDefaultDRB_SCG(); v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_MCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC( nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_MasterKeys, -, -, noRACH_Check, false, cr_38508_RRCReconfigurationComplete, false); //@sic R5s211470 sic@ //sujith@NRDC Wk49 8.2.2.9.2 // @siclog "Step 2" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using MCG radio path in the uplink? v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 3", -, -, nr_Cell1); // @siclog "Step 4" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to SCG radio path from MCG radio path v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_SCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC(nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_SecondaryKeys, -, -, noRACH_Check, false, cr_38508_RRCReconfigurationComplete, false); //@sic R5s211470 sic@ // @siclog "Step 5" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 6" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using SCG radio path in the uplink? f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 6", -, -, nr_Cell10); }

2.2 Change 2

| Function name | f_ReconfigureDRBs_NRDC () |
| --- | --- |
| Reason for change | At step 1 and Step 4, there is no need to reactivate the Security Keys since there is no change in the Keys sent in the reconfiguration message. However, in the current implementation Security Keys are always reapplied in the function f_ReconfigureDRBs_NRDC() |
| Summary of change | Add new parameter p_activateSecurity to control security activation, with default value true. The new parameter is passed to function f_ReconfigureSS_DRBs_NRDC() . |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_ReconfigureDRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) RadioBearerConfig p_RadioBearerConfig1, template (omit) RadioBearerConfig p_RadioBearerConfig2, template (omit) CellGroupConfig p_CellGroupConfigMCG, template (omit) CellGroupConfig p_CellGroupConfigSCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH, boolean p_AddingSCG := false, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))) ) runs on NR5GC_PTC { var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template(omit) SK_Counter v_SK_Counter := omit; var boolean v_PDUSessionModificationComplete := false; if (p_AddingSCG) { // @sic R5s210716 sic@ v_PDUSessionModificationComplete := true; // This field is always included upon initial configuration of an NR SCG v_SK_Counter := 0; } v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( p_CellGroupConfigSCG, p_CellGroupConfigMCG, p_RadioBearerConfig2, -, v_SK_Counter ); // @sic R5s210716 sic@ //Reconfigure SS f_ReconfigureSS_DRBs_NRDC( p_MCG_CellId, p_SCG_CellId, p_TimingInfo, p_SS_ConfigMCG, p_SS_ConfigSCG, p_NRDC_SecurityActivation, p_RNTI_Value); //Send RRCReconfiguration Receive RRCReconfiurationComplete and checl PRACH premable in the SCG cell f_NR_SendRRCReconfiguration_NRDC(p_MCG_CellId, p_SCG_CellId, p_RadioBearerConfig1, v_V1530Ext, p_NASMsg, p_TimingInfo, p_SRB, p_WaitForRachPreamble, v_PDUSessionModificationComplete, // @sic R5s210716 sic@ p_RRCConnReconfigComplete ); }

After Change

function f_ReconfigureDRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) RadioBearerConfig p_RadioBearerConfig1, template (omit) RadioBearerConfig p_RadioBearerConfig2, template (omit) CellGroupConfig p_CellGroupConfigMCG, template (omit) CellGroupConfig p_CellGroupConfigSCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH, boolean p_AddingSCG := false, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))), boolean p_activateSecurity := true ) runs on NR5GC_PTC { var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template(omit) SK_Counter v_SK_Counter := omit; var boolean v_PDUSessionModificationComplete := false; if (p_AddingSCG) { // @sic R5s210716 sic@ v_PDUSessionModificationComplete := true; // This field is always included upon initial configuration of an NR SCG v_SK_Counter := 0; } v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( p_CellGroupConfigSCG, p_CellGroupConfigMCG, p_RadioBearerConfig2, -, v_SK_Counter ); // @sic R5s210716 sic@ //Reconfigure SS f_ReconfigureSS_DRBs_NRDC( p_MCG_CellId, p_SCG_CellId, p_TimingInfo, p_SS_ConfigMCG, p_SS_ConfigSCG, p_NRDC_SecurityActivation, p_RNTI_Value, p_activateSecurity); //Send RRCReconfiguration Receive RRCReconfiurationComplete and checl PRACH premable in the SCG cell f_NR_SendRRCReconfiguration_NRDC(p_MCG_CellId, p_SCG_CellId, p_RadioBearerConfig1, v_V1530Ext, p_NASMsg, p_TimingInfo, p_SRB, p_WaitForRachPreamble, v_PDUSessionModificationComplete, // @sic R5s210716 sic@ p_RRCConnReconfigComplete ); }

2.3 Change 3

| Function name | F_ReconfigureSS_DRBs_NRDC () |
| --- | --- |
| Reason for change | At step 1 and Step 4, there is no need to reactivate the Security at the SS since there is no change in the Keys sent in the reconfiguration message. However, in the current implementation Security Keys are always reapplied in the function f_ReconfigureDRBs_NRDC() |
| Summary of change | Add new parameter p_activateSecurity to control security activation, with default value true |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_ReconfigureSS_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4 ) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release if (isvalue(p_SS_ConfigMCG)) { f_NR_SS_CommonRadioBearerConfig(p_MCG_CellId, p_SS_ConfigMCG, p_TimingInfo); } //Reconfigure SS SCG if (isvalue(p_SS_ConfigSCG)) { f_NR_SS_CommonRadioBearerConfig(p_SCG_CellId, p_SS_ConfigSCG, p_TimingInfo); f_NR_CellInfo_SetRNTI(p_SCG_CellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_SCG_CellId, p_RNTI_Value); f_NR_ULGrantConfiguration_Start(p_SCG_CellId); } //Configure PDCP security on MCG and SCG select (p_NRDC_SecurityActivation) { case (MCG) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_MasterKeys) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_SecondaryKeys) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } case(SCG) { v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } } }

After Change

function f_ReconfigureSS_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, boolean p_activateSecurity := true ) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release if (isvalue(p_SS_ConfigMCG)) { f_NR_SS_CommonRadioBearerConfig(p_MCG_CellId, p_SS_ConfigMCG, p_TimingInfo); } //Reconfigure SS SCG if (isvalue(p_SS_ConfigSCG)) { f_NR_SS_CommonRadioBearerConfig(p_SCG_CellId, p_SS_ConfigSCG, p_TimingInfo); f_NR_CellInfo_SetRNTI(p_SCG_CellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_SCG_CellId, p_RNTI_Value); f_NR_ULGrantConfiguration_Start(p_SCG_CellId); } if (p_activateSecurity) { //Configure PDCP security on MCG and SCG select (p_NRDC_SecurityActivation) { case (MCG) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_MasterKeys) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_SecondaryKeys) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } case(SCG) { v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } } } }