3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220188

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2337 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC test case 7.1.1.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Amount of data to be sent in step 1 can exceed size of the DL grant for certain bands, specifically those with limited bandwidth. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to reduce number of PDUs per slot from 5 to 2 so the total amount of data to be sent in DL does not exceed maximum TBS even for bands with bandwidth n25. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cannot be executed on bands with bandwidth <= n50 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

  - Change 1

| Function name | f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames |
| --- | --- |
| Reason for change | In current TTCN implementation the amount of data before adding RLC and MAC headers to be sent in DL in step 1 in a single TTI is 38 400 bits (320B RLC SDU size x 5 PDUs per slot x 3 LCHs). Yet the maximum DL TBS with default parameters and bandwidth=52 is less than 38000. This implies that the test is executable only on bands with BW higher than 52, because otherwise the maximum DL grant is not sufficient for such amount of data. |
| Summary of change | It is proposed to reduce number of PDUs per slot from 5 to 2 so the total amount of data to be sent in DL does not exceed maximum TBS even for bands with bandwidth n25 (e.g. FDD bands n5, n8, n30, …). |
| TTCN module | NR_TC_Common\7_1_1\MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted as improvement |

Before change:

function f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(template (value) NR_PDCP_SDU_Type p_PDCP_SDU,

integer p_NumSDUs) runs on NR_BASE_PTC

return template (value) NR_DRB_DataPerSlotList_DL_Type

{

const integer tsc_NumPDUPerSlot := 5;

var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataList;

var template (value) NR_PDCP_SDUList_Type v_PdcpSduList;

var integer i;

var integer j := p_NumSDUs/tsc_NumPDUPerSlot;

var integer k := p_NumSDUs - (j * tsc_NumPDUPerSlot);

var integer v_SlotOffset := (f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1)) * 5; // half sub frame scheduling periodicity

v_PdcpSduList := {

p_PDCP_SDU,

p_PDCP_SDU,

p_PDCP_SDU,

p_PDCP_SDU,

p_PDCP_SDU

};

for (i:=0; i<j; i:=i+1) {

v_DataList[i] := cs_NR_DRB_DataPerSlot_DL(v_SlotOffset * i, -, cs_NR_L2DataList_PdcpSdu(v_PdcpSduList));

}

if (k > 0) { // the number of SDU's to send is not a multiple of tsc_NumPDUPerSlot

v_PdcpSduList := {};

for (i:=0; i<k; i:=i+1) {

v_PdcpSduList[i] := p_PDCP_SDU;

}

v_DataList[j] := cs_NR_DRB_DataPerSlot_DL(v_SlotOffset * j, -, cs_NR_L2DataList_PdcpSdu(v_PdcpSduList));

}

return v_DataList;

}

After change:

function f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(template (value) NR_PDCP_SDU_Type p_PDCP_SDU,

integer p_NumSDUs) runs on NR_BASE_PTC

return template (value) NR_DRB_DataPerSlotList_DL_Type

{

const integer tsc_NumPDUPerSlot := 2;

var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataList;

var template (value) NR_PDCP_SDUList_Type v_PdcpSduList;

var integer i;

var integer j := p_NumSDUs/tsc_NumPDUPerSlot;

var integer k := p_NumSDUs - (j * tsc_NumPDUPerSlot);

var integer v_SlotOffset := (f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1)) * 5; // half sub frame scheduling periodicity

v_PdcpSduList := {

p_PDCP_SDU,

p_PDCP_SDU

};

for (i:=0; i<j; i:=i+1) {

v_DataList[i] := cs_NR_DRB_DataPerSlot_DL(v_SlotOffset * i, -, cs_NR_L2DataList_PdcpSdu(v_PdcpSduList));

}

if (k > 0) { // the number of SDU's to send is not a multiple of tsc_NumPDUPerSlot

v_PdcpSduList := {};

for (i:=0; i<k; i:=i+1) {

v_PdcpSduList[i] := p_PDCP_SDU;

}

v_DataList[j] := cs_NR_DRB_DataPerSlot_DL(v_SlotOffset * j, -, cs_NR_L2DataList_PdcpSdu(v_PdcpSduList));

}

return v_DataList;

}