3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220192

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0773 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS Supplementary Services test case 7.6a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 7.6a using IWD 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS Supplementary Services test case 7.6a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	4

4	Execution Log Files	4

HiSilicon	4

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 7.4a which is part of the iwd-TTCN3-B2020-09_D21wk49.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	7.6a.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required

  - Correction to function f_TC_7_6a_NR5GC_IMS1

| Function name | f_TC_7_6a_NR5GC_IMS1 |
| --- | --- |
| Reason for change | A common function is used to create a base INVITE, "f_IMS_MTCallSetup_BuildInviteRequestWithSdp", and due to specific requirements of the test case, some modifications are done to attributes in media list ( SDP offer). In order to these changes take effect, INVITE "messagebody" needs to be updated accordingly. |
| Summary of change | Updated messagebody for initial INVITE to include specifics attributes in media list required by this test case. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted |

Before

<<SKIPPED CODE>>

function f_TC_7_6a_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MT Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS

…

f_IMS_CC_StartCall(IPCAN_MT_SpeechCall);

// @siclog "Step 1" siclog@                 INVITE

v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithPreconditions);

v_SDP_Media_Desc := v_InviteRequestWithSdp.SdpOffer.media_list[0];

v_Fmt := f_SDP_MediaDescr_GetAudioCodec(v_SDP_Media_Desc, EVS_16000);

v_FmtpAttribute := cs_SDP_Attribute_fmtp(v_Fmt, cs_Fmtp_ParamList(cs_EVS_Fmtp_AudioParams("5.9-24.4", "nb-swb")));

v_InviteRequestWithSdp.SdpOffer.media_list[0].attributes := valueof(f_SDP_ExchangeAttribute(v_SDP_Media_Desc.attributes, cr_SDP_Attribute_fmtp(v_Fmt), v_FmtpAttribute));

IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite));

// @siclog "Step 2 - 3" siclog@             100 Trying, 183 Session Progress

v_SdpAnswer := f_IMS_BuildSDP_AnswerAudio_RX(InitialAnswer, EVS_16000, tsc_SDP_WithPreconditions, cr_Fmtp_ParamList(cr_EVS_Fmtp_AudioParamsBrBwMaxRed("5.9-24.4", "nb-swb")));

…

}<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_7_6a_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MT Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS

…

f_IMS_CC_StartCall(IPCAN_MT_SpeechCall);

// @siclog "Step 1" siclog@                 INVITE

v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithPreconditions);

v_SDP_Media_Desc := v_InviteRequestWithSdp.SdpOffer.media_list[0];

v_Fmt := f_SDP_MediaDescr_GetAudioCodec(v_SDP_Media_Desc, EVS_16000);

v_FmtpAttribute := cs_SDP_Attribute_fmtp(v_Fmt, cs_Fmtp_ParamList(cs_EVS_Fmtp_AudioParams("5.9-24.4", "nb-swb")));

v_InviteRequestWithSdp.SdpOffer.media_list[0].attributes := valueof(f_SDP_ExchangeAttribute(v_SDP_Media_Desc.attributes, cr_SDP_Attribute_fmtp(v_Fmt), v_FmtpAttribute));

v_InviteRequestWithSdp.Invite.messageBody := valueof(cs_MessageBody_SDP(v_InviteRequestWithSdp.SdpOffer));

IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite));

// @siclog "Step 2 - 3" siclog@             100 Trying, 183 Session Progress

v_SdpAnswer := f_IMS_BuildSDP_AnswerAudio_RX(InitialAnswer, EVS_16000, tsc_SDP_WithPreconditions, cr_Fmtp_ParamList(cr_EVS_Fmtp_AudioParamsBrBwMaxRed("5.9-24.4", "nb-swb")));

…

}

<<SKIPPED CODE>>

# Branches executed

The test case was executed on NR band n78 using NR ciphering algorithm nea1 and integrity algorithm nia1 on Rohde & Schwarz 5G Protocol Conformance Test platform with Hisilicon Balong 5000 UE

# Execution Log Files

## HiSilicon

The HiSilicon Balong 5000 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7.6a_Hisilicon.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220193: Supporting information for addition of IMS 5GS Call Control test case 7.6a |
| --- | --- |