3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220201

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2345 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 8.1.4.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1)In the current implementation of function fl_TC_8_1_4_1_5_TestBody , at step 2 and 8 , reconfiguration to trigger HO is sent which is going to be unsuccesful due to target cell is powered down . Reconfiguration to trigger HO will be resulting in SS retransmissions while UE is doing Handover failure based procedure ; To stop this reconfiguration retransmission rb release and reset is required before changing the CRNTI ( otherwise this RRC Configuration retx will still get continued with updated CRNTI and interfere the reestablishment procedure). 2)At step 8 , reconfiguration is sent with activation time ; this can be sent with immediate activation time which is different from step 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) RB release and configuration are sent so that reconfiguration retx can be avoided 2) At step 8 , reconfiguration message is sent with immediate activation as in step2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to the function fl_TC_8_1_4_1_5_TestBody	2

1.2	Correction to the function f_NR5GC_RRCReestablishment_Steps3To7	4

# Corrections required

## Correction to the function fl_TC_8_1_4_1_5_TestBody

| Function name | fl_TC_8_1_4_1_5_TestBody |
| --- | --- |
| Reason for change | 1)In the current implementation of function fl_TC_8_1_4_1_5_TestBody , at step 2 and 8 , reconfiguration to trigger HO is sent which is going to be unsuccesful due to target cell is powered down . Reconfiguration to trigger HO will be resulting in SS retransmissions while UE is doing Handover failure based procedure ; To stop this reconfiguration retransmission rb release and reset is required before changing the CRNTI ( otherwise this RRC Configuration retx will still get continued with updated CRNTI and interfere the reestablishment procedure). 2)At step 8 , reconfiguration is sent with activation time ; this can be sent with immediate activation time which is different from step 2 |
| Summary of change | 1) RB release and configuration are sent so that reconfiguration retx can be avoided 2) At step 8 , reconfiguration message is sent with immediate activation as in step2 |
| TTCN module | develop/Common/NR5GC/8_1_4/ RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted in principle, implemented as per R5-141467 in f_NR5GC_RRCReestablishment_Def – see proposed implementation below - Accepted |

Before Change:

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var SubFrameTiming_Type v_SubframeTiming; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; //timer t_T304Min; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //t_T304Min.start(v_T304Min); //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 4-6B" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, -, "Step 4"); //@siclog "Step 7" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 8" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. v_SubframeTiming := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_SubframeTiming), v_RRCReconfigurationHO)); //@sic R5s210162 sic@ //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.5.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 9" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@sic R5s210162 R5-211332 sic@ //@siclog "Step 10-12B" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336, R5s210162 R5-211332 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, -, "Step 12"); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message on NR Cell 1. //@sic R5s210162 R5-211332 sic@ f_NR_RRCRelease(nr_Cell1); } // End of fl_TC_8_1_4_1_5_TestBody

After Change:

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var SubFrameTiming_Type v_SubframeTiming; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var IntegerList_Type v_DrbIdList; //timer t_T304Min; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //t_T304Min.start(v_T304Min); //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true); //@sic R5-211417 R5s211169 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); //@siclog "Steps 4-6B" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, -, "Step 4"); //@siclog "Step 7" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 8" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. //v_SubframeTiming := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //@sic R5s210162 sic@ //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.5.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 9" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@sic R5s210162 R5-211332 sic@ v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true); //@sic R5-211417 R5s211169 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); //@siclog "Step 10-12B" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336, R5s210162 R5-211332 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, -, "Step 12"); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message on NR Cell 1. //@sic R5s210162 R5-211332 sic@ f_NR_RRCRelease(nr_Cell1); } // End of fl_TC_8_1_4_1_5_TestBody

MCC160 proposed implementation

function f_NR5GC_RRCReestablishment_Def(NR_CellId_Type            p_NR_SourceCellId,

NR_CellId_Type            p_NR_TargetCellId,

template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure,

NextHopChainingCount      p_NextHopChainingCount := 0,

template (omit) NR_SCellToAddModList_Type p_SCellConfig := omit,

RNTI_Value                p_RNTI_Value :=  tsc_C_RNTI_Value4,

template(omit) charstring p_VerdictMsg := omit,

template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit,

template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *,

boolean p_ReceiveForRRCreconfigurationComplete := true

) runs on NR5GC_PTC

{ //@sic R5-206369 R5s220372 sic@

//Steps 1-2

f_NR5GC_RRCReestablishment_Steps0To2(p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_RNTI_Value, p_VerdictMsg);  //@sic R5-211391 sic@

//Steps 3-7

f_NR5GC_RRCReestablishment_Steps3To7(p_NR_SourceCellId, p_NR_TargetCellId, p_NextHopChainingCount, -, p_RRCReestablishmentComplete_v1610_IEs);

//Steps 8-13

//p_SCellConfig is used only to send RRCReconfiguration to UE - The SCell in SS shall already be configured

f_NR5GC_RRCReestablishment_Steps8To13(p_NR_SourceCellId, p_NR_TargetCellId, p_SCellConfig, p_RlfTimersAndConstants, -, p_ReceiveForRRCreconfigurationComplete);

}

//================================================================================================================

/*

* @desc      RRC Reestablishment sequence as per 38.523-3 cl 7.3.5.4 RRC connection re-establishment steps 1 to 2

* @param     p_NR_SourceCellId

* @param     p_NR_TargetCellId

* @param     p_ReestablishmentCause (default value: handoverFailure)

* @param     p_RNTI_Value        (default value: tsc_C_RNTI_Value4)

* @param     p_VerdictMsg        (default value: omit)

* @param     p_MacBearerRoutingCA (default value: omit)

* @status    APPROVED (NR5GC)

*/

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type            p_NR_SourceCellId,

NR_CellId_Type            p_NR_TargetCellId,

template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure,

RNTI_Value                p_RNTI_Value :=  tsc_C_RNTI_Value4,

template(omit) charstring p_VerdictMsg := omit,

template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit

) runs on NR_BASE_PTC

{ //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@

//@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@

var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId);  //C-RNTI used by UE in RRCReestablismentRequest

var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId);  //@sic R5s211169 change 1.1 sic@

var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId);

// 0. Source Cell:	If the Source Cell is the same as the Target Cell: 	Release SRBs and DRBs. (Re-)configure SRBs.

//@sic R5-221467 sic@

if (p_NR_SourceCellId == p_NR_TargetCellId) {

f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList);  //@sic R5-211417 R5s211169 R5s220176 sic@

f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit);          //@sic R5s220176 sic@

// @sic R5-217830 sic@

}

// 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI.

f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value);

f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag);    // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq,

cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg),

cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters),

cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params)));

// 2. Target Cell: Receive RRCReestablismentRequest.

SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA));

if (isvalue(p_VerdictMsg)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg));

}

}

//================================================================================================================

/*

* @desc      RRC Reestablishment sequence as per 38.523-3 cl 7.3.5.4 RRC connection re-establishment steps 3 to 7

* @param     p_NR_SourceCellId

* @param     p_NR_TargetCellId

* @param     p_NextHopChainingCount (default value: 0)

* @param     p_MacBearerRoutingCA (default value: omit)

* @param     p_RRCReestablishmentComplete_v1610_IEs (default value: omit)

* @status    APPROVED (NR5GC)

*/

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type            p_NR_SourceCellId,

NR_CellId_Type            p_NR_TargetCellId,

NextHopChainingCount      p_NextHopChainingCount := 0,

template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit,

template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *

) runs on NR_BASE_PTC

{ // @sic R5-206369 R5-213516: added p_MacBearerRoutingCA R5s220372  sic@

//    var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId);

// 3. Void

//@sic R5-221467 sic@

// 4. void

// @sic R5-217830 sic@

// 5. Target Cell: Activate Security

f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true);

// 6. Target : Send RRCReestablishment

SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA));

// 7. Target Cell:  Receive RRCReestablishmentComplete

SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) );

}