3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220227

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2360 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NAS 5GMM test case 9.1.5.1.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) After deregistration in Preamble, UE has done a local release of the PDU session established and UE should be allowed to re-establish it during the Initial Registration inside the Test Body #associated draft proseCR would be submitted during RAN5#94-e meeting 2) From Change#1, RRCrelease performed at the end, requiring Postamble be performed in IDLE |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Handle PDU session establishment from UE after Registration Complete. 2) Postamble is in IDLE state |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.15.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | fl_TC_9_1_5_1_15_TestBody() |
| --- | --- |
| Reason for change | Corrections made to R5s211673 on top of D21wk49 TTCN: After deregistration in Preamble, UE has done a local release of the PDU session established and UE should be allowed to re-establish it during the Initial Registration inside the Test Body Steps 19a1-20a1 is done in order to put UE in RRC_IDLE for the postamble. This avoids having to handle postamble via SRB1 or SRB2 depending on whether UE established a PDU session after Registration Complete. #associtated draft proseCR would be submitted during RAN5#94-e meeting |
| Summary of change | Handle PDU session establishment from UE after Registration Complete, and perform RRC release afterwards. |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#94-e meeting. |

Before Change

function fl_TC_9_1_5_1_15_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var TrackingAreaCode v_TAC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_EmptyCAGInfoList); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //prose CR needed f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA); //@sic R5s211673 sic@ v_NrCapability := f_NR_MobileInfo_NRCap_Get(); if (not match (v_NrCapability.accessStratumRelease, rel15)) { //@sic R5-213385 sic@ f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); } } //End of fl_TC_9_1_5_1_15_Body

After Change

function fl_TC_9_1_5_1_15_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var TrackingAreaCode v_TAC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_EmptyCAGInfoList); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //prose CR needed f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA, -, -, rrcConnectionRelease); //@sic R5s211673 sic@ v_NrCapability := f_NR_MobileInfo_NRCap_Get(); if (not match (v_NrCapability.accessStratumRelease, rel15)) { //@sic R5-213385 sic@ f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); } } //End of fl_TC_9_1_5_1_15_Body

  - Change 2

| Function name | f_TC_9_1_5_1_15_NR5GC() |
| --- | --- |
| Reason for change | From Change#1, RRCrelease performed at the end, requiring Postamble be performed in IDLE |
| Summary of change | Postamble is in IDLE state |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#94-e meeting. |

Before Change

function f_TC_9_1_5_1_15_NR5GC() runs on NR5GC_PTC { //Initial registration / Success / Extended and spare fields in UE Network Capability var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_1_5_1_15_TestBody(); f_NR_TestBody_Set(false); //Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

After Change

function f_TC_9_1_5_1_15_NR5GC() runs on NR5GC_PTC { //Initial registration / Success / Extended and spare fields in UE Network Capability var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_1_5_1_15_TestBody(); f_NR_TestBody_Set(false); //Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }