3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220228

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4661 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PLMN Selection testcase 6.1.1.1 with IMS enabled |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Presently TC 6.1.1.1 (with IMS enabled) fails at Step 26Aa1 & Step29Aa1 , since optional IMS PDN is not handled |  |  |  |  |  |  |  |  |
| Summary of change: |  | Used function f_IP_Handling_StartPDN to handle IMS PDN on Cell1 & Cell13 at Step20 & Step28 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_6_1_1_1_EUTRA |
| --- | --- |
| Reason for change | Presently TC 6.1.1.1 (with IMS enabled) fails at Step 26Aa1 & Step29Aa1 , since optional IMS PDN is not handled |
| Summary of change | Used function f_IP_Handling_StartPDN to handle IMS PDN on Cell1 & Cell13 at Step20 & Step28 respectively. |
| TTCN module | LTE\6_1\Idle_PLMNSelection.ttcn |
| MCC160 Comment | Alternative solution implemented: changed the cell id in the routing table within function f_EUTRA_WaitForIMSReRegister once we know the UE will want to re-register i.e. after the RRC Connection Request has been received |

Before change:

function f_TC_6_1_1_1_EUTRA() runs on EUTRA_PTC

{

var template (value) CellPowerList_Type v_CellPowerList_AtT1;

var template (value) CellPowerList_Type v_CellPowerList_AtT2;

var template (value) CellPowerList_Type v_CellPowerList_AtT3;

var float v_HpPlmnUpper;        //@sic R5-192714 sic@

timer t_HpPlmn_lower;

timer t_HpPlmn_upper;

timer t_IdleMode_GenericTimer := tsc_IdleMode_GenericTimer;

v_CellPowerList_AtT1 := {

cs_CellPower(eutra_Cell1, tsc_ServingCellRS_EPRE),

cs_CellPower(eutra_Cell13, tsc_ServingCellRS_EPRE)

};

v_CellPowerList_AtT2 := {

cs_CellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE)  //@sic R5s100675 cl. 4.2 Change 1 sic@

};

v_CellPowerList_AtT3 := { //@sic R5-140751 sic@

cs_CellPower(eutra_Cell14, tsc_ServingCellRS_EPRE)

};

//Initialise all cell, security and mobile parameters

f_EUTRA_Init(c1);//SIB2 & SIB3 are transmitted

//Set Sys info and cell info for cell 12, 13 & 14 to be used @ creation

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell12, cs_HPLMN_001_11);  //PLMN1

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell13, cs_HPLMN_001_21);  //PLMN2

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell14, cs_HPLMN_001_31);  //PLMN3

//Set TAC for Cell1, Cell12, Cell13 & Cell14

//TAC for Cell1 is set to 1 in f_EUTRA_Init

f_EUTRA_CellInfo_SetTAC(eutra_Cell12, tsc_IdleMode_TAC_Cell12);

f_EUTRA_CellInfo_SetTAC(eutra_Cell13, tsc_IdleMode_TAC_Cell13);

f_EUTRA_CellInfo_SetTAC(eutra_Cell14, tsc_IdleMode_TAC_Cell14);

//Configure and activate all cells

//Cell power level will be -85 dBm/15kHz & attenuation set so that cell is seen as OFF

f_EUTRA_CellConfig_Def(eutra_Cell12);

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell13);

f_EUTRA_CellConfig_Def(eutra_Cell14);  //As postamble will be executed on this cell

//Equip UE with a USIM containing default values except for those shown in Table 6.1.1.1.3.1-2

f_UT_USIM_Insert(UT, "36.523-1 Table 6.1.1.1.3.1-2");

//Preamble: The UE is made to camp on Cell 12 and then in state Switched OFF (State 1)

f_EUTRA_Preamble(eutra_Cell12, STATE2_IDLEUPDATE);

f_EUTRA_UE_Detach_SwitchOffUe(eutra_Cell12, RRC_IDLE);

f_EUTRA_TestBody_Set(true);

//@siclog "Step 1" siclog@

//Adjust cell configuration according to T1

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

//Power on UE

f_EUTRA_SwitchOnUEandStartIP(eutra_Cell12); // @sic R5s120065 sic@

if (pc_ue_CategoryDL_M1){       //@sic R5-192714 sic@

v_HpPlmnUpper := 7500.0;

} else {

v_HpPlmnUpper := 660.0;

}

t_HpPlmn_lower.start(f_EUTRA_SetTimerToleranceMin(eutra_Cell12, nonProtocolTimer, 120.0));

t_HpPlmn_upper.start(f_EUTRA_SetTimerToleranceMax(eutra_Cell12, nonProtocolTimer, v_HpPlmnUpper)) ; //@sic R5-115625 R5-140751 sic@

t_IdleMode_GenericTimer.start;

//@siclog "Step 3" siclog@

//Receive RRCConnectionRequest on Cell 12

f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell12);   // @sic R5-120721 change 3 sic@

t_IdleMode_GenericTimer.stop;

//* @verdict pass RRCConnectionRequest message received on Cell 1

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 3");

//@siclog "Step 4 - 18" siclog@

//Perform step 3 to 17 of registration procedure on cell 12

f_EUTRA_IdleUpdated_WithoutRRCConnReq(eutra_Cell12, STATE2_IDLEUPDATE);

//@siclog "Step 19" siclog@

//Check if PLMN1 is indicated by UE

f_UT_PLMN_Check(UT, "00111");

//* @verdict pass PLMN1 indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 19");

//@siclog "Step 20" siclog@

//Check: Does the UE send an RRCConnectionRequest on Cell 1 after 120 seconds, but before 660 / 7500 (for Category M1) seconds (Note 2) from power on?

alt {

[] SRB.receive( car_SRB0_RrcPdu_IND(eutra_Cell1, cr_508_RRCConnectionRequest))

{

//* @verdict fail RRCConnectionRequest received on PLMNN4 / Cell 1 within t_HpPlmn_lower time

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 20");

}

[] t_HpPlmn_lower.timeout

{

if (f_EUTRA_RRC_ConnectionRequest_Common(eutra_Cell1, t_HpPlmn_upper)) {  // @sic R5-120721 change 3 sic@

//* @verdict pass RRC Connection request received on PLMN4 / Cell 1 after t_HpPlmn_lower time but before t_HpPlmn_upper

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 20");

}

else {

//* @verdict fail No RRCConnectionRequest received

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 20");

}

}

}

//@siclog "Step 21 - 25" siclog@

//Perform TAU procedure steps 2 to 6

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq(eutra_Cell1);

// VOID @sic R5-140751 sic@

//@sic "Step 26Aa1" sic@

// The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

//@sic R5-201099 sic@

f_EUTRA_WaitForIMSReRegister(eutra_Cell1);

//@siclog "Step 26B" siclog@

//Check if PLMN4 is indicated by UE

//@sic R5-203812 sic@

f_UT_PLMN_Check(UT, "00101");

//* @verdict pass PLMN4 is indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 26B");

//@siclog "Step 28" siclog@

//Adjusts cell power level according to T2

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Step 29" siclog@

//Check if UE camps on Cell13

f_EUTRA_IdleMode_TAU(eutra_Cell13);

t_HpPlmn_lower.start(f_EUTRA_SetTimerToleranceMin(eutra_Cell13, nonProtocolTimer, 120.0)); //@sic R5-115625 R5s190153 sic@

t_HpPlmn_upper.start(f_EUTRA_SetTimerToleranceMin(eutra_Cell12, nonProtocolTimer, v_HpPlmnUpper)); //@sic R5s190153 sic@

//@sic "Step 29Aa1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

//@sic R5-201099 sic@

f_EUTRA_WaitForIMSReRegister(eutra_Cell13);

//@siclog "Step 30" siclog@

//SS adjusts cell levels according to row T3 of table 6.1.1.1.3.2

// @sic R5-140751 sic@

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT3);

//@siclog "Step 31" siclog@

//Check if PLMN2 is indicated by UE

f_UT_PLMN_Check(UT, "00121");

//* @verdict pass PLMN2 is indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31");

//@siclog "Step 32" siclog@

//Check: Does the UE send an RRCConnectionRequest on Cell 14 after 120 seconds, but before 660 / 7500 (for Category M1) seconds after step29 ? (Note 1 and 2)

alt {

[] SRB.receive( car_SRB0_RrcPdu_IND(eutra_Cell14, cr_508_RRCConnectionRequest))

{

//* @verdict fail RRCConnectionRequest received within t_HpPlmn_upper time

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 32");

}

[] t_HpPlmn_lower.timeout  //@sic R5-192714 sic@

{

if (f_EUTRA_RRC_ConnectionRequest_Common(eutra_Cell14, t_HpPlmn_upper)) {  // @sic R5-120721 change 3 R5-140751 sic@

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 32");

}

}

}

//@siclog "Step 33 - 37" siclog@

//Perform TAU procedure steps 2 to 6

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq(eutra_Cell14);

//@siclog "Step 38" siclog@

//Check if PLMN3 is indicated by UE

f_UT_PLMN_Check(UT, "00131");

//* @verdict PLMN3 is indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 38");

f_EUTRA_TestBody_Set(false);

//Switch/power off UE

f_EUTRA_Postamble(eutra_Cell14, E1_IDLE);

};

After change:

import from IP_PTC_CtrlMsgs all;

function f_TC_6_1_1_1_EUTRA() runs on EUTRA_PTC

{

var template (value) CellPowerList_Type v_CellPowerList_AtT1;

var template (value) CellPowerList_Type v_CellPowerList_AtT2;

var template (value) CellPowerList_Type v_CellPowerList_AtT3;

var float v_HpPlmnUpper;        //@sic R5-192714 sic@

timer t_HpPlmn_lower;

timer t_HpPlmn_upper;

timer t_IdleMode_GenericTimer := tsc_IdleMode_GenericTimer;

v_CellPowerList_AtT1 := {

cs_CellPower(eutra_Cell1, tsc_ServingCellRS_EPRE),

cs_CellPower(eutra_Cell13, tsc_ServingCellRS_EPRE)

};

v_CellPowerList_AtT2 := {

cs_CellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE)  //@sic R5s100675 cl. 4.2 Change 1 sic@

};

v_CellPowerList_AtT3 := { //@sic R5-140751 sic@

cs_CellPower(eutra_Cell14, tsc_ServingCellRS_EPRE)

};

//Initialise all cell, security and mobile parameters

f_EUTRA_Init(c1);//SIB2 & SIB3 are transmitted

//Set Sys info and cell info for cell 12, 13 & 14 to be used @ creation

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell12, cs_HPLMN_001_11);  //PLMN1

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell13, cs_HPLMN_001_21);  //PLMN2

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell14, cs_HPLMN_001_31);  //PLMN3

//Set TAC for Cell1, Cell12, Cell13 & Cell14

//TAC for Cell1 is set to 1 in f_EUTRA_Init

f_EUTRA_CellInfo_SetTAC(eutra_Cell12, tsc_IdleMode_TAC_Cell12);

f_EUTRA_CellInfo_SetTAC(eutra_Cell13, tsc_IdleMode_TAC_Cell13);

f_EUTRA_CellInfo_SetTAC(eutra_Cell14, tsc_IdleMode_TAC_Cell14);

//Configure and activate all cells

//Cell power level will be -85 dBm/15kHz & attenuation set so that cell is seen as OFF

f_EUTRA_CellConfig_Def(eutra_Cell12);

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell13);

f_EUTRA_CellConfig_Def(eutra_Cell14);  //As postamble will be executed on this cell

//Equip UE with a USIM containing default values except for those shown in Table 6.1.1.1.3.1-2

f_UT_USIM_Insert(UT, "36.523-1 Table 6.1.1.1.3.1-2");

//Preamble: The UE is made to camp on Cell 12 and then in state Switched OFF (State 1)

f_EUTRA_Preamble(eutra_Cell12, STATE2_IDLEUPDATE);

f_EUTRA_UE_Detach_SwitchOffUe(eutra_Cell12, RRC_IDLE);

f_EUTRA_TestBody_Set(true);

//@siclog "Step 1" siclog@

//Adjust cell configuration according to T1

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

//Power on UE

f_EUTRA_SwitchOnUEandStartIP(eutra_Cell12); // @sic R5s120065 sic@

if (pc_ue_CategoryDL_M1){       //@sic R5-192714 sic@

v_HpPlmnUpper := 7500.0;

} else {

v_HpPlmnUpper := 660.0;

}

t_HpPlmn_lower.start(f_EUTRA_SetTimerToleranceMin(eutra_Cell12, nonProtocolTimer, 120.0));

t_HpPlmn_upper.start(f_EUTRA_SetTimerToleranceMax(eutra_Cell12, nonProtocolTimer, v_HpPlmnUpper)) ; //@sic R5-115625 R5-140751 sic@

t_IdleMode_GenericTimer.start;

//@siclog "Step 3" siclog@

//Receive RRCConnectionRequest on Cell 12

f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell12);   // @sic R5-120721 change 3 sic@

t_IdleMode_GenericTimer.stop;

//* @verdict pass RRCConnectionRequest message received on Cell 1

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 3");

//@siclog "Step 4 - 18" siclog@

//Perform step 3 to 17 of registration procedure on cell 12

f_EUTRA_IdleUpdated_WithoutRRCConnReq(eutra_Cell12, STATE2_IDLEUPDATE);

//@siclog "Step 19" siclog@

//Check if PLMN1 is indicated by UE

f_UT_PLMN_Check(UT, "00111");

//* @verdict pass PLMN1 indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 19");

//@siclog "Step 20" siclog@

if(pc_IMS) {

f_IP_Handling_StartPDN(IP, PDN_1, cs_DrbInfo_EUTRA(eutra_Cell1, tsc_DRB1));

}

//Check: Does the UE send an RRCConnectionRequest on Cell 1 after 120 seconds, but before 660 / 7500 (for Category M1) seconds (Note 2) from power on?

alt {

[] SRB.receive( car_SRB0_RrcPdu_IND(eutra_Cell1, cr_508_RRCConnectionRequest))

{

//* @verdict fail RRCConnectionRequest received on PLMNN4 / Cell 1 within t_HpPlmn_lower time

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 20");

}

[] t_HpPlmn_lower.timeout

{

if (f_EUTRA_RRC_ConnectionRequest_Common(eutra_Cell1, t_HpPlmn_upper)) {  // @sic R5-120721 change 3 sic@

//* @verdict pass RRC Connection request received on PLMN4 / Cell 1 after t_HpPlmn_lower time but before t_HpPlmn_upper

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 20");

}

else {

//* @verdict fail No RRCConnectionRequest received

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 20");

}

}

}

//@siclog "Step 21 - 25" siclog@

//Perform TAU procedure steps 2 to 6

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq(eutra_Cell1);

// VOID @sic R5-140751 sic@

//@sic "Step 26Aa1" sic@

// The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

//@sic R5-201099 sic@

f_EUTRA_WaitForIMSReRegistration(eutra_Cell1);

//@siclog "Step 26B" siclog@

//Check if PLMN4 is indicated by UE

//@sic R5-203812 sic@

f_UT_PLMN_Check(UT, "00101");

//* @verdict pass PLMN4 is indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 26B");

//@siclog "Step 28" siclog@

//Adjusts cell power level according to T2

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT2);

if(pc_IMS) {

f_IP_Handling_StartPDN(IP, PDN_1, cs_DrbInfo_EUTRA(eutra_Cell13, tsc_DRB1));

}

//@siclog "Step 29" siclog@

//Check if UE camps on Cell13

f_EUTRA_IdleMode_TAU(eutra_Cell13);

t_HpPlmn_lower.start(f_EUTRA_SetTimerToleranceMin(eutra_Cell13, nonProtocolTimer, 120.0)); //@sic R5-115625 R5s190153 sic@

t_HpPlmn_upper.start(f_EUTRA_SetTimerToleranceMin(eutra_Cell12, nonProtocolTimer, v_HpPlmnUpper)); //@sic R5s190153 sic@

//@sic "Step 29Aa1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

//@sic R5-201099 sic@

f_EUTRA_WaitForIMSReRegister(eutra_Cell13);

//@siclog "Step 30" siclog@

//SS adjusts cell levels according to row T3 of table 6.1.1.1.3.2

// @sic R5-140751 sic@

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT3);

//@siclog "Step 31" siclog@

//Check if PLMN2 is indicated by UE

f_UT_PLMN_Check(UT, "00121");

//* @verdict pass PLMN2 is indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31");

//@siclog "Step 32" siclog@

//Check: Does the UE send an RRCConnectionRequest on Cell 14 after 120 seconds, but before 660 / 7500 (for Category M1) seconds after step29 ? (Note 1 and 2)

alt {

[] SRB.receive( car_SRB0_RrcPdu_IND(eutra_Cell14, cr_508_RRCConnectionRequest))

{

//* @verdict fail RRCConnectionRequest received within t_HpPlmn_upper time

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 32");

}

[] t_HpPlmn_lower.timeout  //@sic R5-192714 sic@

{

if (f_EUTRA_RRC_ConnectionRequest_Common(eutra_Cell14, t_HpPlmn_upper)) {  // @sic R5-120721 change 3 R5-140751 sic@

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 32");

}

}

}

//@siclog "Step 33 - 37" siclog@

//Perform TAU procedure steps 2 to 6

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq(eutra_Cell14);

//@siclog "Step 38" siclog@

//Check if PLMN3 is indicated by UE

f_UT_PLMN_Check(UT, "00131");

//* @verdict PLMN3 is indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 38");

f_EUTRA_TestBody_Set(false);

//Switch/power off UE

f_EUTRA_Postamble(eutra_Cell14, E1_IDLE);

};

## MCC160 Implementation:

function f_EUTRA_WaitForIMSReRegister(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC

{

var IPCAN_RAN_Type v_RanType := f_EUTRA_IMS_GetRanType();

timer t_WaitForIMS := 10.0;

if (pc_IMS) {

t_WaitForIMS.start;

alt {

[] t_WaitForIMS.timeout {} // Don't do anything

[] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) {

t_WaitForIMS.stop;

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(p_CellId,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_508_SERVICE_REQUEST(v_KsiValue)));

f_IP_ChangeEutraCell(IP, p_CellId);  // @sic R5s220228 sic@

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStart(v_RanType));

f_EUTRA_Activate_SRB2_DRB_SendRrcMsg(p_CellId,

tsc_RRC_TI_Def,

v_NasInd.SecurityProtection.NasCount);

f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId,

tsc_RRC_TI_Def);

f_EUTRA_DelayForUserPlaneSignalling(waitForIMS);

f_EUTRA_RRC_ConnectionRelease(p_CellId);

}

}

}

}