3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220280

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2372 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.5.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the tdd_UL_DL_ConfigurationCommon configuration provided for cell 1, DL-UL pattern looks as below (D – Downlink Slot , U – Uplink Slot, F – Flexible Slot ) 0; 1; 2; 3; 4; 5; 6; 7; 8; 9. D; D; D; D; D; D; D; F; U; U; D; D; D; D; D; D; D; F; U; U. At the beginning of the test case , nr cell1 is configured with prach_ConfigurationIndex = (160) . As per table 6.3.3.2-3 in spec 38.211 , Prach is expected to come in the subframe = 9 . In response, RAR will be scheduled at a Subframe which makes sure (K2+ Δ)th subframe is receiving Msg3 at the UL slot. From Rar to Message 3 timing , Please refer refer 38.214 section 6.1.2.1.1 When the UE transmits a PUSCH scheduled by RAR or by the fallbackRAR, the Δ value specific to the PUSCH subcarrier spacing µPUSCH is applied in addition to the K2 value . As per the test case configuration K2 = 6 and Δ = 3 (when running the test in 30Khz SCS); When receiving the Prach in Subframe 9 slot = 1 , RAR is sent on Subframe = 5 Slot = 0 , so that Msg3 can be received in Subframe = 9 Slot = 1 ( from RAR it takes K2 + Δ slots to receive Msg3) ; prach_ConfigurationIndex = (160) is working well in TDD pattern . At step 2 , SS changes the prach-configuration index as “94” for which prach is expected to be received in the subframe = 4/9 ( every odd SFN ) ; If rach is received in Subframe =9 and slot = 1 , no issue is observed ; But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . |
| Summary of change: |  | Increase the Rar window size ( from sl20 sl40) when configuring prach configuration index as “94”. Prose CR R5-221105 will be raised for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to the function fl_TC_8_1_5_2_2_TestBody()	3

- Corrections required

  - Correction to the function fl_TC_8_1_5_2_2_TestBody()

| Function name | fl_TC_8_1_5_2_2_TestBody() |
| --- | --- |
| Reason for change | With the prach configuration index configured in step 2 , Rar window is getting expired before RAR is getting transmitted ( in case of TDD ) |
| Summary of change | increase the Rar window size ( from sl20 sl40) when configuring prach configuration index as “94” for TDD this function f_NR_CellInfo_SetRaResponseWindow implementation is proposed as part of R5s220125 |
| TTCN module | ttcn-3\ttcn\develop\NR5GC\8_1_5 \RRC_Others_NR5GC |
| MCC160 Comment | Conditional to prose CR agreement on 38.523-1. |

Before Change:

function fl_TC_8_1_5_2_2_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> //Stop the UL Grant allocation //@sic R5s201136 R5-204334 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Steps 2/3" siclog@ //Reconfigure SS with wew PRACH configuration index //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); // Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); << SKIPPED CODE >> }

After Change:

function fl_TC_8_1_5_2_2_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> //Stop the UL Grant allocation //@sic R5s201136 R5-204334 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Steps 2/3" siclog@ //Reconfigure SS with wew PRACH configuration index //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); f_NR_CellInfo_SetRaResponseWindow(nr_Cell1, sl40); // Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); << SKIPPED CODE >> }