3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220386

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0157 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LPP_Templates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Subclase 9 test cases include a capability check for LPP ecid-ProvideCapabilities based on the value of pc_ECID. Since ECID is a LTE specific positioning method, most of NR test cases do not need to send the LPP ecid-RequestCapabilities and the value of pc_ECID must not be be evaluated and compared to the LPP ecid-RequestCapabilities for these test cases. Since ECID is a LTE specific method, OEMs are not allowed to change the value of pc_ECID for NR test cases. This may lead into fail verdicts since UE ECID capabiltiies and pc_ECID may not be aligned. Similarly happens with pc_OTDOA and OTDOA-RequestCapabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New LPP templates are created specifically for NR test cases for the LPP Request Capabilities message, to remove the ECID and OTDOA request capabilities. Apart from that, ECID and OTDOA Provide capabilities are only checked for those subtests that involve any of these positioning methods; For ECID subtest 6 and for OTDOA subtests 5 and 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.1, 9.3.1.2, 9.3.2.1, 9.3.2.2, 9.3.2.3, 9.3.3.1, 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 37.571-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1.1 Correction to LPP_Templates.ttcn	3

1.2 Correction to f_POS_NR_CheckCapabilities	6

1.3 Correction to f_POS_NR_CapabilityTransferProcedure	9

1.4 Correction to NR5GC_Positioning_Testcases.ttcn	10

## 1.1 Correction to LPP_Templates.ttcn

| Function name | LPP_Templates.ttcn |
| --- | --- |
| Reason for change | The template cs_LppReqCap enforces the ecid and otdoa request capabilities and there is no way to allow or remove these message contents depending on some flag |
| Summary of change | New template cs_LppReqCap_NR is created for NR test cases. Apart from that, other templates that are based on the cs_LppReqCap_NR are created: - cs_LppRequestCapabilities_NR - cs_LppRequestCapabilities_NR_ECID - cs_LppRequestCapabilities_NR_OTDOA - cs_LppRequestCapabilities_ePDU_NR - cs_LppRequestCapabilities_SeqNumber_NR - cs_LppRequestCapabilities_AckReq_NR These templates allow to remove or include the ecid and otdoa request cacabilities based on the value of p_EcidReqCap and p_OtdoaReqCap |
| TTCN module | LPP_Templates.ttcn |
| MCC160 Comment | Postponed until decision at RAN5#95 on the associated prose CR. |

Before

<<SKIPPED CODE>>

template (value) LPP_Message cs_LppReqCap (TransactionNumber p_TransactionNumber,

template (omit) SequenceNumber p_SeqNum := omit,

template (omit) Acknowledgement p_Ack := omit,

template (omit) EPDU_Sequence p_EpduReqCap := omit):=

{ /* @status    APPROVED (POS) */

transactionID := cs_TransId(locationServer, p_TransactionNumber),

endTransaction := false,

sequenceNumber := p_SeqNum,

acknowledgement := p_Ack,

lpp_MessageBody := {

c1 := {

requestCapabilities := {

criticalExtensions := {

c1 := {

requestCapabilities_r9 := {

commonIEsRequestCapabilities := {

lpp_message_segmentation_req_r14 := tsc_NoLPP_Segmentation //@sic R5-18169 sic@

},

a_gnss_RequestCapabilities := {

gnss_SupportListReq := true,

assistanceDataSupportListReq := true,

locationVelocityTypesReq := true

},

otdoa_RequestCapabilities := {},

ecid_RequestCapabilities := {},

epdu_RequestCapabilities := p_EpduReqCap,

sensor_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

tbs_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

wlan_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

bt_RequestCapabilities_r13  := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

nr_ECID_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_Multi_RTT_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_DL_AoD_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_DL_TDOA_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_UL_RequestCapabilities_r16 := {} /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

}

}

}

}

}

}

};

template (value) LPP_Message cs_LppRequestCapabilities (TransactionNumber p_TransactionNumber) := cs_LppReqCap (p_TransactionNumber);                           /* @status

* APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_ePDU(TransactionNumber p_TransactionNumber,

octetstring p_EPDUBody) := cs_LppReqCap(p_TransactionNumber, -, -, cs_EpduReqCap (p_EPDUBody));     /* @status    APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_SeqNumber(TransactionNumber p_TransactionNumber,

SequenceNumber p_SeqNum) := cs_LppReqCap(p_TransactionNumber, p_SeqNum);                       /* @status    APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_AckReq(TransactionNumber p_TransactionNumber,

SequenceNumber p_SeqNum) := cs_LppReqCap(p_TransactionNumber, p_SeqNum, cs_AckTrue);              /* @status

*

* template (value) LPP_Message cs_LppRequestCapabilities (TransactionNumber p_TransactionNumber) := cs_LppReqCap (p_TransactionNumber);                           /* @status    APPROVED (POS) */

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

template (value) LPP_Message cs_LppReqCap (TransactionNumber p_TransactionNumber,

template (omit) SequenceNumber p_SeqNum := omit,

template (omit) Acknowledgement p_Ack := omit,

template (omit) EPDU_Sequence p_EpduReqCap := omit):=

{ /* @status    APPROVED (POS) */

transactionID := cs_TransId(locationServer, p_TransactionNumber),

endTransaction := false,

sequenceNumber := p_SeqNum,

acknowledgement := p_Ack,

lpp_MessageBody := {

c1 := {

requestCapabilities := {

criticalExtensions := {

c1 := {

requestCapabilities_r9 := {

commonIEsRequestCapabilities := {

lpp_message_segmentation_req_r14 := tsc_NoLPP_Segmentation //@sic R5-18169 sic@

},

a_gnss_RequestCapabilities := {

gnss_SupportListReq := true,

assistanceDataSupportListReq := true,

locationVelocityTypesReq := true

},

otdoa_RequestCapabilities := {},

ecid_RequestCapabilities := {},

epdu_RequestCapabilities := p_EpduReqCap,

sensor_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

tbs_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

wlan_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

bt_RequestCapabilities_r13  := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

nr_ECID_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_Multi_RTT_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_DL_AoD_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_DL_TDOA_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_UL_RequestCapabilities_r16 := {} /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

}

}

}

}

}

}

};

template (value) LPP_Message cs_LppReqCap_NR (TransactionNumber p_TransactionNumber,

template (omit) SequenceNumber p_SeqNum := omit,

template (omit) Acknowledgement p_Ack := omit,

template (omit) EPDU_Sequence p_EpduReqCap := omit,

template (omit) ECID_RequestCapabilities p_EcidReqCap := omit,

template (omit) OTDOA_RequestCapabilities p_OtdoaReqCap := omit):=

{ /* @status    APPROVED (POS) */

transactionID := cs_TransId(locationServer, p_TransactionNumber),

endTransaction := false,

sequenceNumber := p_SeqNum,

acknowledgement := p_Ack,

lpp_MessageBody := {

c1 := {

requestCapabilities := {

criticalExtensions := {

c1 := {

requestCapabilities_r9 := {

commonIEsRequestCapabilities := {

lpp_message_segmentation_req_r14 := tsc_NoLPP_Segmentation //@sic R5-18169 sic@

},

a_gnss_RequestCapabilities := {

gnss_SupportListReq := true,

assistanceDataSupportListReq := true,

locationVelocityTypesReq := true

},

otdoa_RequestCapabilities := p_OtdoaReqCap,

ecid_RequestCapabilities := p_EcidReqCap,

epdu_RequestCapabilities := p_EpduReqCap,

sensor_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

tbs_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

wlan_RequestCapabilities_r13 := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

bt_RequestCapabilities_r13  := {}, /* @sic R5s160713 BASELINE MOVING 2016, R5-165996: iPOS inclusion sic@ */

nr_ECID_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_Multi_RTT_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_DL_AoD_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_DL_TDOA_RequestCapabilities_r16 := {}, /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

nr_UL_RequestCapabilities_r16 := {} /* @sic R5s201388 BASELINE MOVING 2020 Rel-16 sic@ */

}

}

}

}

}

}

};

template (value) LPP_Message cs_LppRequestCapabilities (TransactionNumber p_TransactionNumber) := cs_LppReqCap (p_TransactionNumber);                           /* @status

* APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_ePDU(TransactionNumber p_TransactionNumber,

octetstring p_EPDUBody) := cs_LppReqCap(p_TransactionNumber, -, -, cs_EpduReqCap (p_EPDUBody));     /* @status    APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_SeqNumber(TransactionNumber p_TransactionNumber,

SequenceNumber p_SeqNum) := cs_LppReqCap(p_TransactionNumber, p_SeqNum);                       /* @status    APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_AckReq(TransactionNumber p_TransactionNumber,

SequenceNumber p_SeqNum) := cs_LppReqCap(p_TransactionNumber, p_SeqNum, cs_AckTrue);              /* @status

*

* template (value) LPP_Message cs_LppRequestCapabilities (TransactionNumber p_TransactionNumber) := cs_LppReqCap (p_TransactionNumber);                           /* @status    APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_NR(TransactionNumber p_TransactionNumber) := cs_LppReqCap_NR (p_TransactionNumber);

template (value) LPP_Message cs_LppRequestCapabilities_NR_ECID(TransactionNumber p_TransactionNumber) := cs_LppReqCap_NR (p_TransactionNumber,-,-,-,{});

template (value) LPP_Message cs_LppRequestCapabilities_NR_OTDOA(TransactionNumber p_TransactionNumber) := cs_LppReqCap_NR (p_TransactionNumber,-,-,-,-, {});

template (value) LPP_Message cs_LppRequestCapabilities_ePDU_NR(TransactionNumber p_TransactionNumber,

octetstring p_EPDUBody) := cs_LppReqCap_NR(p_TransactionNumber, -, -, cs_EpduReqCap (p_EPDUBody));     /* @statusAPPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_SeqNumber_NR(TransactionNumber p_TransactionNumber,

SequenceNumber p_SeqNum) := cs_LppReqCap(p_TransactionNumber, p_SeqNum, -,-);                       /* @status    APPROVED (POS) */

template (value) LPP_Message cs_LppRequestCapabilities_AckReq_NR(TransactionNumber p_TransactionNumber,

SequenceNumber p_SeqNum) := cs_LppReqCap(p_TransactionNumber, p_SeqNum, cs_AckTrue, -);

<<SKIPPED CODE>>

## 1.2 Correction to f_POS_NR_CheckCapabilities

| Function name | f_POS_NR_CheckCapabilities |
| --- | --- |
| Reason for change | Current TTCN implementation always checks ECID and OTDOA capabilities, which is not correct for those subtests that are not 5, 6 or 7 |
| Summary of change | Include p_CheckEcidCap and p_CheckOtdoaCap to determine if ECID and OTDOA Provide Capabilities message has to be checked |
| TTCN module | NR5GCPositioning_Functions.ttcn |
| MCC160 Comment | Postponed until decision at RAN5#95 on the associated prose CR. |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities (LPP_Message p_LppMessage, boolean p_CheckEpduCap) runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

//CommonIEs

if (pc_LPP_MsgSegmentation) {

if (not isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in Common capabilities PICS");

}

else {

if (not match(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities, cr_CommonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in LPP-Segmentation capabilities");

}

}

}

//AGNSS

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) and (pc_UEB_AGNSS or pc_UEA_AGNSS)) {

f_POS_NR_CheckCapabilities_Agnss(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities);

}

else if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) or pc_UEB_AGNSS or pc_UEA_AGNSS) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in A-GNSS capabilities PICS");

}

//OTDOA

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities)) {

if (not (p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities.otdoa_Mode[0]== '1'B) and pc_OTDOA) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE reported OTDOA UE-Assisted mode not supported");

}

else if((p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities.otdoa_Mode[0]== '1'B) and pc_OTDOA){

f_POS_NR_OTDOA_CheckEutraBand(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities);

}

}

else if (pc_OTDOA) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in OTDOA capabilities PICS");

}

//ECID

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.ecid_ProvideCapabilities) and pc_ECID) {

f_POS_NR_CheckCapabilities_Ecid(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.ecid_ProvideCapabilities);

}

else if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.ecid_ProvideCapabilities) or pc_ECID) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in ECID capabilities PICS");

}

//LPPe

if(p_CheckEpduCap){

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.epdu_ProvideCapabilities) and pc_OMA_LPPe) {

v_Octetstring := p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.epdu_ProvideCapabilities[0].ePDU_Body;

f_POS_NR_OmaLppeMessage_DecodeAndMatch(v_Octetstring, cr_OmaLppe_ProvideCapabilities);

}

else if(isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.epdu_ProvideCapabilities) or pc_OMA_LPPe) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in EPDU capabilities PICS!");

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities (LPP_Message p_LppMessage, boolean p_CheckEpduCap, boolean p_CheckEcidCap:=false, boolean p_CheckOtdoaCap:=false) runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

//CommonIEs

if (pc_LPP_MsgSegmentation) {

if (not isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in Common capabilities PICS");

}

else {

if (not match(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities, cr_CommonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in LPP-Segmentation capabilities");

}

}

}

//AGNSS

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) and (pc_UEB_AGNSS or pc_UEA_AGNSS)) {

f_POS_NR_CheckCapabilities_Agnss(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities);

}

else if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) or pc_UEB_AGNSS or pc_UEA_AGNSS) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in A-GNSS capabilities PICS");

}

//OTDOA

if(p_CheckOtdoaCap){

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities)) {

if (not (p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities.otdoa_Mode[0]== '1'B) and pc_OTDOA) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE reported OTDOA UE-Assisted mode not supported");

}

else if((p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities.otdoa_Mode[0]== '1'B) and pc_OTDOA){

f_POS_NR_OTDOA_CheckEutraBand(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities);

}

}

else if (pc_OTDOA) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in OTDOA capabilities PICS");

}

}

if(p_CheckEcidCap){

//ECID

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.ecid_ProvideCapabilities) and pc_ECID) { //@sic R5s15805 sic@ @sic R5s160257 sic@

f_POS_NR_CheckCapabilities_Ecid(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.ecid_ProvideCapabilities);

}

else if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.ecid_ProvideCapabilities) or pc_ECID) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in ECID capabilities PICS");

}

}

//LPPe

if(p_CheckEpduCap){

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.epdu_ProvideCapabilities) and pc_OMA_LPPe) {

v_Octetstring := p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.epdu_ProvideCapabilities[0].ePDU_Body;

f_POS_NR_OmaLppeMessage_DecodeAndMatch(v_Octetstring, cr_OmaLppe_ProvideCapabilities);

}

else if(isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.epdu_ProvideCapabilities) or pc_OMA_LPPe) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in EPDU capabilities PICS!");

}

}

<<SKIPPED CODE>>

## 1.3 Correction to f_POS_NR_CapabilityTransferProcedure

| Function name | f_POS_NR_CapabilityTransferProcedure |
| --- | --- |
| Reason for change | Current TTCN implementation do not allow to differentiate the need to check ECID and OTDOA capabilities. Changes proposed in 1.1 and 1.2 need to be included in this function. |
| Summary of change | Include p_CheckEcidCap and p_CheckOtdoaCap to determine if ECID and OTDOA Provide Capabilities message has to be checked. Depending on these values, also include here the changes from 1.1 and 1.2. |
| TTCN module | NR5GCPositioning_Functions.ttcn |
| MCC160 Comment | Postponed until decision at RAN5#95 on the associated prose CR. |

Before

<<SKIPPED CODE>>

function f_POS_NR_CapabilityTransferProcedure () runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var LPP_Message v_LppMessage;

//@siclog "Step 0" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities(tsc_POS_TransactionNumber));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

//@siclog "Step 0a" siclog@

//Receive LPP PROVIDE CAPABILITIES

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, cr_LppProvideCapabilities (tsc_POS_TransactionNumber));

// Further matching is required according to PICS values:

f_POS_NR_CheckCapabilities (v_LppMessage, false);

//@siclog "Step 0b" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 2

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CapabilityTransferProcedure (boolean p_CheckEcidCap := false, boolean p_CheckOtdoaCap := false) runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var LPP_Message v_LppMessage;

//@siclog "Step 0" siclog@

//SS sends LPP REQUEST CAPABILITIES

if (p_CheckEcidCap)

{

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_NR_ECID(tsc_POS_TransactionNumber));

}

else if (p_CheckOtdoaCap)

{

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_NR_OTDOA(tsc_POS_TransactionNumber));

}

else

{

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_NR(tsc_POS_TransactionNumber));

}

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

//@siclog "Step 0a" siclog@

//Receive LPP PROVIDE CAPABILITIES

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, cr_LppProvideCapabilities (tsc_POS_TransactionNumber));

// Further matching is required according to PICS values:

f_POS_NR_CheckCapabilities (v_LppMessage, false, p_CheckEcidCap, p_CheckOtdoaCap);

//@siclog "Step 0b" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 2

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

<<SKIPPED CODE>>

## 1.4 Correction to NR5GC_Positioning_Testcases.ttcn

| Function name | NR5GC_Positioning_Testcases.ttcn |
| --- | --- |
| Reason for change | Current TTCN implementation do not allow to differentiate the need to check ECID and OTDOA capabilities. Changes proposed in 1.1 and 1.3 need to be included in this function. |
| Summary of change | Include changes from 1.1 and 1.3 in the test case implementation |
| TTCN module | NR5GC_Positioning_Testcases.ttcn |
| MCC160 Comment | Postponed until decision at RAN5#95 on the associated prose CR. |

Before

<<SKIPPED CODE>>

function f_TC_9_3_1_1_NR5GC_TestBody() runs on NR_BASE_PTC

{

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_ePDU(tsc_POS_TransactionNumber,

f_POS_EncodeOmaLppeMessage(cs_OmaLppe_RequestCapabilities)));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

//@siclog "Step 2" siclog@

//Receive LPP PROVIDE CAPABILITIES

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, cr_LppProvideCapabilities(tsc_POS_TransactionNumber));

// Further matching is required according to PICS values:

f_POS_NR_CheckCapabilities (v_LppMessage, true);

//@siclog "Step 2a" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 2

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

<<SKIPPED CODE>>

function f_TC_9_3_1_2_5s_NR5GC_TestBody (E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (present) NG_NAS_UL_Message_Type v_GMMMsg;

timer t_Wait;

//@siclog "Step 00" siclog@

//Clear stored OTDOA assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 0, 0a, 0b" siclog@

f_POS_NR_CapabilityTransferProcedure();

<<SKIPPED CODE>>

function f_TC_9_3_2_1_NR5GC_TestBody() runs on NR_BASE_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

timer t_Wait;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_SeqNumber(tsc_POS_TransactionNumber, 0));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

<<SKIPPED CODE>>

function f_TC_9_3_2_2_NR5GC_TestBody() runs on NR_BASE_PTC

{

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_AckReq(tsc_POS_TransactionNumber, 0));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

<<SKIPPED CODE>>

function f_TC_9_3_2_3_NR5GC_TestBody() runs on NR_BASE_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

var integer v_SeqNum;

timer t_Wait;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities(tsc_POS_TransactionNumber));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

<<SKIPPED CODE>>

function f_TC_9_3_4_2_5s_NR5GC_TestBody (E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR_BASE_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure();

<<SKIPPED CODE>>

function f_TC_9_3_4_2_7s_NR5GC_TestBody (PositioningSystemList_Type p_PosSystemListId,

E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR5GC_POS_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var ProvideAssistanceData_r9_IEs v_ProvideAssistData;

var GNSS_MeasurementList v_GNSS_MeasurementList;

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_AgnssUEPosTech);

//@siclog "Step 1a" siclog@

//Clear stored OTDOA assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure();

<<SKIPPED CODE>>

function f_TC_9_3_4_4_5s_NR5GC_TestBody (E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR_BASE_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var TransactionNumber v_TransactionNumber := 0; // To store Transaction number sent by the UE in LPP Request Assistance Data

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure();

<<SKIPPED CODE>>

function f_TC_9_3_4_4_7s_NR5GC_TestBody (PositioningSystemList_Type p_PosSystemListId,

E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR5GC_POS_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var octetstring v_LppProvideAssistData;

var ProvideAssistanceData_r9_IEs v_ProvideAssistData;

var RequestAssistanceData_r9_IEs v_RequestAssistData;

var GNSS_MeasurementList v_GNSS_MeasurementList;

var TransactionNumber v_TransactionNumber := 0; // To store Transaction number sent by the UE in LPP Request Assistance Data

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_AgnssUEPosTech);

//@siclog "Step 1a" siclog@

//Clear stored OTDOA assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure();

After

<<SKIPPED CODE>>

function f_TC_9_3_1_1_NR5GC_TestBody() runs on NR_BASE_PTC

{

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_ePDU_NR(tsc_POS_TransactionNumber,

f_POS_EncodeOmaLppeMessage(cs_OmaLppe_RequestCapabilities)));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

//@siclog "Step 2" siclog@

//Receive LPP PROVIDE CAPABILITIES

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, cr_LppProvideCapabilities(tsc_POS_TransactionNumber));

// Further matching is required according to PICS values:

f_POS_NR_CheckCapabilities (v_LppMessage, true);

//@siclog "Step 2a" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 2

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

<<SKIPPED CODE>>

function f_TC_9_3_1_2_5s_NR5GC_TestBody (E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (present) NG_NAS_UL_Message_Type v_GMMMsg;

timer t_Wait;

//@siclog "Step 00" siclog@

//Clear stored OTDOA assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 0, 0a, 0b" siclog@

f_POS_NR_CapabilityTransferProcedure(false, true);

<<SKIPPED CODE>>

function f_TC_9_3_2_1_NR5GC_TestBody() runs on NR_BASE_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

timer t_Wait;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_SeqNumber_NR(tsc_POS_TransactionNumber, 0));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

<<SKIPPED CODE>>

function f_TC_9_3_2_2_NR5GC_TestBody() runs on NR_BASE_PTC

{

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_AckReq_NR(tsc_POS_TransactionNumber, 0));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

<<SKIPPED CODE>>

function f_TC_9_3_2_3_NR5GC_TestBody() runs on NR_BASE_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var octetstring v_EncodedLppMessage;

var octetstring v_Octetstring;

var LPP_Message v_LppMessage;

var integer v_SeqNum;

timer t_Wait;

//@siclog "Step 1" siclog@

//SS sends LPP REQUEST CAPABILITIES

v_EncodedLppMessage := f_POS_EncodeLppMessage(cs_LppRequestCapabilities_NR(tsc_POS_TransactionNumber));

f_POS_NR_SendEncodedLppMessage(v_EncodedLppMessage);

<<SKIPPED CODE>>

function f_TC_9_3_4_2_5s_NR5GC_TestBody (E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR_BASE_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure(false, true);

<<SKIPPED CODE>>

function f_TC_9_3_4_2_7s_NR5GC_TestBody (PositioningSystemList_Type p_PosSystemListId,

E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR5GC_POS_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var ProvideAssistanceData_r9_IEs v_ProvideAssistData;

var GNSS_MeasurementList v_GNSS_MeasurementList;

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_AgnssUEPosTech);

//@siclog "Step 1a" siclog@

//Clear stored OTDOA assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure(false, true);

<<SKIPPED CODE>>

function f_TC_9_3_4_4_5s_NR5GC_TestBody (E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR_BASE_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var TransactionNumber v_TransactionNumber := 0; // To store Transaction number sent by the UE in LPP Request Assistance Data

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure(false, true);

<<SKIPPED CODE>>

function f_TC_9_3_4_4_7s_NR5GC_TestBody (PositioningSystemList_Type p_PosSystemListId,

E_OTDOA_NR_CoOrd_OtdoaInfo_Type p_OtdoaInfo) runs on NR5GC_POS_PTC

{

var LPP_Message v_LppMessage;

var octetstring v_Octetstring;

var octetstring v_EncodedLppMessage;

var octetstring v_LppProvideAssistData;

var ProvideAssistanceData_r9_IEs v_ProvideAssistData;

var RequestAssistanceData_r9_IEs v_RequestAssistData;

var GNSS_MeasurementList v_GNSS_MeasurementList;

var TransactionNumber v_TransactionNumber := 0; // To store Transaction number sent by the UE in LPP Request Assistance Data

//@siclog "Step 1" siclog@

//Clear stored GNSS assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_AgnssUEPosTech);

//@siclog "Step 1a" siclog@

//Clear stored OTDOA assistance data

f_POS_NR_ClearStoredData(nr_Cell1, tsc_OtdoaUEPosTech);

//@siclog "Steps 1b, 1c, 1d" siclog@

f_POS_NR_CapabilityTransferProcedure(false, true);