3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220389

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4667 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to multiple legacy LTE-WCDMA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. In order to fix this issue, it will require changes in test cases listed in “Clauses affected” and they will consist in just declare and assign templates in later test steps where security has been already established and IntegrityCheckInfo is not omit and contains a value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 8.4.2.4, 8.4.2.7.1, 8.4.2.7.2, 8.4.2.7.3, 8.5.2.1, 8.6.3.1, 8.6.3.4, 8.6.5.1, 8.6.5.1a, 8.6.7.4 and 8.6.9.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Corrections required	1

1.1	Correction to function f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup	1

# Corrections required

## Correction to function f_TC_8_4_2_2_UTRAN

| Function name | f_TC_8_4_2_2_UTRAN |
| --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |
| TTCN module | LTE_IRAT\8_4\InterRat_Handover_UtoE_UTRAN.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); // @sic R5s120941 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-172896 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def (5), cs_Eutra_FreqAndPriList_OneEntry_Def (v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-172896 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def (5), omit, // @sic R5-174500 sic@ {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225, R5s201661 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } // Create and configure Cell 5 //* Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH ( utran_Cell5 ); <<SKIPPED CODE>> //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if ( pc_InterRAT_DL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); //@sic R5s120686 sic@ } if ( pc_InterRAT_UL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); //@sic R5s120686 sic@ } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired ) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 // @sic R5s130221 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } // Send trigger from UTRAN side f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); //@siclog "Step 9" siclog@ //SS changes Cell1 and UTRAN cell5 level acc. to row T2 in table 8.4.2.2.3.2-1 // Sets the cell power attenuation and stores the value in the EUTRA_CellInfo f_UTRAN_SetCellPower(utran_Cell5, v_T2_CPICH_Cell5, v_T2_PCCPCH_Cell5); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); // @sic R5s120941 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-172896 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def (5), cs_Eutra_FreqAndPriList_OneEntry_Def (v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-172896 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def (5), omit, // @sic R5-174500 sic@ {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225, R5s201661 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } // Create and configure Cell 5 //* Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH ( utran_Cell5 ); <<SKIPPED CODE>> //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if ( pc_InterRAT_DL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); //@sic R5s120686 sic@ } if ( pc_InterRAT_UL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); //@sic R5s120686 sic@ } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired ) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } else { v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 // @sic R5s130221 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ }else{ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ } U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } // Send trigger from UTRAN side f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); //@siclog "Step 9" siclog@ //SS changes Cell1 and UTRAN cell5 level acc. to row T2 in table 8.4.2.2.3.2-1 // Sets the cell power attenuation and stores the value in the EUTRA_CellInfo f_UTRAN_SetCellPower(utran_Cell5, v_T2_CPICH_Cell5, v_T2_PCCPCH_Cell5); <<SKIPPED CODE>>

## Correction to function f_TC_8_4_2_4_UTRAN

| Function name | f_TC_8_4_2_4_UTRAN |
| --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |
| TTCN module | LTE_IRAT\8_4\InterRat_Handover_UtoE_UTRAN.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); // @sic R5s120941 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5s180225 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def (5), cs_Eutra_FreqAndPriList_OneEntry_Def (v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-172896 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def (5), omit, // @sic R5-174500 sic@ {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225, R5s201661 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //@sic R5s180225 sic@ // Create and configure Cell 5 //* Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH ( utran_Cell5 ); <<SKIPPED CODE>> //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if ( pc_InterRAT_DL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); // changed to use activation time } if ( pc_InterRAT_UL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); // changed to use activation time } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired ) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 // @sic R5s130219 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } // Send trigger from UTRAN side f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); //@siclog "Step 9" siclog@ //SS changes Cell1 and UTRAN cell5 level acc. to row T2 in table 8.4.2.4.3.2-1 // Sets the cell power attenuation and stores the value in the EUTRA_CellInfo f_UTRAN_SetCellPower(utran_Cell5, v_T2_CPICH_Cell5, v_T2_PCCPCH_Cell5); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); // @sic R5s120941 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5s180225 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def (5), cs_Eutra_FreqAndPriList_OneEntry_Def (v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-172896 sic@ f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def (5), omit, // @sic R5-174500 sic@ {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225, R5s201661 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //@sic R5s180225 sic@ // Create and configure Cell 5 //* Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH ( utran_Cell5 ); <<SKIPPED CODE>> //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if ( pc_InterRAT_DL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); // changed to use activation time } if ( pc_InterRAT_UL_CompressedModeRequired ) { f_UTRAN_CPHY_RL_Modify ( cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN ( v_CFNInfo.actTime)) ); // changed to use activation time } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired ) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } else { v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //@siclog "Step 8" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 // @sic R5s130219 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ }else{ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ } U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } // Send trigger from UTRAN side f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); //@siclog "Step 9" siclog@ //SS changes Cell1 and UTRAN cell5 level acc. to row T2 in table 8.4.2.4.3.2-1 // Sets the cell power attenuation and stores the value in the EUTRA_CellInfo f_UTRAN_SetCellPower(utran_Cell5, v_T2_CPICH_Cell5, v_T2_PCCPCH_Cell5); <<SKIPPED CODE>>

## Correction to function f_TC_8_6_3_1_UTRAN

| Function name | f_TC_8_6_3_1_UTRAN |  |
| --- | --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |  |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |  |
| TTCN module | LTE_A_IRAT\8_6\RRC_MDT_Logged_UG_UTRAN.ttcn |  |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def(5), cs_Eutra_FreqAndPriList_OneEntry_Def(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-173007 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def(5), omit, //@sic R5-174500 sic@ {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //Create and configure Cell 5 //Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH(utran_Cell5); //@sic R5s140397 sic@ <<SKIPPED CODE>> //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); //ss tx a measurement control to setup inter-RAT on cell 5 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ f_UTRAN_CPHY_MeasurementConfig_TDD(utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def(5), cs_Eutra_FreqAndPriList_OneEntry_Def(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); <<REMOVED CODE>> v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-173007 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def(5), omit, //@sic R5-174500 sic@ {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //Create and configure Cell 5 //Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH(utran_Cell5); //@sic R5s140397 sic@ <<SKIPPED CODE>> //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } else { v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ } else { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ } v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ f_UTRAN_CPHY_MeasurementConfig_TDD(utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ }; <<SKIPPED CODE>>

## Correction to function f_TC_8_6_3_4_UTRAN

| Function name | f_TC_8_6_3_4_UTRAN |  |
| --- | --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |  |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |  |
| TTCN module | LTE_A_IRAT\8_6\RRC_MDT_Logged_UG_UTRAN.ttcn |  |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell1 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[0] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150513 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[0] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150513 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ } //Get Cell Parameters for EUTRA cell12 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[1] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[1] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } <<SKIPPED CODE>> //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { v_SecurityInfo := f_UTRAN_Security_Get(); v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell9, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell1 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[0] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[0] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> } //Get Cell Parameters for EUTRA cell12 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[1] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[1] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } <<SKIPPED CODE>> //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150513 sic@ } else { v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150513 sic@ } //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ } else { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150513 sic@ } f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell9, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

## Correction to function f_TC_8_6_5_1_UTRAN

| Function name | f_TC_8_6_5_1_UTRAN |  |
| --- | --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |  |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |  |
| TTCN module | LTE_A_IRAT\8_6\RRC_LoggedRLF_UG_UTRAN.ttcn |  |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //@siclog Step 7 - 11 siclog@ //Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 //using the UTRA reference radio bearer parameters and combination "UTRA PS RB" according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. //NOTE: The UE performs NW initiated RAB re-establishment in a UTRAN cell. f_UTRAN_MT_RAB_ReEst(utran_Cell5, cell_DCH_HS_DSCH); f_UTRAN_SendAuthCoOrdParameters (EUTRA, omit); //@sic R5s140366 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150377 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150377 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //@sic R5s180225 sic@ <<SKIPPED CODE>> //@siclog "Step 13" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); //ss tx a measurement control to setup inter-RAT on cell 5 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //@siclog "Step 13" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog Step 7 - 11 siclog@ //Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 //using the UTRA reference radio bearer parameters and combination "UTRA PS RB" according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. //NOTE: The UE performs NW initiated RAB re-establishment in a UTRAN cell. f_UTRAN_MT_RAB_ReEst(utran_Cell5, cell_DCH_HS_DSCH); f_UTRAN_SendAuthCoOrdParameters (EUTRA, omit); //@sic R5s140366 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //@sic R5s180225 sic@ <<SKIPPED CODE>> //@siclog "Step 13" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ (utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { // @sic R5s140149 sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } else { v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150377 sic@ } else { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150377 sic@ } f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5-134957 sic@ //@siclog "Step 13" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

## Correction to function f_TC_8_6_5_1a_UTRAN

| Function name | f_TC_8_6_5_1a_UTRAN |  |
| --- | --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |  |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |  |
| TTCN module | LTE_A_IRAT\8_6\RRC_LoggedRLF_UG_UTRAN.ttcn |  |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> // UE's re-done authentication in establishing the RAB so need to send the new params to EUTRA f_UTRAN_SendAuthCoOrdParameters (EUTRA, omit); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150377 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150377 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150377 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150377 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //@sic R5s180225 sic@ <<SKIPPED CODE>> <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // UE's re-done authentication in establishing the RAB so need to send the new params to EUTRA f_UTRAN_SendAuthCoOrdParameters (EUTRA, omit); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ //WA#WI=891017 <<REMOVED CODE>> v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5s180225 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } //@sic R5s180225 sic@ <<SKIPPED CODE>> <<SKIPPED CODE>>

## Correction to function f_TC_8_6_7_4_UTRAN

| Function name | f_TC_8_6_7_4_UTRAN |  |
| --- | --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |  |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |  |
| TTCN module | LTE_A_IRAT\8_6\RRC_LoggedHOFailure_UG_UTRAN.ttcn |  |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell12 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[1] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180248, R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[1] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s210510, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180248, R5s180225 sic@ } <<SKIPPED CODE>> //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { v_SecurityInfo := f_UTRAN_Security_Get(); v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell9, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell12 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); //WA#WI=891017 Corrections to 8.6.7.4 if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[1] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180248, R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[1] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ <<REMOVED CODE>> v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s210510, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180248, R5s180225 sic@ } <<SKIPPED CODE>> //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell9, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired) { //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ } else { v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ } //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ } else { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ } f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell9, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@siclog "Step 21" siclog@ //ss tx a measurement control to setup inter-RAT on cell 9 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

## Correction to function f_TC_8_6_9_1_UTRAN

| Function name | f_TC_8_6_9_1_UTRAN |  |
| --- | --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |  |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |  |
| TTCN module | LTE_A_IRAT\8_6\RRC_Connection_Establishment_Failure_UG_UTRAN.ttcn |  |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell1 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[0] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def (5),v_EUTRA_FrequencyAndPriorityInfoList)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150448 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150448 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[0] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def (5), omit, v_EUTRA_FrequencyAndPriorityInfoList_ext)); //@sic R5-174500 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150448 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150448 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } <<SKIPPED CODE>> //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { v_SecurityInfo := f_UTRAN_Security_Get(); v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell1 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[0] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def (5),v_EUTRA_FrequencyAndPriorityInfoList)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<SKIPPED CODE>> v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[0] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); f_UTRAN_SysInfo_SetSIB19 (utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def (5), omit, v_EUTRA_FrequencyAndPriorityInfoList_ext)); //@sic R5-174500 sic@ v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ <<SKIPPED CODE>> v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s201661, R5s180225 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } <<SKIPPED CODE>> //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150448 sic@ } else { v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225, R5s150448 sic@ } U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150448 sic@ }else{ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5s150448 sic@ } f_UTRAN_CPHY_MeasurementConfig_TDD (utran_Cell5, cs_ActivateNow, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@siclog "Step 12" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

## Correction to function f_TC_8_4_2_7_Common_UTRAN

| Function name | f_TC_8_4_2_7_Common_UTRAN |
| --- | --- |
| Reason for change | Several LTE-WCDMA test cases makes use of the template IntegrityCheckInfo before security is even established. That can cause problems as, even being of optional nature, different tempaltes having them as input parameters expect them to be present. |
| Summary of change | Moved templates declaration using IntegrityCheckInfo as parameter to later stages of the test case where security has been already established. |
| TTCN module | LTE_A_IRAT\8_4\InterRat_Handover_UtoE_CA_UTRAN.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); v_EUTRASysInfo2 := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-174500 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def(5), cs_Eutra_FreqAndPriList_OneEntry_Def(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-173007 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def(5), omit, {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 5, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225, R5s201661 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } <<SKIPPED CODE>> //@siclog "Step 9" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); // changed to use activation time } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); // changed to use activation time } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired ) { // @sic R5s140149 subject to prose CR approval sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); //@siclog "Step 9" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 v_MeasurementControl_InterRATMeasurement_NoCompMode.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); //@siclog "Step 9" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 // @sic R5s130219 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); v_EUTRASysInfo2 := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-174500 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntry(cs_Utra_PriorityInfoList_Def(5), cs_Eutra_FreqAndPriList_OneEntry_Def(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 4))); v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-173007 sic@ f_UTRAN_SysInfo_SetSIB19(utran_Cell5, cs_SIB19_UtraAndOneEutraEntryExtension(cs_Utra_PriorityInfoList_Def(5), omit, {cs_EUTRA_FrequencyAndPriorityInfoExtension_Common( v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, 5, -53, 2, 1, omit, true)})); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225, R5s201661 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180225 sic@ } <<SKIPPED CODE>> //@siclog "Step 9" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement)); //@sic R5s180225 sic@ if (pc_InterRAT_DL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_DL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); // changed to use activation time } if (pc_InterRAT_UL_CompressedModeRequired) { f_UTRAN_CPHY_RL_Modify(cas_CompressedModeStatusInfo_REQ(utran_Cell5, tsc_UL_DPCH1, cs_DPCH_CompressedModeStatusInfoActive(v_Tgps_Reconfiguration_CFN, v_TGCFN), cs_ActivateCFN(v_CFNInfo.actTime))); // changed to use activation time } } else if (v_FDD_TDD_Mode == UTRAN_FDD and not pc_UTRA_CompressedModeRequired ) { // @sic R5s140149 subject to prose CR approval sic@ //refresh security info v_SecurityInfo := f_UTRAN_Security_Get(); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } else { // EUTRA Band > 64 v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180225 sic@ } //@siclog "Step 9" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 U_AM.send ( cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_NoCompMode)); //@sic R5s180225 sic@ } else { // TDD branch v_SecurityInfo := f_UTRAN_Security_Get(); //@siclog "Step 9" siclog@ //ss tx a measurement control to setup inter-RAT on cell 5 if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ } else { v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180225, R5-131872 sic@ } // @sic R5s130219 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9.integrityCheckInfo := v_SecurityInfo.dl_IntegrityCheckInfo; // @sic R5s180254 sic@ U_AM.send(cas_RLC_Data_Req_NoCnf(utran_CellDedicated, tsc_RB2, v_MeasurementControl_InterRATMeasurement_TDD_r9)); //@sic R5s180225 sic@ } <<SKIPPED CODE>>