3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220423

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2423 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Step 3G states: “Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?” Current TTCN implementation uses the function f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession() and so the test case will give an error verdict if the UE has established only an IMS PDU session during the test case preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify TTCN to perform step 3G using “first PDU session” rather than “first non-IMS PDU session” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE which creates a PDU session for IMS only will fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | fl_TC_8_1_1_3_3_TestBody() |
| --- | --- |
| Reason for change | Step 3G states: “Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?” Current TTCN implementation uses the function f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession() and so the test case will give an error verdict if the UE has established only an IMS PDU session during the test case preamble. |
| Summary of change | Modify TTCN to perform step 3G using “first PDU session” rather than “first non-IMS PDU session” |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_1_1_3_3_TestBody() runs on NR5GC_PTC { //@sic R5-197055 sic@ var ARFCN_ValueNR v_NRf1_SSB_Cell1; var ARFCN_ValueNR v_NRf2_SSB_Cell3; var ARFCN_ValueNR v_NRf3_SSB_Cell6; ………………….. //@siclog "Step 3G" siclog@ //@sic R5-201136 sic@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_PDUSessionInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); if (isvalue(v_PDUSessionInfo)) { // @sic R5s200517 sic@ f_NR5GC_CheckDataPath(nr_Cell3, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 3G"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } /*Skipping Code */ //@siclog "Step 16" siclog@ //@sic R5-201136 sic@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 6? f_NR5GC_CheckDataPath(nr_Cell6, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 16"); // @sic R5s200517 sic@ }

After Change:

function fl_TC_8_1_1_3_3_TestBody() runs on NR5GC_PTC { //@sic R5-197055 sic@ var ARFCN_ValueNR v_NRf1_SSB_Cell1; var ARFCN_ValueNR v_NRf2_SSB_Cell3; var ARFCN_ValueNR v_NRf3_SSB_Cell6; ….. var PDUSessionInfoList_Type v_PDUSessionInfoList; ……………… //@siclog "Step 3G" siclog@ //@sic R5-201136 sic@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession() v_PDUSessionInfoList_Type := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionInfo := v_PDUSessionInfoList_Type[ 0]; if (isvalue(v_PDUSessionInfo)) { // @sic R5s200517 sic@ f_NR5GC_CheckDataPath(nr_Cell3, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 3G"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } ………………. //@siclog "Step 16" siclog@ //@sic R5-201136 sic@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 6? f_NR5GC_CheckDataPath(nr_Cell6, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 16"); // @sic R5s200517 sic@ }