3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220430

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2427 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.4.1.7.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 8.1.4.1.7.3 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.4.1.7.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.7.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

1.1 Verification Test Summary	3

2 Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

2.3 Change 3	7

2.4 Change 4	8

2.5 Change 5	9

3 Branches executed	10

4 Execution Log Files	10

4.1 Media Tek M80	10

5 References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.4.1.7.3 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@cictmobile.cn

## Verification Test Summary

Test Case: 	8.1.4.1.7.3

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	Media Tek M80

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_NR_GetCA_PrimaryBand |
| --- | --- |
| Reason for change | 1. Add branch for CA_n78_2A. |
| Summary of change | 1. Add branch for CA_n78_2A. |
| TTCN module | NR_CA_CommonFunctions.ttcn |
| MCC160 Comment | Accepted implemented as per R5-221389 |

Before Change

function f_NR_GetCA_PrimaryBand ( NR_CA_BandCombination_Type p_NR_CA_BandCombination ) return integer { var integer v_PrimaryBandIndicatorNR ; select (p_NR_CA_BandCombination) { //@sic {R5-200921 R5-204378 R5-206712 R5-206306 R5-214019, R5-212986, R5-215971, R5-217259, R5-217587, R5-217588, R5-218267} sic@ ...... case( CA_n66_2A ) { v_PrimaryBandIndicatorNR := 66 ;} case( CA_n71_2A ) { v_PrimaryBandIndicatorNR := 71 ;} case( CA_n261_2A ) { v_PrimaryBandIndicatorNR := 261 ;} case else { FatalError (__FILE__, __LINE__, "Invalid NR CA band combination")} } return v_PrimaryBandIndicatorNR ; }

After Change

function f_NR_GetCA_PrimaryBand ( NR_CA_BandCombination_Type p_NR_CA_BandCombination ) return integer { var integer v_PrimaryBandIndicatorNR ; select (p_NR_CA_BandCombination) { //@sic {R5-200921 R5-204378 R5-206712 R5-206306 R5-214019, R5-212986, R5-215971, R5-217259, R5-217587, R5-217588, R5-218267} sic@ ...... case( CA_n66_2A ) { v_PrimaryBandIndicatorNR := 66 ;} case( CA_n71_2A ) { v_PrimaryBandIndicatorNR := 71 ;} case( CA_n78_2A ) { v_PrimaryBandIndicatorNR := 78 ;} case( CA_n261_2A ) { v_PrimaryBandIndicatorNR := 261 ;} case else { FatalError (__FILE__, __LINE__, "Invalid NR CA band combination")} } return v_PrimaryBandIndicatorNR ; }

## Change 2

| Function name | f_NR_GetCA_IntraBandFrequency |
| --- | --- |
| Reason for change | 1. Add branch for CA_n78_2A. |
| Summary of change | 1. Add branch for CA_n78_2A. |
| TTCN module | NR_CA_FrequencyFunctions.ttcn |
| MCC160 Comment | Rejected, not according to R5-221389 |

Before Change

function f_NR_GetCA_IntraBandFrequency ( NR_CA_BandCombination_Type p_NR_CA_BandCombination_Type ) return template(value) NR_Frequencyf1tof4_Type { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_GetCA_PrimaryBand(p_NR_CA_BandCombination_Type); var P_Max v_P_Max := f_NR_InitP_Max(); select ( p_NR_CA_BandCombination_Type ) { ...... case( CA_n71_2A ) { //@sic R5-214929, R5-215970 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 123890 , v_FreqBandIndicatorNR , 123464 , 0 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 1 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 132664 , 0 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 128890 , v_FreqBandIndicatorNR , 110320 , 504 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 505 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 137448 , 6 , kHz15 , 52 , v_P_Max), 10 ); } case( CA_n261_2A ) { //@sic R5-206306, R5-206861 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2071387 , v_FreqBandIndicatorNR , 2070875 , 0 , kHz120 , 132 ), 100 , kHz120 , 8 , 1 , 22 , 14 , 8 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz120 , 132 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2083771 , v_FreqBandIndicatorNR , 2077325 , 504 , kHz120 , 132 ), 100 , kHz120 , 2 , 1 , 517 , 5 , 8 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz120 , 132 , v_P_Max), 100 ); } case else { FatalError (__FILE__, __LINE__, "Invalid NR-SA band combination") } } return v_NR_Frequencyf1tof4; }

After Change

function f_NR_GetCA_IntraBandFrequency ( NR_CA_BandCombination_Type p_NR_CA_BandCombination_Type ) return template(value) NR_Frequencyf1tof4_Type { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_GetCA_PrimaryBand(p_NR_CA_BandCombination_Type); var P_Max v_P_Max := f_NR_InitP_Max(); select ( p_NR_CA_BandCombination_Type ) { ...... case( CA_n71_2A ) { //@sic R5-214929, R5-215970 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 123890 , v_FreqBandIndicatorNR , 123464 , 0 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 1 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 132664 , 0 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 128890 , v_FreqBandIndicatorNR , 110320 , 504 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 505 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 137448 , 6 , kHz15 , 52 , v_P_Max), 10 ); } case( CA_n78_2A ) { v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , v_FreqBandIndicatorNR , 620058 , 0 , kHz30 , 273 ), 100 , kHz30 , 6 , 2 , 4 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 620058 , 0 , kHz30 , 273 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 647040 , v_FreqBandIndicatorNR , 634628 , 504 , kHz30 , 273 ), 100 , kHz30 , 4 , 3 , 1014 , 0 , 3 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 634628 , 504 , kHz30 , 273 , v_P_Max), 100 ); } case( CA_n261_2A ) { //@sic R5-206306, R5-206861 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2071387 , v_FreqBandIndicatorNR , 2070875 , 0 , kHz120 , 132 ), 100 , kHz120 , 8 , 1 , 22 , 14 , 8 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz120 , 132 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2083771 , v_FreqBandIndicatorNR , 2077325 , 504 , kHz120 , 132 ), 100 , kHz120 , 2 , 1 , 517 , 5 , 8 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz120 , 132 , v_P_Max), 100 ); } case else { FatalError (__FILE__, __LINE__, "Invalid NR-SA band combination") } } return v_NR_Frequencyf1tof4; }

## Change 3

| Function name | NR_CA_BandCombination_Type |
| --- | --- |
| Reason for change | 1. Add branch for CA_n78_2A. |
| Summary of change | 1. Add branch for CA_n78_2A. |
| TTCN module | Common4G5G.ttcn |
| MCC160 Comment | Accepted implemented as per R5-221389 |

Before Change

type enumerated NR_CA_BandCombination_Type { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //@sic R5-200597 R5-200921 R5-203056 R5-206306 R5-214019 R5-212986, R5-214853, R5-214727, R5-214929, R5-215970, R5-2159771, R5-217259, R5-217587, R5-217588, R5-218267 sic@ ...... //Acc to 38.508-1 Table 6.2.3.4-3: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR1 CA_n48_2A, CA_n66_2A, CA_n71_2A, //Acc to 38.508-1 Table 6.2.3.4-4: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR2 CA_n261_2A }

After Change

type enumerated NR_CA_BandCombination_Type { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //@sic R5-200597 R5-200921 R5-203056 R5-206306 R5-214019 R5-212986, R5-214853, R5-214727, R5-214929, R5-215970, R5-2159771, R5-217259, R5-217587, R5-217588, R5-218267 sic@ ...... //Acc to 38.508-1 Table 6.2.3.4-3: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR1 CA_n48_2A, CA_n66_2A, CA_n71_2A, CA_n78_2A, //Acc to 38.508-1 Table 6.2.3.4-4: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR2 CA_n261_2A }

## Change 4

| Function name | f_NR_GetCA_SecondaryBand |
| --- | --- |
| Reason for change | 1. Add branch for CA_n78_2A. |
| Summary of change | 1. Add branch for CA_n78_2A. |
| TTCN module | NR_CA_CommonFunctions.ttcn |
| MCC160 Comment | Accepted implemented as per R5-221389 |

Before Change

function f_NR_GetCA_SecondaryBand ( NR_CA_BandCombination_Type p_NR_CA_BandCombination ) return integer { var integer v_SecondaryBandIndicatorNR ; select (p_NR_CA_BandCombination) { //@sic {R5-200921 R5-204378 R5-206712 R5-206306 R5-214019, R5-212986, R5-215971, R5-217259, R5-217587, R5-217588, R5-218267} sic@ ...... case( CA_n48_2A ) { v_SecondaryBandIndicatorNR := 48 ;} case( CA_n66_2A ) { v_SecondaryBandIndicatorNR := 66 ;} case( CA_n71_2A ) { v_SecondaryBandIndicatorNR := 71 ;} case( CA_n261_2A ) { v_SecondaryBandIndicatorNR := 261 ;} case else { FatalError (__FILE__, __LINE__, "Invalid NR CA band combination")} } return v_SecondaryBandIndicatorNR ; }

After Change

function f_NR_GetCA_SecondaryBand ( NR_CA_BandCombination_Type p_NR_CA_BandCombination ) return integer { var integer v_SecondaryBandIndicatorNR ; select (p_NR_CA_BandCombination) { //@sic {R5-200921 R5-204378 R5-206712 R5-206306 R5-214019, R5-212986, R5-215971, R5-217259, R5-217587, R5-217588, R5-218267} sic@ ...... case( CA_n48_2A ) { v_SecondaryBandIndicatorNR := 48 ;} case( CA_n66_2A ) { v_SecondaryBandIndicatorNR := 66 ;} case( CA_n71_2A ) { v_SecondaryBandIndicatorNR := 71 ;} case( CA_n78_2A ) { v_SecondaryBandIndicatorNR := 78 ;} case( CA_n261_2A ) { v_SecondaryBandIndicatorNR := 261 ;} case else { FatalError (__FILE__, __LINE__, "Invalid NR CA band combination")} } return v_SecondaryBandIndicatorNR ; }

## Change 5

| Function name | f_NR_GetCA_CombinationType |
| --- | --- |
| Reason for change | 1. Add branch for CA_n78_2A. |
| Summary of change | 1. Add branch for CA_n78_2A. |
| TTCN module | NR_CA_CommonFunctions.ttcn |
| MCC160 Comment | Accepted implemented as per R5-221389 |

Before Change

function f_NR_GetCA_CombinationType( NR_CA_BandCombination_Type p_NR_CA_BandCombination ) return NR_CA_Tested_Type { var NR_CA_Tested_Type v_NR_CA_Tested; select (p_NR_CA_BandCombination){ ...... case( CA_n48_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case( CA_n66_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case( CA_n71_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case( CA_n261_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case else { FatalError (__FILE__, __LINE__, "Invalid NR CA band combination")} } return v_NR_CA_Tested; }

After Change

function f_NR_GetCA_CombinationType( NR_CA_BandCombination_Type p_NR_CA_BandCombination ) return NR_CA_Tested_Type { var NR_CA_Tested_Type v_NR_CA_Tested; select (p_NR_CA_BandCombination){ ...... case( CA_n48_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case( CA_n66_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case( CA_n71_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case( CA_n78_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case( CA_n261_2A ) { v_NR_CA_Tested := CA_IntraBandNonContiguous ;} case else { FatalError (__FILE__, __LINE__, "Invalid NR CA band combination")} } return v_NR_CA_Tested; }

# Branches executed

NR Band CA_n78_2A , nea2, Nia2.

# Execution Log Files

## Media Tek M80

The Media Tek M80  passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_8_1_4_1_7_3_NR5GC.log

- PIXCIT settings used:
\ Pics(MTK_SA).xml

\ Pixit(TC_8_1_4_1_7_3_NR5GC).xml

# References

| [1] | R5s220431: Supporting information for agreement of NR5GC test case 8.1.4.1.7.3 in FR1 |
| --- | --- |