3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220455

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2437 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR5GC_RRC_Idle_Extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation of R5s220107 is not aligned with Prose 38.508 Table 4.5.2.2-4 (step 1a1 and 1b1). With current implementation the steps 8 and 9 will get executed whether PDU session is established or not which is incorrect and step 2-9a1 should be skipped in case there is no PDU session establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If PDU session is not established then skip the steps 8 and 9 in function f_NR5GC_RRC_Idle_Extension |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All test cases executed with IMS enabled and pixits set as pc_noOf_PDUsNewConnection=1 and pc_noOf_PDUsSameConnection=1 will fail in Preamble |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All test cases executed with IMS enabled and pixits set as pc_noOf_PDUsNewConnection=1 and pc_noOf_PDUsSameConnection=1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	     Kalahasti, Narendra
                          Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR5GC_RRC_Idle_Extension_Steps1_7 |
| --- | --- |
| Reason for change | Current implementation of R5s220107 is not aligned with Prose 38.508 Table 4.5.2.2-4 (step 1a1 and 1b1). With current implementation the steps 8 and 9 will get executed whether PDU session is established or not which is incorrect and step 2-9a1 should be skipped in case there is no PDU session establishment. |
| Summary of change | If PDU session is not established then skip the steps 8 and 9 in function f_NR5GC_RRC_Idle_Extension |
| TTCN module | Common\NR5GC\NR5GC_NASSteps.ttcn |
| MCC160 Comments | Accepted |

Before Change

function f_NR5GC_RRC_Idle_Extension_Steps1_7(NR_CellId_Type p_NR_CellId, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC return boolean { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ var PDU_PDN_DNN_Type v_PDN_Type; var charstring v_DNN := ""; timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@ select (v_PDN_Type) { case (Internet_DNN, None) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } } f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@ } else { // @sic R5s210084 sic@ // if an establishment request message hasn't been sent, or triggered, then don't need to carry on return; } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, p_ServiceType)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ }

After Change

function f_NR5GC_RRC_Idle_Extension_Steps1_7(NR_CellId_Type p_NR_CellId, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC return boolean { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ var PDU_PDN_DNN_Type v_PDN_Type; var charstring v_DNN := ""; timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@ select (v_PDN_Type) { case (Internet_DNN, None) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } } f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@ } else { // @sic R5s210084 sic@ // if an establishment request message hasn't been sent, or triggered, then don't need to carry on return false; } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, p_ServiceType)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ return true; }

## Change 2

| Function name | f_NR5GC_RRC_Idle_Extension |
| --- | --- |
| Reason for change | Current implementation of R5s220107 is not aligned with Prose 38.508 Table 4.5.2.2-4 (step 1a1 and 1b1). With current implementation the steps 8 and 9 will get executed whether PDU session is established or not which is incorrect and step 2-9a1 should be skipped in case there is no PDU session establishment. |
| Summary of change | If PDU session is not established then skip the steps 8 and 9 in function f_NR5GC_RRC_Idle_Extension |
| TTCN module | Common\NR5GC\NR5GC_NASSteps.ttcn |
| MCC160 Comments | Accepted |

Before Change

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { // Split into steps1_7 @sic R5s220107 sic@ f_NR5GC_RRC_Idle_Extension_Steps1_7(p_NR_CellId, // TTCN CR R5s220107 MCC p_ServiceType); // Step 8 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } }

After Change

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // Split into steps1_7 @sic R5s220107 sic@ if (f_NR5GC_RRC_Idle_Extension_Steps1_7(p_NR_CellId, p_ServiceType)) { // Step 8 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } } }