3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220458

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2439 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC RRC test case 8.1.5.7.1.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of NR5GC RRC test case 8.1.5.7.1.1 in FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.5.7.1.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.7.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	9

2.4	MCC160 Additional change – Correction to f_TC_8_1_5_7_1_X_NR5GC_TestBody	13

3	Branches executed	15

4	Execution Log Files	15

4.1	Hisilicon Balong 5000 UE	15

5	References	15

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.5.7.1.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Ting Gao
	gaoting@starpointcomm.com

## Verification Test Summary

Test Case: 	8.1.5.7.1.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk49

System Simulator used:	Starpoint SP9500 Test System

UE used:	Hisilicon Balong 5000 UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_7_1_X_NR5GC_TestBody |
| --- | --- |
| Reason for change | The function f_SS_CellConfig_ActivateSCell_CA( ) is incorrectly called for p_SCell, when it should be for p_PCell. |
| Summary of change | Change the p_SCell to p_PCell. |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {NotACK_NextRLC_PDU := Start}}; var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := lengthof(v_ListOfDRBs); //DRB is used ONLY for SS configuration! var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigListNot_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode)}; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true); //CA UL enabled //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_SCell); // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); ... }

After Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {NotACK_NextRLC_PDU := Start}}; var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := lengthof(v_ListOfDRBs); //DRB is used ONLY for SS configuration! var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigListNot_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode)}; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true); //CA UL enabled //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); ... }

## Change 2

| Function name | f_TC_8_1_5_7_1_X_NR5GC_TestBody |
| --- | --- |
| Reason for change | Due to the absence of UL Grant, UE can not loop back PDCP SDU on Scell in step 9A, so it is necessary to add UL Grant on Scell. |
| Summary of change | Add UL Grant on Scell for UE |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment | Rejected.UL grants for Scell is already provided in Step 2. |

Before Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {NotACK_NextRLC_PDU := Start}}; var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := lengthof(v_ListOfDRBs); //DRB is used ONLY for SS configuration! var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigListNot_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode)}; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true); //CA UL enabled //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_SCell); // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_38508_DRB_ToAddMod(v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), -, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ f_NR5GC_CloseUE_TestLoopModeA (p_PCell,cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling); //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments for the next RLC PDU on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigListNot_Ack); //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); ... }

After Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {NotACK_NextRLC_PDU := Start}}; var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := lengthof(v_ListOfDRBs); //DRB is used ONLY for SS configuration! var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigListNot_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode)}; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true); //CA UL enabled //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_SCell); // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_38508_DRB_ToAddMod(v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), -, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ f_NR5GC_CloseUE_TestLoopModeA (p_PCell,cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling); //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments for the next RLC PDU on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigListNot_Ack); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4096, v_NR_UplinkBWP); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(p_SCell, -, v_NR_ResourceAllocation, 20, 10); //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); ... }

## Change 3

| Function name | f_TC_8_1_5_7_1_1_NR5GC |
| --- | --- |
| Reason for change | To ensure UE treated fairly, the MCG RLC acknowledgments for the next RLC PDU should be prohibited until UE has looped back PDCP PDU. |
| Summary of change | Change NR_RLC_TestModeConfig_Type from NotACK_NextRLC_PDU := Start to AckProhibit := Prohibit. |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment | Postponed, further clarification needed and conditional to prose CR agreement at RAN5#95. |

Before Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {NotACK_NextRLC_PDU := Start}}; var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := lengthof(v_ListOfDRBs); //DRB is used ONLY for SS configuration! var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigListNot_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode)}; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true); //CA UL enabled //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_SCell); // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_38508_DRB_ToAddMod(v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), -, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ f_NR5GC_CloseUE_TestLoopModeA (p_PCell,cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling); //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments for the next RLC PDU on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigListNot_Ack); //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_SCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} } //@siclog "Step 10" siclog@ //Check: Does the UE transmit a FailureInformation message with failureType set to rlc-failure? /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_FailureInformation(tsc_NR_CellGroupId_MCG,-,rlc_failure), -, crs_MacBearerRouting(p_PCell))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Steps 11 - 12" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //@sic R5-214617 sic@ }

After Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {AckProhibit := Prohibit}}; var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := lengthof(v_ListOfDRBs); //DRB is used ONLY for SS configuration! var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigListNot_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode)}; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true); //CA UL enabled //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_SCell); // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_38508_DRB_ToAddMod(v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), -, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ f_NR5GC_CloseUE_TestLoopModeA (p_PCell,cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling); //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments for the next RLC PDU on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigListNot_Ack); //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_SCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} } v_SS_Drb_ConfigListNot_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, {Info := {AckProhibit := Continue}}), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, {Info := {AckProhibit := Continue}})}; f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigListNot_Ack); //@siclog "Step 10" siclog@ //Check: Does the UE transmit a FailureInformation message with failureType set to rlc-failure? /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_FailureInformation(tsc_NR_CellGroupId_MCG,-,rlc_failure), -, crs_MacBearerRouting(p_PCell))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Steps 11 - 12" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //@sic R5-214617 sic@ }

## MCC160 Additional change – Correction to f_TC_8_1_5_7_1_X_NR5GC_TestBody

| Function name | f_TC_8_1_5_7_1_X_NR5GC_TestBody |
| --- | --- |
| Reason for change | DRB Data should be received at PCell instead of SCell. |
| Summary of change | PCell Replaced by SCell parameter |
| TTCN module | RRC_Others_CA_NR5GC |

Before Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { .. //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_SCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} } //@siclog "Step 10" siclog@ //Check: Does the UE transmit a FailureInformation message with failureType set to rlc-failure? /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_FailureInformation(tsc_NR_CellGroupId_MCG,-,rlc_failure), -, crs_MacBearerRouting(p_PCell))); … }

After Change

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { .. //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} } …

# Branches executed

The test case was executed on NR band CA_n41C using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Hisilicon Balong 5000 UE

The Hisilicon Balong 5000 UE passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- \TC_8_1_5_7_1_1_NR5GC_PASS.spm.

- \PCT_PICS_PIXIT.sppar

# References

| [1] | R5s220459: Supporting information for agreement of NR5GC RRC test case 8.1.5.7.1.1 in FR1. |
| --- | --- |