3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220471

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2447 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_Get_NG_PDUSessionEstablishmentAccept |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 22wk12 implementation , in the function f_Get_NG_PDUSessionEstablishmentAccept , v_QoSFlowDescr is not updated for the case when there is a need to build a specific QoS flow |  |  |  |  |  |  |  |  |
| Summary of change: |  | v_QoSFlowDescr is updated correctly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case and test case execution will result in run time issue (due to v_QoSFlowDescr being uninitialized ) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the cases where additional Pdu Session Establishment procedure happens (Example : 8.1.2.1.1.NR5GC) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_Get_NG_PDUSessionEstablishmentAccept ()	2

# Corrections required

## Correction to function f_Get_NG_PDUSessionEstablishmentAccept ()

| Function name | f_Get_NG_PDUSessionEstablishmentAccept |
| --- | --- |
| Reason for change | In 22wk12 implementation , in the function f_Get_NG_PDUSessionEstablishmentAccept , v_QoSFlowDescr is not updated for the case when there is a need to build a specific QoS flow |
| Summary of change | v_QoSFlowDescr is updated correctly |
| TTCN module | ttcn-3/ttcn/develop/Common/NG_NAS/NG_NASTemplateFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_Get_NG_PDUSessionEstablishmentAccept(GSM_MobilityInfo_Type p_GSM_MobilityInfo, GMM_MobilityInfo_Type p_GMM_MobilityInfo, template (value) DNN p_DNN, // @sic R5-205939 sic@ template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) PDU_SessionType p_PDU_SessionType := omit, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) S_NSSAI_Type p_S_NSSAI := omit, // @sic R5s190919, R5-211168 sic@ template (omit) AlwaysOnPDUSessionInd p_AlwaysOnPDUSessionInd := omit, template (omit) EAP_Message p_EAP := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPCO := omit, template (omit) GPRS_Timer3 p_BackOff := omit, template (omit) charstring p_BuildQosFlow := omit, boolean p_InterworkWithEPS := true, template (omit) NGSM_NetworkFeatureSupport p_NGSM_NtwkFeatSupport := omit, // @sic R5s201387 Baseline Moving sic@ template (omit) ServingPLMNRateControl p_ServingPLMNRateCtrl := omit, template (omit) ATSSSContainer p_ATSSSContainer := omit, template (omit) ControlPlaneInd p_ControlPlaneInd := omit, template (omit) IPHeaderCompressionConfig p_IPHeaderCompConfig := omit, template (omit) EthernetHeaderCompressConfig p_EthernetHeaderCompConfig := omit, template (omit) PDN_AddressInfo_Type p_ExtraPDN_Address := omit) return template (value) NG_NAS_DL_Pdu_Type { var template (value) NG_NAS_DL_Message_Type v_GSMMsg; var template (value) PDU_SessionType v_PDU_SessionType; var B3_Type v_PDN_TypeToBeUsed; var ListOfPDN_AddressInfo_Type v_PDN_Addresses := {}; var template (omit) PDU_Address v_PDU_Address := omit; var template (omit) AlwaysOnPDUSessionInd v_AlwaysOn := omit; var template (value) QoS_Rules v_QoS_Rules := f_BuildDefaultQoSRules ("1"); // @sic R5-202585 sic@ var template (value) QoSFlowDescr v_QoSFlowDescr; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO; var template (omit) MappedEPSBearerContexts v_MappedEPSBearerContexts := omit; var template (value) MappedEPSContext v_MappedEPSContext; var bitstring v_EPSParam; // If used var O1_Type v_EPSBearerId := '00'O; // If used var boolean v_InterworkWithEPS := false; var template (value) QoS_Flow v_QoS_Flow; var octetstring v_ParamOctets; var template (value) S_NSSAI_Type v_S_NSSAI; if (isvalue(p_PDU_SessionType)) { v_PDU_SessionType := valueof(p_PDU_SessionType); } else { v_PDU_SessionType := cs_PDU_SessionType (p_GSM_MobilityInfo.SessionType); } if (isvalue(p_GMM_MobilityInfo.GMMCap) and p_InterworkWithEPS) { // @sic R5s190464 R5s191172, R5s201367 sic@ if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { // @sic R5-195328 sic@ v_InterworkWithEPS := true; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // @sic R5-204400 sic@ v_EPSBearerId := hex2oct(int2hex(valueof(p_GSM_MobilityInfo.EPS_Bearer), 1) & '0'H); } else { FatalError(__FILE__, __LINE__, "EPS Supported but no bearer id calculated"); } if (p_GSM_MobilityInfo.PDUType == IMS_DNN) { // @sic R5-205939 sic@ v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_2)); } else { v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_1)); } v_ParamOctets := bit2oct(v_EPSParam); // @sic R5s210302 sic@ // This is only correct if the mappedEPSContext contains EPSQoS v_MappedEPSContext := cs_OneMappedEPSContext(v_EPSBearerId, lengthof(v_ParamOctets), cs_EPSParameter_QoS(v_ParamOctets)); // @sic R5s210302 sic@ v_MappedEPSBearerContexts := cs_OneMappedEPSBearerContextList (v_MappedEPSContext); } } if (isvalue(p_QoS_Rules)) { v_QoS_Rules := valueof(p_QoS_Rules); } else { select(p_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case (IMS_DNN) { // @sic R5-205939 sic@ v_QoS_Rules := f_BuildDefaultQoSRules ("2"); // @sic R5-202585 sic@ } case (URLLC_DNN) { // @sic R5-221467 sic@ v_QoS_Rules := f_BuildDefaultQoSRules ("8"); } } } // else use QoS 1 already defined above if (isvalue(p_QoSFlowDescr)) { // @sic R5-194896 sic@ v_QoSFlowDescr := valueof(p_QoSFlowDescr); } else { if (isvalue(p_BuildQosFlow)) { // @sic R5-202550 sic@ // need to build a specific QoS flow v_QoS_Flow := f_BuildDefaultQosFlow(valueof(p_BuildQosFlow), v_InterworkWithEPS, v_EPSBearerId); } else{ select(p_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case (IMS_DNN) { // @sic R5-205939 sic@ v_QoS_Flow := cs_QoS_Flow2(v_InterworkWithEPS, v_EPSBearerId); // @sic R5-204400, R5s201323 sic@ } case (URLLC_DNN) { // @sic R5-221467 sic@ v_QoS_Flow := f_BuildDefaultQosFlow ("7"); } case else { v_QoS_Flow := cs_QoS_Flow1(v_InterworkWithEPS, v_EPSBearerId); // @sic R5-204400, R5s201323 sic@ } } v_QoSFlowDescr := cs_QoSFlowDescr ({v_QoS_Flow}); } } << SKIPPED CODE >> }

After Change:

function f_Get_NG_PDUSessionEstablishmentAccept(GSM_MobilityInfo_Type p_GSM_MobilityInfo, GMM_MobilityInfo_Type p_GMM_MobilityInfo, template (value) DNN p_DNN, // @sic R5-205939 sic@ template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) PDU_SessionType p_PDU_SessionType := omit, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) S_NSSAI_Type p_S_NSSAI := omit, // @sic R5s190919, R5-211168 sic@ template (omit) AlwaysOnPDUSessionInd p_AlwaysOnPDUSessionInd := omit, template (omit) EAP_Message p_EAP := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPCO := omit, template (omit) GPRS_Timer3 p_BackOff := omit, template (omit) charstring p_BuildQosFlow := omit, boolean p_InterworkWithEPS := true, template (omit) NGSM_NetworkFeatureSupport p_NGSM_NtwkFeatSupport := omit, // @sic R5s201387 Baseline Moving sic@ template (omit) ServingPLMNRateControl p_ServingPLMNRateCtrl := omit, template (omit) ATSSSContainer p_ATSSSContainer := omit, template (omit) ControlPlaneInd p_ControlPlaneInd := omit, template (omit) IPHeaderCompressionConfig p_IPHeaderCompConfig := omit, template (omit) EthernetHeaderCompressConfig p_EthernetHeaderCompConfig := omit, template (omit) PDN_AddressInfo_Type p_ExtraPDN_Address := omit) return template (value) NG_NAS_DL_Pdu_Type { var template (value) NG_NAS_DL_Message_Type v_GSMMsg; var template (value) PDU_SessionType v_PDU_SessionType; var B3_Type v_PDN_TypeToBeUsed; var ListOfPDN_AddressInfo_Type v_PDN_Addresses := {}; var template (omit) PDU_Address v_PDU_Address := omit; var template (omit) AlwaysOnPDUSessionInd v_AlwaysOn := omit; var template (value) QoS_Rules v_QoS_Rules := f_BuildDefaultQoSRules ("1"); // @sic R5-202585 sic@ var template (value) QoSFlowDescr v_QoSFlowDescr; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO; var template (omit) MappedEPSBearerContexts v_MappedEPSBearerContexts := omit; var template (value) MappedEPSContext v_MappedEPSContext; var bitstring v_EPSParam; // If used var O1_Type v_EPSBearerId := '00'O; // If used var boolean v_InterworkWithEPS := false; var template (value) QoS_Flow v_QoS_Flow; var octetstring v_ParamOctets; var template (value) S_NSSAI_Type v_S_NSSAI; if (isvalue(p_PDU_SessionType)) { v_PDU_SessionType := valueof(p_PDU_SessionType); } else { v_PDU_SessionType := cs_PDU_SessionType (p_GSM_MobilityInfo.SessionType); } if (isvalue(p_GMM_MobilityInfo.GMMCap) and p_InterworkWithEPS) { // @sic R5s190464 R5s191172, R5s201367 sic@ if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { // @sic R5-195328 sic@ v_InterworkWithEPS := true; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // @sic R5-204400 sic@ v_EPSBearerId := hex2oct(int2hex(valueof(p_GSM_MobilityInfo.EPS_Bearer), 1) & '0'H); } else { FatalError(__FILE__, __LINE__, "EPS Supported but no bearer id calculated"); } if (p_GSM_MobilityInfo.PDUType == IMS_DNN) { // @sic R5-205939 sic@ v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_2)); } else { v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_1)); } v_ParamOctets := bit2oct(v_EPSParam); // @sic R5s210302 sic@ // This is only correct if the mappedEPSContext contains EPSQoS v_MappedEPSContext := cs_OneMappedEPSContext(v_EPSBearerId, lengthof(v_ParamOctets), cs_EPSParameter_QoS(v_ParamOctets)); // @sic R5s210302 sic@ v_MappedEPSBearerContexts := cs_OneMappedEPSBearerContextList (v_MappedEPSContext); } } if (isvalue(p_QoS_Rules)) { v_QoS_Rules := valueof(p_QoS_Rules); } else { select(p_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case (IMS_DNN) { // @sic R5-205939 sic@ v_QoS_Rules := f_BuildDefaultQoSRules ("2"); // @sic R5-202585 sic@ } case (URLLC_DNN) { // @sic R5-221467 sic@ v_QoS_Rules := f_BuildDefaultQoSRules ("8"); } } } // else use QoS 1 already defined above if (isvalue(p_QoSFlowDescr)) { // @sic R5-194896 sic@ v_QoSFlowDescr := valueof(p_QoSFlowDescr); } else { if (isvalue(p_BuildQosFlow)) { // @sic R5-202550 sic@ // need to build a specific QoS flow v_QoS_Flow := f_BuildDefaultQosFlow(valueof(p_BuildQosFlow), v_InterworkWithEPS, v_EPSBearerId); } else{ select(p_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case (IMS_DNN) { // @sic R5-205939 sic@ v_QoS_Flow := cs_QoS_Flow2(v_InterworkWithEPS, v_EPSBearerId); // @sic R5-204400, R5s201323 sic@ } case (URLLC_DNN) { // @sic R5-221467 sic@ v_QoS_Flow := f_BuildDefaultQosFlow ("7"); } case else { v_QoS_Flow := cs_QoS_Flow1(v_InterworkWithEPS, v_EPSBearerId); // @sic R5-204400, R5s201323 sic@ } } //v_QoSFlowDescr := cs_QoSFlowDescr ({v_QoS_Flow}); //Removed and moved down } v_QoSFlowDescr := cs_QoSFlowDescr ({v_QoS_Flow}); } << SKIPPED CODE >> }