3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220489

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2463 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current test case the function f_NR5GC_RRCReestablishment_Steps0To2 is used to handle the re-establishment procedure, this results in updating the C-RNTI and also providing the DL/UL DCCH configuration to use CSS search space. Since this is not a successful reestablishment procedure, this reconfiguration is not required as we only need to handle RRCRestablishment. Therefore, it is proposed to handle the RRCRestablishment directly and reset the SRBs/DRBs on Cell1 for the subsequent connections |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the code to handle the reestablishment message in the test case and reset SRBs / DRBs for Cell 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Corrections required to test case 8.1.6.1.3.3	2

1.1	f_TC_8_1_6_1_3_3_TestBody	2

- Corrections required to test case 8.1.6.1.3.3

  - f_TC_8_1_6_1_3_3_TestBody

| Function name | f_TC_8_1_6_1_3_3_TestBody |
| --- | --- |
| Reason for change | In current test case the function f_NR5GC_RRCReestablishment_Steps0To2 is used to handle the re-establishment procedure, this results in updating the C-RNTI and also providing the DL/UL DCCH configuration to use CSS search space. Since this is not a successful reestablishment procedure, this reconfiguration is not required as we only need to handle RRCRestablishment. Therefore, it is proposed to handle the RRCRestablishment directly and reset the SRBs/DRBs on Cell1 for the subsequent connections |
| Summary of change | Modified the code to handle the reestablishment message in the test case and reset SRBs / DRBs for Cell 1 |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC |
| MCC160 Comment | 1. Accepted for handling the RRCRestablishment directly. 2. Not needed for resetting SRBs / DRBs for Cell 1. T301 is started after this resetting may cause test case failure when the T301 expires in the UE earlier than TTCN and RRCSetupRequest is received early. Since Cell1 is not be used anymore after step1, the resetting SRBs / DRBs for Cell 1 is not necessary. Additional proposed implementation, TC 8.1.6.1.3.3 needs to be added attribute EarlyContentionResolution due to the requirements for RRCReestablishment/RRCSetup. |

Before Change:

function fl_TC_8_1_6_1_3_3_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var RNTI_Value v_RNTI_Value_Cell2; // @sic R5-216290, R5-221579 sic@ var PhysCellId v_PhysCellId_Cell2 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); var ARFCN_ValueNR v_SSB_Freq_Cell2 := f_NR_CellInfo_GetFrequencySSB(nr_Cell2); var CellIdentity v_CellIdentity_Cell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var PLMN_Identity v_PLMN_Identity_Cell2 := f_NR_CellInfo_GetPLMN (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var TrackingAreaCode v_TAC_Cell2 := f_NR_CellInfo_GetTAC (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var CellIdentity v_CellIdentity_Cell4 := f_NR_CellInfo_GetCellIdentity(nr_Cell4); var PLMN_Identity v_PLMN_Identity_Cell4 := f_NR_CellInfo_GetPLMN (nr_Cell4); var TrackingAreaCode v_TAC_Cell4 := f_NR_CellInfo_GetTAC (nr_Cell4); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var integer v_WaitTime := 1;//T301 is ms1000 by default // @sic R5s211600 sic@ var float v_T301Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_WaitTime)); // @sic R5s211600 sic@ //@siclog "Step 1" siclog@ //The SS changes NR Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 3. f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell2, otherFailure); //@sic R5-221467, R5-221579 sic@ v_RNTI_Value_Cell2 := f_NR_CellInfo_GetRNTI(nr_Cell2); // @sic R5-216290, R5-221579 sic@ //@siclog "Step 3" siclog@ //The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 3. f_Delay(v_T301Min); // @sic R5s211600 sic@ //@siclog "Steps 4-5" siclog@ //Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR_RRC_SetupRequest_Def(nr_Cell2); // @sic R5-221579 sic@ f_NR_RRCSetup_Def(nr_Cell2); //@siclog "Step 6" siclog@

After Change:

module RRC_MDT_IntraNR_RLF_NR5GC { … import from NR5GC_RRC_Templates all; … … function fl_TC_8_1_6_1_3_3_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var RNTI_Value v_RNTI_Value_Cell2, v_RNTI_Value; // @sic R5-216290, R5-221579 sic@ var PhysCellId v_PhysCellId_Cell2 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); var PhysCellId v_PhysCellId_Cell1 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ARFCN_ValueNR v_SSB_Freq_Cell2 := f_NR_CellInfo_GetFrequencySSB(nr_Cell2); var CellIdentity v_CellIdentity_Cell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var PLMN_Identity v_PLMN_Identity_Cell2 := f_NR_CellInfo_GetPLMN (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var TrackingAreaCode v_TAC_Cell2 := f_NR_CellInfo_GetTAC (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var CellIdentity v_CellIdentity_Cell4 := f_NR_CellInfo_GetCellIdentity(nr_Cell4); var PLMN_Identity v_PLMN_Identity_Cell4 := f_NR_CellInfo_GetPLMN (nr_Cell4); var TrackingAreaCode v_TAC_Cell4 := f_NR_CellInfo_GetTAC (nr_Cell4); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var integer v_WaitTime := 1;//T301 is ms1000 by default // @sic R5s211600 sic@ var float v_T301Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_WaitTime)); // @sic R5s211600 sic@ var ShortMAC_I v_ShortMAC_I; var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@siclog "Step 1" siclog@ //The SS changes NR Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 3. //f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell2, otherFailure); //@sic R5-221467, R5-221579 sic@ // handle reestablishment procedure and reset DRB/SRBs without any changes to grant v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); v_ShortMAC_I:= f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2); SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId_Cell1, v_ShortMAC_I), otherFailure))); v_RNTI_Value_Cell2 := f_NR_CellInfo_GetRNTI(nr_Cell2); // @sic R5-216290, R5-221579 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit, true); //@siclog "Step 3" siclog@ //The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 3. f_Delay(v_T301Min); // @sic R5s211600 sic@ //@siclog "Steps 4-5" siclog@ //Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR_RRC_SetupRequest_Def(nr_Cell2); // @sic R5-221579 sic@ f_NR_RRCSetup_Def(nr_Cell2); //@siclog "Step 6" siclog@

Additional comments from MCC160

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean

{

select (p_Testcase) {

case ("TC_11_3_2_NR5GC") { return true; }

case ("TC_11_3_6_NR5GC") { return true; }

case ("TC_6_4_1_1_NR5GC") { return true; }

case ("TC_6_4_2_2_NR5GC") { return true; }

case ("TC_7_1_1_1_3_NR5GC") { return true; }

case ("TC_7_1_1_1_5_NR5GC") { return true; }

case ("TC_8_1_1_1_1_NR5GC") { return true; }

case ("TC_8_1_1_2_1_NR5GC") { return true; }

case ("TC_8_1_1_2_3_NR5GC") { return true; }

case ("TC_8_1_1_4_1_NR5GC") { return true; }

case ("TC_8_1_1_4_2_NR5GC") { return true; }

case ("TC_8_1_2_1_4_NR5GC") { return true; }

case ("TC_8_1_3_1_23_NR5GC") { return true; }

case ("TC_8_1_4_1_5_NR5GC") { return true; }

case ("TC_8_1_4_1_6_NR5GC") { return true; }

case ("TC_8_1_5_6_1_NR5GC") { return true; }

case ("TC_8_1_5_8_1_NR5GC") { return true; }

case ("TC_8_1_5_8_2_1_NR5GC") { return true; }

case ("TC_8_1_5_8_2_2_NR5GC") { return true; }

case ("TC_8_1_5_8_2_3_NR5GC") { return true; }

case ("TC_8_1_6_1_2_7_NR5GC") { return true; }

case ("TC_8_1_6_1_3_3_NR5GC") { return true; }

case ("TC_8_1_6_1_4_1_NR5GC") { return true; }

case ("TC_8_1_6_1_4_2_NR5GC") { return true; }

case ("TC_8_1_6_1_4_3_NR5GC") { return true; }

case ("TC_8_1_6_1_4_4_NR5GC") { return true; }

case ("TC_8_1_6_1_4_5_NR5GC") { return true; }

case ("TC_8_1_6_1_4_6_NR5GC") { return true; }

case ("TC_8_1_6_1_4_7_NR5GC") { return true; }

}

return false;

}