3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220490

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2464 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.1.4.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SSB frequency that is checked in UE Information response message for neighbour cell shall be based on Cell3, as this is the neighbour cell that is configured in this test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the cellid in the the get function call to cell3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Corrections required to test case 8.1.6.1.4.7	2

1.1	f_TC_8_1_6_1_4_7_TestBody	2

- Corrections required to test case 8.1.6.1.4.7

  - f_TC_8_1_6_1_4_7_TestBody

| Function name | f_TC_8_1_6_1_4_7_TestBody |
| --- | --- |
| Reason for change | The SSB frequency that is checked in UE Information response message for neighbour cell shall be based on Cell3, as this is the neighbour cell that is configured in this test case |
| Summary of change | Changed the cellid in the the get function call to cell3 |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_6_1_4_7_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_PLMN_Identity_Cell1; var CellIdentity v_CellIdentity_Cell1; var PhysCellId v_PhysCellId_Cell3;//@sic R5-217925 sic@ var template MeasResultList2NR_r16 v_MeasResult2NR_r16; var template ARFCN_ValueNR v_SsbFrequency_r16;//@sic R5-221570 sic@ … … … //@siclog "Step 16" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_FAIL)); v_SsbFrequency_r16 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_MeasResult2NR_r16 := cr_MeasResultNeighCellListNR_1Entry(v_SsbFrequency_r16,*,cr_MeasResultListNR_1Entry(v_PhysCellId_Cell3));//@sic R5-217925 R5-221570sic@ //@siclog "Step 17" siclog@ //Check: Does the UE send a UEInformationResponse message with connEstFailReport on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def, -, -, -, cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1)), v_MeasResult2NR_r16)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); }//end

After Change:

function f_TC_8_1_6_1_4_7_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_PLMN_Identity_Cell1; var CellIdentity v_CellIdentity_Cell1; var PhysCellId v_PhysCellId_Cell3;//@sic R5-217925 sic@ var template MeasResultList2NR_r16 v_MeasResult2NR_r16; var template ARFCN_ValueNR v_SsbFrequency_r16;//@sic R5-221570 sic@ … … … //@siclog "Step 16" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_FAIL)); v_SsbFrequency_r16 := f_NR_CellInfo_GetFrequencySSB(nr_Cell3); // cell3 SSB v_MeasResult2NR_r16 := cr_MeasResultNeighCellListNR_1Entry(v_SsbFrequency_r16,*,cr_MeasResultListNR_1Entry(v_PhysCellId_Cell3));//@sic R5-217925 R5-221570sic@ //@siclog "Step 17" siclog@ //Check: Does the UE send a UEInformationResponse message with connEstFailReport on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def, -, -, -, cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1)), v_MeasResult2NR_r16)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); }//end of f_TC_8_1_6_1_4_7_NR5GC_TestBody