3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220500

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0805 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS call control test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections for IMS call control video addition / removal test cases. - Request URI in 200OK at step 9 shall be the same as provided by the UE in the Contact header field of the INVITE creating the dialog - SIP UPDATE at step 6 is received from the UE, and hence the SIP URL expectation for Contact header should be done using the RX function f_SIP_BuildSipUri_RX() - The preliminary pass verdict at step 1 has incorrect step number - The UE may also include a port number in a URI - Incorrect operation code is being used for the QoS rules IE in the PDU SESSION MODIFICATION COMMAND for removing the video stream |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the SIP URI sent in 200OK of step 9 - Used the correct function for the expected value of the URI in contact header - Corrected the preliminary pass verdict step number at step 1 - Changed the default value for port to ‘anyOrOmit’ - Removed the check on v_RemoveVoice so that cs_QoS_DeleteRule template is used Note : An associated prose CR on 38.508-1 will be raised at RAN5#95 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.20.NR5GC, 7.22.NR5GC, 7.23.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	7

2.3	Change 3	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_22_NR5GC_IMS1() |
| --- | --- |
| Reason for change | - Request URI in 200OK at step 9 shall be the same as provided by the UE in the Contact header field of the INVITE creating the dialog - SIP UPDATE at step 6 is received from the UE, and hence the SIP URL expectation for Contact header should be done using the RX function f_SIP_BuildSipUri_RX() - The preliminary pass verdict at step 1 has incorrect step number |
| Summary of change | - Corrected the SIP URI sent in 200OK of step 9 - Used the correct function for the expected value of the URI in contact header - Corrected the preliminary pass verdict step number at step 1 |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | - A SIP response does not have any Request URI and the INVITE creating the dialog is sent by the SS. Instead the Contact address provided by the SS in the INVITE creating the dialog shall be used by the UE in its requests sent to the SS and it shall be used in responses sent by the SS where applicable (see MCC160 implementation below). - The Request URI of the SIP UPDATE sent to the SS shall be (exactly) the same as provided by the SS in the Contact of the INVITE creating the dialog (see MCC160 implementation below). - Accepted as improvement |

Before Change:

function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); f_IMS_MTCallSetup_NR5GC_A_5_1(); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @siclog "Step 1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo, -, tsc_IMS_SDP_AudioCodecList_EVS_only, tsc_IMS_SDP_VideoCodecList_H265_H264); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress v_ResponseWithSdp := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp, EVS_16000, H265_90000); // @siclog "Step 4 - 5" siclog@ PRACK, 200 OK f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_ResponseWithSdp.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5A" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 6 - 7" siclog@ UPDATE, 200 OK v_LatestSdpOfferFromUE := f_IMS_MOCallSetup_NR5GC_Video_Step6_7(AddVideo, v_InviteRequestWithSdp); v_LatestSdpAnswerFromSS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_LatestSdpOfferFromUE, v_DontIncrementSessVers); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 - 14" siclog@ Release Video f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_LatestSdpAnswerFromSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_SpeechCall); }

After Change:

… … import from IMS_CommonParameters all; … … function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; var charstring v_ContactUrlStrSS; var SipUrl v_ContactSipUrlSS; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); f_IMS_MTCallSetup_NR5GC_A_5_1(); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @siclog "Step 1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo, -, tsc_IMS_SDP_AudioCodecList_EVS_only, tsc_IMS_SDP_VideoCodecList_H265_H264); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 1"); v_ContactSipUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); v_ContactUrlStrSS := f_SIP_SipUrl2Str(v_ContactSipUrlSS); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress v_ResponseWithSdp := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp, EVS_16000, H265_90000); // @siclog "Step 4 - 5" siclog@ PRACK, 200 OK f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_ResponseWithSdp.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5A" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 6 - 7" siclog@ UPDATE, 200 OK v_LatestSdpOfferFromUE := f_IMS_MOCallSetup_NR5GC_Video_Step6_7(AddVideo, v_InviteRequestWithSdp, f_SIP_BuildSipUri_RX(px_IMS_CalleeContactUri, *)); v_LatestSdpAnswerFromSS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_LatestSdpOfferFromUE, v_DontIncrementSessVers); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp, v_ContactUrlStrSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 - 14" siclog@ Release Video f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_LatestSdpAnswerFromSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_SpeechCall); }

## MCC160 Implementation

function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; var charstring v_ContactUrlStrSS; var SipUrl v_ContactSipUrlSS; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1(); v_ContactSipUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); v_ContactUrlStrSS := f_SIP_SipUrl2Str(v_ContactSipUrlSS); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @siclog "Step 1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo, v_ContactSipUrlSS, tsc_IMS_SDP_AudioCodecList_EVS_only, tsc_IMS_SDP_VideoCodecList_H265_H264); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 1"); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress v_ResponseWithSdp := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp, EVS_16000, H265_90000, v_ContactUrlStrSS); // @siclog "Step 4 - 5" siclog@ PRACK, 200 OK f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_ResponseWithSdp.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5A" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 6 - 7" siclog@ UPDATE, 200 OK v_LatestSdpOfferFromUE := f_IMS_MOCallSetup_NR5GC_Video_Step6_7(AddVideo, v_InviteRequestWithSdp, v_ContactSipUrlSS); v_LatestSdpAnswerFromSS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_LatestSdpOfferFromUE, v_DontIncrementSessVers); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp, v_ContactUrlStrSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 - 14" siclog@ Release Video f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_LatestSdpAnswerFromSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_SpeechCall); }

## Change 2

| Function name | f_SIP_BuildSipUri_RX() |
| --- | --- |
| Reason for change | The UE may also include a port number in a URI |
| Summary of change | Changed the default value for port to ‘anyOrOmit’ |
| TTCN module | IMS_SIP_Templates.ttcn |
| MCC160 Comment | Rejected: In cases where it is allowed that the UE comes up with or without portnumber this can easily be expressed by handing over '*' (see e.g. implementation of f_IMS_INVITE_CalleeUriRX). In addition the proposed change would affect more than 60 places where f_SIP_BuildSipUri_RX is used. |

Before Change:

function f_SIP_BuildSipUri_RX(charstring p_URI, template integer p_Port := omit, template SemicolonParam_List p_UrlParameters := *, template AmpersandParam_List p_Headers := omit) return template (present) SipUrl { var template (present) SipUrl v_SipUrl := f_SIP_ParseURI(p_URI); if (ischosen(v_SipUrl.components.sip)) { /* @sic R5s130195 BASELINE MOVING 2013: change of SipUrl sic@ @sic R5s140020 change 11: p_Port may be * therefore shall not be checked to be present sic@ */ v_SipUrl.components.sip.hostPort.portField := p_Port; } v_SipUrl.urlParameters := p_UrlParameters; v_SipUrl.headers := p_Headers; /* @sic R5s150382 change 4 sic@ */ return v_SipUrl; }

After Change:

function f_SIP_BuildSipUri_RX(charstring p_URI, template integer p_Port := *, template SemicolonParam_List p_UrlParameters := *, template AmpersandParam_List p_Headers := omit) return template (present) SipUrl { var template (present) SipUrl v_SipUrl := f_SIP_ParseURI(p_URI); if (ischosen(v_SipUrl.components.sip)) { /* @sic R5s130195 BASELINE MOVING 2013: change of SipUrl sic@ @sic R5s140020 change 11: p_Port may be * therefore shall not be checked to be present sic@ */ v_SipUrl.components.sip.hostPort.portField := p_Port; } v_SipUrl.urlParameters := p_UrlParameters; v_SipUrl.headers := p_Headers; /* @sic R5s150382 change 4 sic@ */ return v_SipUrl; }

## Change 3

| Function name | fl_NR5GC_ModifyPDUSession_RemoveIMS_Media() |
| --- | --- |
| Reason for change | Incorrect operation code is being used for the QoS rules IE in the PDU SESSION MODIFICATION COMMAND for removing the video stream |
| Summary of change | Removed the check on v_RemoveVoice so that cs_QoS_DeleteRule template is used Note : An associated prose CR on 38.508-1 will be raised at RAN5#95 |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change:

function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType) runs on NR5GC_PTC { … … // @sic R5-216150 sic@ if (v_RemoveVoice) { // @sic R5-217798 sic@ v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@ j := j+1; } if (v_RemoveVideo) { if (v_RemoveVoice) { v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O); } else { // @sic R5-217798 sic@ FFS: error in prose v_QoS_Rules[j] := cs_QoS_ModifyRule('04'O); } v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B); } … … }

After Change:

function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType) runs on NR5GC_PTC { … … // @sic R5-216150 sic@ if (v_RemoveVoice) { // @sic R5-217798 sic@ v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@ j := j+1; } if (v_RemoveVideo) { //REMOVED if (v_RemoveVoice){ v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O); /* REMOVED } else { // @sic R5-217798 sic@ FFS: error in prose v_QoS_Rules[j] := cs_QoS_ModifyRule('04'O); }*/ v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B); } … … }