3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220503

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2475 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.523-1 table 7.1.3.0-2, the primary DRB for testing in the PDCP test cases is the default DRB of the 1st PDU session on NR cell. The UE may have established other DRBs during the preamble, currently these are not being reconfigured at the SS side following the handover in this test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configure all existing DRBs on the SS side during handover, with the 1st DRB modified to use UM RLC and PDCP transparent mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE configured for multi-PDU session operation may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_3_4_2_NR5GC_TestBody() |
| --- | --- |
| Reason for change | According to 38.523-1 table 7.1.3.0-2, the primary DRB for testing in the PDCP test cases is the default DRB of the 1st PDU session on NR cell. The UE may have established other DRBs during the preamble, currently these are not being reconfigured at the SS side following the handover in this test case. |
| Summary of change | Configure all existing DRBs on the SS side during handover, with the 1st DRB modified to use UM RLC and PDCP transparent mode. |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle. The 1st DRB modified to use UM RLC and PDCP transparent mode and SDAP none. |

Before Change:

function fl_TC_7_1_3_4_2_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig ; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (omit) MasterKeyUpdate v_MasterKeyUpdate:= cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); //@sic R5-202618 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode(PDCP_SNLength18), //@sic R5s220124 sic@ cs_SDAP_Configuration_None)}; …………. }

After Change:

function fl_TC_7_1_3_4_2_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig ; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (omit) MasterKeyUpdate v_MasterKeyUpdate:= cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); //@sic R5-202618 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); v_SS_Drb_ConfigList[0] := f_NR_GetRadioBearerConfig_FullUM(nr_Cell1,p_NR_PDCP_State.DRB_Id,-, cs_NR_PDCP_RbConfig_TransparentMode(PDCP_SNLength18)); …………… }