3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220508

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2477 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of NR5GC R16 SON-MDT test case 8.1.6.1.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 SON MDT test case 8.1.6.1.3.7 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Rel-16 SON MDT test case 8.1.6.1.3.7 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	SM8450 + SDX65	3

5	References	4

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.3.7 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Rajanna, Lokesha
	lokesha.rajanna@anritsu@anritsu.com

Harry ,Shaun

shaun.harry@keysight.com

  - Verification Test Summary

Test Case: 	8.1.6.1.3.7

ATS Version:	iwd-TTCN3-B2020-09_D22wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR, Keysight 5G Protocol Conformance Toolset

UE used:	SM8450 + SDX65

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | function f_TC_8_1_6_1_3_7_TestBody() |
| --- | --- |
| Reason for change | Prose is expecting rlf report of nr cell 1 at step 16 on ueInfromationResponse. So RNTI should be used for nr cell1 instead nr cell 12. |
| Summary of change | Updated cell 1 in UEInformationResponse template reception |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_TC_8_1_6_1_3_7_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template (value) DL_DCCH_Message v_RRCReconfiguration; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template RRCReestablishmentComplete_v1610_IEs <<skip code>> -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Steps 16" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 12? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell12, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-, cr_NR_Rlf_Report(f_NR_CellInfo_GetRNTI(nr_Cell12),-, (0..127), (0..127), // rsrp, rsrq omit,// measResultNeighCells_r16 omit,//PreviousPCellId_r16 cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16(cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1)),// failedPCellId_r16 omit,omit,cr_NR_CGI_Info_Logging_r16_Def(v_CellIdentity_NRCell1),*,?, rlf, t310_Expiry,*,omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Steps 17" siclog@ //void @sic R5-216293 sic@ }//End of f_TC_8_1_6_1_3_7_TestBody() }

After Change:

function f_TC_8_1_6_1_3_7_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template (value) DL_DCCH_Message v_RRCReconfiguration; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template RRCReestablishmentComplete_v1610_IEs <<Skip code>> -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Steps 16" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 12? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell12, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-, cr_NR_Rlf_Report(f_NR_CellInfo_GetRNTI(nr_Cell1),-, (0..127), (0..127), // rsrp, rsrq omit,// measResultNeighCells_r16 omit,//PreviousPCellId_r16 cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16(cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1)),// failedPCellId_r16 omit,omit,cr_NR_CGI_Info_Logging_r16_Def(v_CellIdentity_NRCell1),*,?, rlf, t310_Expiry,*,omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Steps 17" siclog@ //void @sic R5-216293 sic@ }//End of f_TC_8_1_6_1_3_7_TestBody() }

- Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

On Keysight, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

- Execution Log Files

  - SM8450 + SDX65

The Qualcomm SM 8450+SDX65 5G UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight 5G Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_6_1_3_7_LOG.xml

Keysight\ TC_8_1_6_1_3_7_LOG

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_6_1_3_7_PIXIT.txt

Keysight\ TC_8_1_6_1_3_7_PIXIT

- References

| [1] | R5s220494- Supporting information for re-verification of NR5GC R16 SON-MDT test case 8.1.6.1.3.7 |
| --- | --- |