3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220547

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2486 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test , 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step4-6, the TTCN is sending the PDCP Data PDU in an iteration every 75ms. While inside the iteration, TTCN also perform additional configuration at SS, which consumes time and sometimes the PDU send APS reach SS too later to be scheduled on time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Scheduling the PDCP data at 100ms interval to increate the stabability of the test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case may have low pass rate with reason not releated to any TP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.NR5GC, 7.1.3.4.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_4_1_NR_Step1to7 |
| --- | --- |
| Reason for change | At step4-6, the TTCN is sending the PDCP Data PDU in an iteration every 75ms. While inside the iteration, TTCN also perform additional configuration at SS, which consumes time and sometimes the PDU send APS reach SS too later to be scheduled on time. |
| Summary of change | Scheduling the PDCP data at 100ms interval to increate the stabability of the test. Note – prose CR required for this change. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#95. |

Before Change

function f_TC_7_1_3_4_1_NR_Step1to7(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var NR_AbsoluteCellPower_Type v_CellPowerAtT1Cell2_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT1Cell2_FR2 := -82; var SubFrameTiming_Type v_TimingInfoStart; var integer k; /*skipping Code*/ v_TimingInfoStart := f_NR_GetNextSendOccasion_DL(nr_Cell1);//@sic R5-201219 sic@ for (k:= 0; k <= 2; k:= k+1) { //@sic R5s190844 sic@ //@siclog "Step 4" siclog@ //Configure the SS to not ACK next RLC PDU @sic R5-190752 ,R5-191296 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@siclog "Step 5" siclog@ //The SS sends the PDCP Data PDU f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData, f_SubFrameTiming_AddMilliSeconds(v_TimingInfoStart, 75*k)); //@sic R5-201219 sic@ //@siclog "Step 6" siclog@ //The UE sends the PDCP Data PDU f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData, cr_RlcBearerRouting_NR(nr_Cell1)); } /*skipping Code*/

After Change

function f_TC_7_1_3_4_1_NR_Step1to7(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var NR_AbsoluteCellPower_Type v_CellPowerAtT1Cell2_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT1Cell2_FR2 := -82; var SubFrameTiming_Type v_TimingInfoStart; var integer k; /*skipping Code*/ v_TimingInfoStart := f_NR_GetNextSendOccasion_DL(nr_Cell1);//@sic R5-201219 sic@ for (k:= 0; k <= 2; k:= k+1) { //@sic R5s190844 sic@ //@siclog "Step 4" siclog@ //Configure the SS to not ACK next RLC PDU @sic R5-190752 ,R5-191296 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@siclog "Step 5" siclog@ //The SS sends the PDCP Data PDU f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData, f_SubFrameTiming_AddMilliSeconds(v_TimingInfoStart, 100*k)); //@sic R5-201219 sic@ //@siclog "Step 6" siclog@ //The UE sends the PDCP Data PDU f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData, cr_RlcBearerRouting_NR(nr_Cell1)); } /*skipping Code*/ }