3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220551

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2490 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following T317 timeout, RRC local release is performed but the default timing delay of 100ms may not be sufficient to process all ASPs before re-starting the UL grant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use the delay constant tsc_NR_DelayBeforeIntraCellHO (300ms) for RRC local release, as used in f_NR_RRCRelease_Common() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_7_1_Body |
| --- | --- |
| Reason for change | Following T317 timeout, RRC local release is performed but the default timing delay of 100ms may not be sufficient to process all ASPs before re-enableing the UL grant. |
| Summary of change | Use the delay constant tsc_NR_DelayBeforeIntraCellHO (300ms) for RRC local release, as used in f_NR_RRCRelease_Common() |
| TTCN module | Service_Request_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_9_1_7_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_GutiParameters_Type v_GutiParams_A := f_NR5GC_CellInfo_GetGutiParameters(v_NGC_NASCellA); var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(v_NGC_NASCellA, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); //Get the QosFlow ID information from the DRB ID var NG_MobileIdentity v_GUTIToSend; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (present) NG_MobileIdentity v_S_TMSI_A := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_A); var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var integer v_Count := 0; //@sic R5-194869 sic@ var integer v_PSI := valueof(v_QosFlowId.PDU_SessionId); //@sic R5s200087 sic@ var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var boolean v_Result := false; //@sic R5s200087 sic@ var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; timer t_T3517 := 15.0; //@sic R5-194869 sic@ timer t_T3525 := 60.0; //@sic R5-194869 sic@ timer t_Wait; //@sic R5-201241 sic@ …………………….. //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { //@sic R5s210839 sic@ v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA)); //@sic R5s200087, R5s200662 sic@ f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //UL grant already configured with activation time v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, -, -, -, -, -, false); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } } } //For loop //@siclog "Step 11" siclog@ ….. }

After Change

function fl_TC_9_1_7_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_GutiParameters_Type v_GutiParams_A := f_NR5GC_CellInfo_GetGutiParameters(v_NGC_NASCellA); var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(v_NGC_NASCellA, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); //Get the QosFlow ID information from the DRB ID var NG_MobileIdentity v_GUTIToSend; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (present) NG_MobileIdentity v_S_TMSI_A := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_A); var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var integer v_Count := 0; //@sic R5-194869 sic@ var integer v_PSI := valueof(v_QosFlowId.PDU_SessionId); //@sic R5s200087 sic@ var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var boolean v_Result := false; //@sic R5s200087 sic@ var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; timer t_T3517 := 15.0; //@sic R5-194869 sic@ timer t_T3525 := 60.0; //@sic R5-194869 sic@ timer t_Wait; //@sic R5-201241 sic@ ………………………….. //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { //@sic R5s210839 sic@ v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA, tsc_NR_DelayBeforeIntraCellHO)); //@sic R5s200087, R5s200662 sic@ f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //UL grant already configured with activation time v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, -, -, -, -, -, false); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } } } //For loop //@siclog "Step 11" siclog@ …….. }