3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220567

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2502 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDCP State Variables need to be incremented at step 2C, 3 and 7. However, in the current implementation, PDCP State Variables are not incremented causing PDCP Data at Steps 2C, 3 and 7 transmitted with PDCP SN set as 0, which is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined new Variable v_NR_PDCP_State which shall be incremented before Steps 2C, 3 and 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.NR5GC, 7.1.3.5.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	10

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_3_5_2_NR_TestBody |
| --- | --- |
| Reason for change | PDCP State Variables need to be incremented at step 2C, 3 and 7. However, in the current implementation, PDCP State Variables are not incremented causing PDCP Data at Steps 2C, 3 and 7 transmitted with PDCP SN set as 0, which is wrong. |
| Summary of change | Defined new Variable NR_PDCP_SS_State_Type v_NR_PDCP_State which shall be incremented before Steps 2C, 3 and 7. |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle. Solution will be addressed as per proposal in CR R5s220613 change 2.4 |

Before Change

function fl_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on NR_BASE_PTC { var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ // SCG NR Cell10, Primary path NR Cell 1, Secondary path NR Cell 10 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell10), "Step2"); //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ //SCG NR Cell10, Primary path NR Cell 10, Secondary path NR Cell 1 //@siclog "Step 2C-2D" siclog@ f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), "Step2D"); f_TC_7_1_3_5_2_Steps3to4b2(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1)); //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,b0,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ f_TC_7_1_3_5_2_Steps7to8b2(p_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1)); }

After Change

function fl_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on NR_BASE_PTC { var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_PDCP_SS_State_Type v_NR_PDCP_State := p_NR_PDCP_State; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ // SCG NR Cell10, Primary path NR Cell 1, Secondary path NR Cell 10 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell10), "Step2"); //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ //SCG NR Cell10, Primary path NR Cell 10, Secondary path NR Cell 1 //@siclog "Step 2C-2D" siclog@ v_NR_PDCP_State.TX_NEXT := p_NR_PDCP_State.TX_NEXT + 1; v_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(v_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), "Step2D"); v_NR_PDCP_State.TX_NEXT := v_NR_PDCP_State.TX_NEXT + 1; v_NR_PDCP_State.RX_NEXT := v_NR_PDCP_State.RX_NEXT + 1; f_TC_7_1_3_5_2_Steps3to4b2(v_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1)); //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,b0,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ v_NR_PDCP_State.TX_NEXT := v_NR_PDCP_State.TX_NEXT + 1; v_NR_PDCP_State.RX_NEXT := v_NR_PDCP_State.RX_NEXT + 1; f_TC_7_1_3_5_2_Steps7to8b2(v_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1)); }

## Change 2

| Function name | fl_TC_7_1_3_5_2_NR_TestBody_ENDC() |
| --- | --- |
| Reason for change | - PDCP State Variables need to be incremented at step 2D, 4a1 and 8a1. However, in the current implementation, PDCP State Variables are not incremented causing PDCP Data at Steps 2D, 4a1 and 8a1 transmitted with PDCP SN set as 0, which is wrong. - Coordination messages between NR and EUTRA thread are missing. After NR PTC sends rrcConnectionReconfigurations request to EUTRA at steps 0B,2a-2b and 5-6, it should wait for trigger from EUTRA before sending PDCP Data Req. - For step 0 when we configure MCG for primary Path , the logical channel id should be 4 instead of 5 for EUTRA Radio bearer |
| Summary of change | - Defined new Variable NR_PDCP_SS_State_Type v_NR_PDCP_State which shall be incremented before Steps 2D, 4a1 and 8a1. - Coordination message added for NR - Modified the Drb_identity in cs_NR_PDCP_Config_UlPath for step 0-0B |
| TTCN module | PDCP_ENDC_NR.ttcn |
| MCC160 Comment | - Accepted in principle see change 1 - Accepted - Accepted in principle |

Before Change

function fl_TC_7_1_3_5_2_NR_TestBody_ENDC(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on ENDC_NR_PTC {// @sic R5-220641 sic@ var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) DRB_ToAddModList v_DRB_ToAddModList; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_MCG))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit);//@siclog "Step 0B" siclog@ //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), "Step2"); //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); //SCG NR Cell 1, Primary path Eutra Cell 1, Secondary path NR Cell 1 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), "Step2D"); f_TC_7_1_3_5_2_Steps3to4b2(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG,b0))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_TC_7_1_3_5_2_Steps7to8b2(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); }

After Change

function fl_TC_7_1_3_5_2_NR_TestBody_ENDC(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on ENDC_NR_PTC {// @sic R5-220641 sic@ var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) NR_PDCP_SS_State_Type v_PDCP_state := p_NR_PDCP_State; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_MCG,-,-,-,tsc_NR_DRB1))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit);//@siclog "Step 0B" siclog@ //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), "Step2"); v_PDCP_state.TX_NEXT := p_NR_PDCP_State.TX_NEXT + 1; v_PDCP_state.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //SCG NR Cell 1, Primary path Eutra Cell 1, Secondary path NR Cell 1 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(v_PDCP_state, cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), "Step2D");//orig v_PDCP_state.TX_NEXT := v_PDCP_state.TX_NEXT + 1; v_PDCP_state.RX_NEXT := v_PDCP_state.RX_NEXT + 1; f_TC_7_1_3_5_2_Steps3to4b2(v_PDCP_state, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); v_PDCP_state.TX_NEXT := v_PDCP_state.TX_NEXT + 1; v_PDCP_state.RX_NEXT := v_PDCP_state.RX_NEXT + 1; //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG,b0))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_TC_7_1_3_5_2_Steps7to8b2(v_PDCP_state, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); }

MCC160 Additional change

function fl_TC_7_1_3_5_2_NR_TestBody_ENDC(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on ENDC_NR_PTC {// @sic R5-220641 sic@ var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) DRB_ToAddModList v_DRB_ToAddModList; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPathOnEUTRA(ms500, len18bits, len18bits,tsc_NR_CellGroupId_MCG))};//@sic R5s211471 R5s220567sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit);//@siclog "Step 0B" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //@sic R5s220567 sic@ .. }

MCC160 Additional change – New template

template (value) PDCP_Config cs_NR_PDCP_Config_UlPathOnEUTRA(NR_PDCP_DiscardTimer_Type p_DiscardTimer, NR_PDCP_SN_UL_Type p_SN_UL, NR_PDCP_SN_DL_Type p_SN_DL, CellGroupId p_CellGroup := tsc_NR_CellGroupId_SCG, UL_DataSplitThreshold p_UlDataSplitThreshold := infinity_, template (omit) NR_PDCP_CipheringDisabled_Type p_CipheringDisabled := omit, //@sic R5s200701 sic@ boolean p_Pdcp_Duplication := false, integer p_DRB_SCG := 2) := // Logical channel Id on EUTRA is DRB + 2. This is different to on NR cs_38508_NR_PDCP_Config (cs_38508_PDCP_Config_DRB(p_DiscardTimer, p_SN_UL, p_SN_DL), cs_NR_PDCP_MoreThanOneRLC(cs_NR_PDCP_PrimaryPath(p_CellGroup, (p_DRB_SCG+2), p_UlDataSplitThreshold, p_Pdcp_Duplication), -, p_CipheringDisabled);

## Change 3

| Function name | function fl_TC_7_1_3_5_2_EUTRA_TestBody() |
| --- | --- |
| Reason for change | - Coordination messages between NR and EUTRA thread are missing. After NR PTC sends rrcConnectionReconfigurations request to EUTRA at steps 0B,2a-2b and 5-6, it should wait for trigger from EUTRA before sending PDCP Data Req. (Change 2 mentioned above). EUTRA Proc should send Coordination message with trigger to NR proc at above steps. - In the current implementation f_EUTRA_NR_WaitForCoOrd_Trigger(NR); is called before steps 2B and 6, this is not required, since there is a Wait for Coordination message inside f_EUTRA38_508_ENDC_Reconfiguration() for the Reconfig message conetents from NR Proc. Hence need to be removed. |
| Summary of change | - Coordination message added to trigger NR thread - Call to f_EUTRA_NR_WaitForCoOrd_Trigger(NR); is removed from step 2B and 6. |
| TTCN module | PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_7_1_3_5_2_EUTRA_TestBody()runs on EUTRA_5GS_PTC { //@siclog "Step 0B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-204426 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); //@siclog "Step 2B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); //@siclog "Step 6" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); }

After Change

function fl_TC_7_1_3_5_2_EUTRA_TestBody()runs on EUTRA_5GS_PTC { //@siclog "Step 0B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-204426 sic@ f_EUTRA_NR_SendCoOrd(NR,cms_EUTRA_NR_Trigger); //@siclog "Step 2B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_SendCoOrd(NR,cms_EUTRA_NR_Trigger); //@siclog "Step 6" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_SendCoOrd(NR,cms_EUTRA_NR_Trigger); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); }