3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220591

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2517 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_TC_7_1_1_3_8_EUTRA_Common |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of function f_TC_7_1_1_3_8_EUTRA_Common , following is called : f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_Only, RRC_IDLE);//@sic R5s211385 sic@ f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TESTMode_OFF, -, -);//@sic R5s211385 sic@ Later in this function following is also called : f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1); Since test mode is set to TESTMode_OFF, no Actiavate RB Test Mode will happen, and no Close UE Test Loop procedure will be executed. Thus, it does not make sense to call the function to Open UE Test Loop. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the function to open UE test loop mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Corrections required	4

1.1	Change 1	4

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_8_EUTRA_Common() |
| --- | --- |
| Reason for change | In the current TTCN implementation of function f_TC_7_1_1_3_8_EUTRA_Common , following is called : f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_Only, RRC_IDLE);//@sic R5s211385 sic@ f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TESTMode_OFF, -, -);//@sic R5s211385 sic@ Later in this function following is also called : f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1); Since test mode is set to TESTMode_OFF, no Actiavate RB Test Mode will happen, and no Close UE Test Loop procedure will be executed. Thus, it does not make sense to call the function to Open UE Test Loop. This needs to be corrected. |
| Summary of change | Remove the function to Open UE test loop mode. |
| TTCN module | MAC_CA_ENDC_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_TC_7_1_1_3_8_EUTRA_Common() runs on EUTRA_5GS_PTC

{

// UE power headroom reporting / SCell activation / DL pathloss change reporting

var template (omit) octetstring v_NR_SCGConfig := omit;

f_EUTRA38_Init_CA(EN_DC, c1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

//Preamble to enter UE into RRC Connected

f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_Only, RRC_IDLE);//@sic R5s211385 sic@

f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TESTMode_OFF, -, -);//@sic R5s211385 sic@

f_EUTRA_TestBody_Set(true);

// Send trigger to NR to test on SCG bearer

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// is this to be done in the preamble and test body is empty ?

//@siclog "Steps 1&2" siclog@

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, -, -, v_NR_SCGConfig, -, -, omit);

//@siclog "Steps 4-5" siclog@

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, -, -, v_NR_SCGConfig, -, -, omit);

//@siclog "Steps 21&22" siclog@

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, -, -, v_NR_SCGConfig, -, -, omit);

// Test body now finished

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

f_EUTRA_TestBody_Set(false);

f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1);

f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED);

// If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell!

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble"));

}

After change:

function f_TC_7_1_1_3_8_EUTRA_Common() runs on EUTRA_5GS_PTC {

// UE power headroom reporting / SCell activation / DL pathloss change reporting

var template (omit) octetstring v_NR_SCGConfig := omit;

f_EUTRA38_Init_CA(EN_DC, c1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

//Preamble to enter UE into RRC Connected

f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_Only, RRC_IDLE);//@sic R5s211385 sic@

f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TESTMode_OFF, -, -);//@sic R5s211385 sic@

f_EUTRA_TestBody_Set(true);

// Send trigger to NR to test on SCG bearer

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// is this to be done in the preamble and test body is empty ?

//@siclog "Steps 1&2" siclog@

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, -, -, v_NR_SCGConfig, -, -, omit);

//@siclog "Steps 4-5" siclog@

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, -, -, v_NR_SCGConfig, -, -, omit);

//@siclog "Steps 21&22" siclog@

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, -, -, v_NR_SCGConfig, -, -, omit);

// Test body now finished

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

f_EUTRA_TestBody_Set(false);

//f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1);  //### Remove the function to open UE test loop mode

f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED);

// If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell!

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble"));

}