3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220669

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2529 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In step 27 the 38.523-1 dictates: “The SS allocates an UL Grant with with size 384 bits corresponding to HARQ process X, with NDI not toggled compared to step 19 “ But TTCN currently doesn’t signal that NDI is not toggled so SS will toggle by default. - The MAC PDU in step 6 is required to use the same HARQ id as in step 1. For that reason this information needs to be signalled to SS in the DRB_COMMON_REQ the same way as it is done in step 1. It may not be sufficient to generate just a specificTransportBlock configuration with given HarqProcessConfig, because the SS interprets this as an explicit configuration for this given HARQid. And since the HARQ allocation depends on SS implementation the SS may not assign the very same HARQ to the step 6 transmission. However the proposed change ensures that the desired HARQ with specific TransportBlock configuration shall be used for the data in step 6. - To ensure that test procedure at step 25 is meet, additional restriction list of UL HARQ process id shall be provided to the SS to prevent that SS is not using UL HARQ process id from step 17 in step 23. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding “ToggleNDI=false” when configuring UL grant in step 27 - Added HARQ process information to the DRB_COMMON_REQ in step 6 - Configured UL HARQ process id list of specif HARQ process to be used in step 23 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test purpose may not be fulfilled as intended in steps 6 and 27. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_9_1_Step24to29 |
| --- | --- |
| Reason for change | In step 27 the 38.523-1 dictates: “The SS allocates an UL Grant with with size 384 bits corresponding to HARQ process X, with NDI not toggled compared to step 19 “ But TTCN currently doesn’t signal that NDI is not toggled so SS will toggle by default. |
| Summary of change | Adding “ToggleNDI=false” when configuring UL grant in step 27. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted in principle |

Before Change

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList);//@sic R5s200855 sic@ //@siclog "Step 24A" siclog@ //The SS ignores scheduling requests and does not allocate any uplink grant. //@sic R5s201457 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 25" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 17. //PDCP SDU = 37B + 3B PDCP Header + 3B RLC Header + 2B MAC Header = 45B = 360 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s201457 sic@ v_EncodedRlcPdu1:= f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(3,crs_NR_PDCP_SDU_37B))); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(1))); //@siclog "Step 26 " siclog@ //Indication of Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); //@siclog "Step 27" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits @sic R5s201457 sic@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1)); // @siclog "Step 28" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now); //Default Grant configuration. }

After Change

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList);//@sic R5s200855 sic@ //@siclog "Step 24A" siclog@ //The SS ignores scheduling requests and does not allocate any uplink grant. //@sic R5s201457 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 25" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 17. //PDCP SDU = 37B + 3B PDCP Header + 3B RLC Header + 2B MAC Header = 45B = 360 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s201457 sic@ v_EncodedRlcPdu1:= f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(3,crs_NR_PDCP_SDU_37B))); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(1))); //@siclog "Step 26 " siclog@ //Indication of Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); //@siclog "Step 27" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, cs_NR_FreqDomainSchedulExplicitUL_Def, -, {cs_NR_TransportBlockSingleTransmission(1, 0, false)},cs_HarqProcessConfig(1)); f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OneTime); // @siclog "Step 28" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now); //Default Grant configuration. }

MCC160 Implementation

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ .. //@siclog "Step 27" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits @sic R5s201457 sic@ f_NR_OneULGrantTransmission_NDI(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1),-,false); // @siclog "Step 28" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ … }

MCC160 Additional change – New function

/* * @desc To configure on UL grant * @param p_NR_CellId * @param p_TimingInfo (default value: cs_TimingInfo_Now) * @param p_ResourceAllocation (default value: cs_NR_ResourceAllocationDef) * @param p_NR_HarqProcessConfig (default value: omit) * @param p_PUCCH_Synch (default value: cs_NR_UplinkTimeAlignment_Start) * @param p_ToggleNDI (default value: true) */ function f_NR_OneULGrantTransmission_NDI(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ResourceAllocation_Type p_ResourceAllocation := cs_NR_ResourceAllocationDef, template (omit) NR_HarqProcessConfig_Type p_NR_HarqProcessConfig := omit, template (omit) NR_UplinkTimeAlignment_Synch_Type p_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Start, boolean p_ToggleNDI:= true ) runs on NR_BASE_PTC { var template (value) NR_TransportBlockRetransmissionList_Type v_TransportBlockRetransmissionList := { cs_NR_TransportBlockSingleTransmission(p_ResourceAllocation.Imcs, 0, p_ToggleNDI) }; var template (value) NR_DciUlInfo_Type v_DciUlInfo := cds_NR_DciUlInfo_ModifyGrant(cs_NR_FreqDomainSchedulExplicit(-, p_ResourceAllocation.Lrbs), -, p_ResourceAllocation.Imcs, p_NR_HarqProcessConfig, -); v_DciUlInfo.ResoureAssignment.TransportBlockScheduling := v_TransportBlockRetransmissionList; f_NR_ULGrantConfiguration_Common(p_NR_CellId, p_TimingInfo, v_DciUlInfo, p_PUCCH_Synch, cs_UL_GrantConfig_OneTime); }

## Change 2

| Function name | f_TC_7_1_1_9_1_Step5to9A |
| --- | --- |
| Reason for change | The MAC PDU in step 6 is required to use the same HARQ id as in step 1. For that reason this information needs to be signalled to SS in the DRB_COMMON_REQ the same way as it is done in step 1. It may not be sufficient to generate just a specificTransportBlock configuration with given HarqProcessConfig, because the SS interprets this as an explicit configuration for this given HARQid. And since the HARQ allocation depends on SS implementation the SS may not assign the very same HARQ to the step 6 transmission. However the proposed change ensures that the desired HARQ with specific TransportBlock configuration shall be used for the data in step 6. |
| Summary of change | Added HARQ process information to the DRB_COMMON_REQ in step 6 |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Rejected. The HARD process is already provided when configuring the DCI As agreed in sidebar meeting of RAN5-94e. |

Before Change

function f_TC_7_1_1_9_1_Step5to9A(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ // @siclog "Step 5" sigloc@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Step 5A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(5));//@sic R5s211710 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // 100 ms later v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)})); //@sic R5s200855 R5s211710 sic@ // @siclog "Step 7 " siclog@ //Check: Does the UE transmit a Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7: UE sent Scheduling Request"); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 8" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, cs_HarqProcessConfig(5));//@sic R5s200855 sic@ //@siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})); deactivate(v_IgnoreNRSR); //@sic R5-204418 sic@ //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-213132 sic@ }

After Change

function f_TC_7_1_1_9_1_Step5to9A(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ // @siclog "Step 5" sigloc@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Step 5A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(5));//@sic R5s211710 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // 100 ms later v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(5))); //@sic R5s200855 R5s211710 sic@ // @siclog "Step 7 " siclog@ //Check: Does the UE transmit a Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7: UE sent Scheduling Request"); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 8" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, cs_HarqProcessConfig(5));//@sic R5s200855 sic@ //@siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})); deactivate(v_IgnoreNRSR); //@sic R5-204418 sic@ //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-213132 sic@ }

## Change 3 – MCC 160 Additional Change

| Function name | fl_TC_7_1_1_9_1_NR_TestBody |
| --- | --- |
| Reason for change | To ensure that test procedure step at 25 is meet, additional restriction list of UL HARQ process id, shall be provided to the SS to prevent that SS is not using UL HARQ process id from step 17 in step 23. |
| Summary of change | Configured UL HARQ process id list of specif HARQ process to be used in step 23. |
| TTCN module | MAC_NR5GC |

Before Change

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { … //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit),v_SS_DRB_Config,true, v_CellGroupConfig,v_NR_HarqProcessConfig_UL); //@sic R5-204418 R5s210138 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList); //Reconfiguring MAC to normal mode //@sic R5s220581 sic@ v_DRB_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_DRB_ConfigList);

After Change

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5S210138 sic@ var boolean v_IsMacDataReceived := false; var boolean v_RrcReconfigurationCompleteReceived := false; var template (value) NR_RadioBearerList_Type v_DRB_ConfigList; var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; timer t_Watchdog := 1.0; ... //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669 sic@ f_NR_ULGrantConfiguration_Common(p_NR_DRB_Id, cs_TimingInfo_Now, v_DciUlInfo, -, -); f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit),v_SS_DRB_Config,true, v_CellGroupConfig,v_NR_HarqProcessConfig_UL); //@sic R5-204418 R5s210138 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList, v_NR_HarqProcessConfig_UL); //Reconfiguring MAC to normal mode //@sic R5s220581 sic@ v_DRB_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_DRB_ConfigList);

## Change 4 – MCC 160 Additional Change

| Function name | f_TC_7_1_1_9_1_Step24to29 |
| --- | --- |
| Reason for change | Same as 2.3 |
| Summary of change | Configured the specific UL HARQ List after step 27. |
| TTCN module | MAC_TC_Common_NR |

Before Change

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList);//@sic R5s200855 sic@ //@siclog "Step 24A" siclog@ //The SS ignores scheduling requests and does not allocate any uplink grant. //@sic R5s201457 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 25" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 17. //PDCP SDU = 37B + 3B PDCP Header + 3B RLC Header + 2B MAC Header = 45B = 360 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s201457 sic@ f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(1)); //@sic R5s211710 sic@ v_EncodedRlcPdu1:= f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(1,crs_NR_PDCP_SDU_37B))); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)})); //@sic R5s211710 sic@ //@siclog "Step 26 " siclog@ //Indication of Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); //@siclog "Step 27" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits @sic R5s201457 sic@ f_NR_OneULGrantTransmission_NDI(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1),-, false);// @sic R5s201457 R5s220669 sic@ // @siclog "Step 28" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now); //Default Grant configuration. }

After Change

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, template (omit) NR_HarqProcessConfig_Type p_NR_HarqProcessConfig_UL:= omit) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ .. //@siclog "Step 27" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits @sic R5s201457 sic@ f_NR_OneULGrantTransmission_NDI(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1),-, false);// @sic R5s201457 R5s220669 sic@ // @siclog "Step 28" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, -,-,-,-,-,p_NR_HarqProcessConfig_UL); //Default Grant configuration. }