3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220679

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0831 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_WLAN_IPCAN_Init |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test, MTSI_WLAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When pixit px_WLAN_BeaconRSSI <93, which default value in pixit is 78, it will cause TCI error indicated attenuation value out of constraint. Because Attenuation value = MaxReferenceBeaconRSSI - px_WLAN_BeaconRSSI, current definition of tsc_MaxReferenceBeaconRSSI = 220 is too high and attenuation value range in (0…127). Hence, need to reset MaxReferenceBeaconRSSI for a cell of IMS over WLAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reset MaxReferenceBeaconRSSI to 127 in WLAN SimulatedAP parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC will have TCI exception error when px_WLAN_BeaconRSSI <93. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.8.1 and other G.x.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | G.8.1 and other G.x.x |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

## f_WLAN_IPCAN_Init

| Function name | f_WLAN_IPCAN_Init |
| --- | --- |
| Reason for change | When pixit px_WLAN_BeaconRSSI <93, it will cause TCI error indicated attenuation value out of constraint. Because attenuation value = MaxReferenceBeaconRSSI - px_WLAN_BeaconRSSI, current definition of tsc_MaxReferenceBeaconRSSI = 220 is too high and attenuation value range in (0…127). |
| Summary of change | Reset MaxReferenceBeaconRSSI to 127 in WLAN SimulatedAP parameter. |
| TTCN module | Common_IMS\PART3_IPCAN\IPCAN_WLAN.ttcn |
| MCC160 Comment | Accepted in principle. The value of tsc_MaxReferenceBeaconRSSI is instead set to 140, WLAN Beacon RSSI is defined in IEEE 802.11-2012, Part 11 table 6.7 as integer -100..40 but defined in 36.331/25.331 as integer(0..255) so actual beacon RSSI = value -100, and values 141..255 are not used or spares |

Before Change:

function f_WLAN_IPCAN_Init(IMS_IPCAN_CO_ORD_PORT p_PortIMS, WLAN_CellId_Type p_CellId, PDN_Index_List_Type p_PDN_Id_List:= {ePDG_IMS1}) runs on WLAN_PTC { var boolean v_IMSoWLAN := true; f_WLAN_Init(WLAN_Only); f_WLAN_CellConfig_Def(p_CellId, p_PDN_Id_List,-, v_IMSoWLAN); f_IMS_IPCAN_SendCoOrdMsg(p_PortIMS, cms_IPCAN_IMS_Response(IPCAN_INIT)); }

After Change:

function f_WLAN_IPCAN_Init(IMS_IPCAN_CO_ORD_PORT p_PortIMS, WLAN_CellId_Type p_CellId, PDN_Index_List_Type p_PDN_Id_List:= {ePDG_IMS1}) runs on WLAN_PTC { var boolean v_IMSoWLAN := true; var template (value) WLAN_APList_Type v_WLAN_APs; v_WLAN_APs := {cs_WLAN_SimulatedAP(p_CellId, tsc_WLAN_SSID_AP1, 127)}; f_WLAN_Init(WLAN_Only, v_WLAN_APs); f_WLAN_CellConfig_Def(p_CellId, p_PDN_Id_List,-, v_IMSoWLAN); f_IMS_IPCAN_SendCoOrdMsg(p_PortIMS, cms_IPCAN_IMS_Response(IPCAN_INIT)); }