3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220680

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2536 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5G NAS type definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 24.501 9.11.3.38, the field standradized access category in Operator-defined access category definition may or may not be included, depending on the value of Presence of standardized access category (PSAC) field. This means the octet a* in figure 9.11.3.38.2 can be omitted. 8; 7; 6; 5; 4; 3; 2; 1. Length of operator-defined access category definition contents; octet 4. Precedence value; octet 5. PSAC; 0 Spare; 0 Spare; Operator-defined access category number; octet 6. Length of criteria; octet 7. Criteria; octet 8 octet a-1. 0 Spare; 0 Spare; 0 Spare; Standardized access category; octet a*. Figure 9.11.3.38.2: Operator-defined access category definition |
| Summary of change: |  | Make the fields standardAccessCat and spare3 optional, as the final octet can be omitted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Type AccessCatDefinition |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | Type AccessCatDefinition |
| --- | --- |
| Reason for change | According to 24.501 9.11.3.38, the field standradized access category in Operator-defined access category definition may or may not be included, depending on the value of Presence of standardized access category (PSAC) field. This means the octet a* in figure 9.11.3.38.2 can be omitted. 8; 7; 6; 5; 4; 3; 2; 1. Length of operator-defined access category definition contents; octet 4. Precedence value; octet 5. PSAC; 0 Spare; 0 Spare; Operator-defined access category number; octet 6. Length of criteria; octet 7. Criteria; octet 8 octet a-1. 0 Spare; 0 Spare; 0 Spare; Standardized access category; octet a*. Figure 9.11.3.38.2: Operator-defined access category definition |
| Summary of change | Make the fields standardAccessCat and spare3 optional, as the final octet can be omitted. |
| TTCN module | NG_NAS_TypeDefs.ttcn |
| MCC160 Comment | Accepted |

Before Change:

type record AccessCatDefinition { Type4Length_Type iel, O1_Type precendence, // @sic R5w200211 sic@ B1_Type psac, B2_Type spare2, B5_Type catNum, Type4Length_Type criteriaLength, octetstring criteria, B3_Type spare3, B5_Type standardAccessCat };

After Change:

type record AccessCatDefinition { Type4Length_Type iel, O1_Type precendence, // @sic R5w200211 sic@ B1_Type psac, B2_Type spare2, B5_Type catNum, Type4Length_Type criteriaLength, octetstring criteria, B3_Type spare3 optional, B5_Type standardAccessCat optional };