3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220694

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2545 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC Rel-16 test case 8.1.4.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 14-16, after T304 has expired, the power level of cell4 is atill at -91 and is changed to -113 on the expiry of the 2s timer in TTCN. During this time the UE may try to re-access cell 4 in order to send RRCReestablishmentRequest with cause “handover failure”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SS is configured with to ignore RACH attempts on NR Cell 4 as well as cell 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

2.1 Change 1

| Function name | fl_TC_8_1_4_4_3_TestBody () |
| --- | --- |
| Reason for change | At step 14-16, after T304 has expired, the power level of cell4 is atill at -91 and is changed to -113 on the expiry of the 2s timer in TTCN. During this time the UE may try to re-access cell 4 in order to send RRCReestablishmentRequest with cause “handover failure”. |
| Summary of change | SS is configured with to ignore RACH attempts on NR Cell 4 as well as cell 1. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_8_1_4_4_3_TestBody() runs on NR5GC_PTC { ………… //@siclog "Step 12" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 4. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell4, cr_38508_RRCReconfigurationComplete)); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@ //@siclog "Step 13" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ //EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304. //The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 1. //The SS does not respond. f_Delay(2.0); // make sure T304 expires // @sic R5s220375 sic@ //@siclog "Step 16" siclog@ //SS adjusts the cell-specific reference signal level according to row "T4". f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 17" siclog@ //Wait 5s to ensure that T304 and T311 expire. (Note 1: The wait time is selected to cover T304 (1000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE.) f_Delay(5.0); //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T5". f_NR_SetCellPowerList(v_CellPowerList_AtT5); //@siclog "Step 19" siclog@ //Check: Does the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 2. f_NR5GC_MobilityRegistration(nr_Cell12, -,-,-,-,-); // @sic R5s220375 sic@ }// End of fl_TC_8_1_4_4_3_TestBody

After Change

function fl_TC_8_1_4_4_3_TestBody() runs on NR5GC_PTC { …………….. //@siclog "Step 12" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 4. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell4, cr_38508_RRCReconfigurationComplete)); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@ f_NR_SS_RachProcedureConfig(nr_Cell4, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 13" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ //EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304. //The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 1. //The SS does not respond. f_Delay(2.0); // make sure T304 expires // @sic R5s220375 sic@ //@siclog "Step 16" siclog@ //SS adjusts the cell-specific reference signal level according to row "T4". f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 17" siclog@ //Wait 5s to ensure that T304 and T311 expire. (Note 1: The wait time is selected to cover T304 (1000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE.) f_Delay(5.0); //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T5". f_NR_SetCellPowerList(v_CellPowerList_AtT5); //@siclog "Step 19" siclog@ //Check: Does the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 2. f_NR5GC_MobilityRegistration(nr_Cell12, -,-,-,-,-); // @sic R5s220375 sic@ }// End of fl_TC_8_1_4_4_3_TestBody