3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220697

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2548 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test cases 8.1.4.1.5 and 8.1.4.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 4, and strep 10 of 8.1.4.1.5, the UE may send the Reestablishment Req before the expiry of T304 timer as soon as the Cell 2 is turned off at T2 time instance. This could fail the test case. The reestablishment req sent by the UE at this step should be because of T304 timer expiry, and not because of radio link failure on cell 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The power level change for Cell 2 at T2 is done after 800ms of sending the Reconfiguration, to ensure that the T304 is started and expired at the UE side, before it sends the Reestablishsment Req |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.5.NR5GC, 8.1.4.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_1_5_TestBody |
| --- | --- |
| Reason for change | At steps 4 and 10, the UE may send the Reestablishment Req before the expiry of T304 timer as soon as the Cell 2 is turned off at T2 time instance. This could fail the test case. The reestablishment req sent by the UE at these steps should be because of T304 timer expiry, and not because of radio link failure on cell 2. |
| Summary of change | The power level change for Cell 2 at T2 is done after 800ms of sending the Reconfiguration, to ensure that the T304 is started and expired at the UE side, before it sends the Reestablishsment Req |
| TTCN module | RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | Postponed conditional to prose CR agreement |

Before Change

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //@sic R5s220176 sic@ //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 4-6B" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, "Step 4"); //@siclog "Step 7" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 8" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. //@sic R5s220201 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //@sic R5s210162 sic@ //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.5.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 9" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@sic R5s210162 R5-211332 sic@ //@siclog "Step 10-12B" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336, R5s210162 R5-211332 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, "Step 12"); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message on NR Cell 1. //@sic R5s210162 R5-211332 sic@ f_NR_RRCRelease(nr_Cell1); } // End of fl_TC_8_1_4_1_5_TestBody

After Change

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var SubFrameTiming_Type v_TimingInfoDL_1, v_TimingInfoDL_2; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); v_TimingInfoDL_1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfoDL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL_1, 800); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoDL_1), v_RRCReconfigurationHO)); //@sic R5s220176 sic@ //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2, cs_TimingInfo_NR(v_TimingInfoDL_2),nr_Cell1); //@siclog "Steps 4-6B" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, "Step 4"); //@siclog "Step 7" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); v_TimingInfoDL_1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfoDL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL_1, 800); //@siclog "Step 8" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. //@sic R5s220201 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoDL_1), v_RRCReconfigurationHO)); //@sic R5s210162 sic@ //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.5.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 9" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2, cs_TimingInfo_NR(v_TimingInfoDL_2),nr_Cell1); //@siclog "Step 10-12B" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? //@sic R5-206369, R5-206336, R5s210162 R5-211332 sic@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, "Step 12"); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message on NR Cell 1. //@sic R5s210162 R5-211332 sic@ f_NR_RRCRelease(nr_Cell1); } // End of fl_TC_8_1_4_1_5_TestBody

## Change 2

| Function name | fl_TC_8_1_4_1_6_TestBody |
| --- | --- |
| Reason for change | At steps 4, the UE may send the Reestablishment Req before the expiry of T304 timer as soon as the Cell 2 is turned off at T2 time instance. This could fail the test case. The reestablishment req sent by the UE at this step should be because of T304 timer expiry, and not because of radio link failure on cell 2. |
| Summary of change | The power level change for Cell 2 at T2 is done after 800ms of sending the Reconfiguration, to ensure that the T304 is started and expired at the UE side, before it sends the Reestablishsment Req |
| TTCN module | RRC_IntraNR_Handover_NR5GC |
| MCC160 Comment | Postponed conditional to prose CR agreement |

Before Change

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC

{

var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1);

var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1);

var ShortMAC_I v_ShortMAC_I;

var template (value) DL_DCCH_Message v_RRCReconfigurationHO;

var NextHopChainingCount v_NextHopChainingCount;

var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19.

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //As per Table 8.1.4.1.6.3.2-1

var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1);     //@sic R5s201221 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

var NG_NAS_MSG_Indication_Type v_NasInd;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

timer t_T304Min;

// Reconfigure nr_Cell2 to no respond to any RA Preamble

// Table 8.1.4.1.6.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond.

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse);

//@siclog "Step 1" siclog@

// The SS changes the power level setting according to the row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

//The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2.

v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO));

t_T304Min.start(v_T304Min);

//EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.6.3.2-4 should take place.

//Nothing to do nr_Cell2 already configured to not respond to any RA Preamble

//@siclog "Step 3" siclog@

//The SS changes the power level setting according to the row "T2".

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell1);

//UE re-establish SRB1 - UE releases SRB2 and DRB

//In SS: re-establish SRB1 and SRB2 - Release DRBs

//In SS: It helps with clearing the RB buffer handling reconfiguration retx

//@sic R5s201221  R5s210197 sic@

f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true);

f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true);

…

…

…

}

After Change

…

import from Common4G5G_Timing all;

…

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC

{

var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1);

var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1);

var ShortMAC_I v_ShortMAC_I;

var template (value) DL_DCCH_Message v_RRCReconfigurationHO;

var NextHopChainingCount v_NextHopChainingCount;

var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19.

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //As per Table 8.1.4.1.6.3.2-1

var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1);     //@sic R5s201221 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

var NG_NAS_MSG_Indication_Type v_NasInd;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

timer t_T304Min;

var SubFrameTiming_Type v_TimingInfoDL_1, v_TimingInfoDL_2;

// Reconfigure nr_Cell2 to no respond to any RA Preamble

// Table 8.1.4.1.6.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond.

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse);

//@siclog "Step 1" siclog@

// The SS changes the power level setting according to the row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

//The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2.

v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2);

v_TimingInfoDL_1 := f_NR_GetNextSendOccasion_DL(nr_Cell1);

v_TimingInfoDL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL_1, 800);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoDL_1), v_RRCReconfigurationHO));

t_T304Min.start(v_T304Min);

//EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.6.3.2-4 should take place.

//Nothing to do nr_Cell2 already configured to not respond to any RA Preamble

//@siclog "Step 3" siclog@

//The SS changes the power level setting according to the row "T2".

f_NR_SetCellPowerList(v_CellPowerList_AtT2, cs_TimingInfo_NR(v_TimingInfoDL_2),nr_Cell1);

v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell1);

//UE re-establish SRB1 - UE releases SRB2 and DRB

//In SS: re-establish SRB1 and SRB2 - Release DRBs

//In SS: It helps with clearing the RB buffer handling reconfiguration retx

//@sic R5s201221  R5s210197 sic@

f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true);

f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true);

…

…

…

}