3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220766

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0836 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to A.2.3 implementation check involving ReInvite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The R5-223453 specifies the Record-Route in 183SessionProgress for different scenarios, namely how it differs in case of re-INVITE voice call tests. However current implementation of this CR in D22wk23 TTCN is such that it checks whether dialog has already been established which is not sufficient to fully detect “reINVITE” scenarios and so it sometimes affect others as well. There may be number of solutions, we propose following one: To add new global variable that will signal in each specific test (and step) that this is going to be “reINVITE” scenario so the change in Record-Route shall be applied, otherwise it shall not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New global variable is added - All tests with re-INVITE are now setting the new global variable to true in the test body from the specific step - Current implementation of R5-223453 is modified so that it now checks the new global variable. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of R5-223453 affects many tests without the “reINVITE” scenarios and so the conformant UE fail them if run on D22wk23 TTCN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IMS voice call test scenarios |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

2.4	Change 4	7

2.5	Change 5	8

2.6	Change 6	10

2.7	Change 7	12

2.8	Change 8	13

2.9	Change 9	15

2.10	Change 10	17

2.11	Change 11	19

2.12	Change 12	22

2.13	Change 13	23

2.14	Change 14	24

2.15	Change 15	26

2.16	Change 16	28

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Type name | IMS_DialogInfo_Type |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | New global Boolean variable within IMS_DialogInfo_Type added |
| TTCN module | IMS_Component.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

type record IMS_DialogInfo_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS) @sic R5s140092 change 1.5: RemoteURI, RemoteCseq, LocalURI, LocalCseq stored in this structure sic@ @sic R5-174544: SessionId removed sic@ */ boolean MobileOriginated optional, /* needed e.g. to get the right route-set for re-INVITE */ charstring RemoteTag optional, /* acc. to RFC 3261 when creating a dialog in first request there is a From tag but no To tag; the To tag is chosen by the other end and sent in a response; in all further messages within the dialog the same From and To tags shall be used (=> especially in request messages (e.g. BYE) it may be necessary to use this stored value whereas in responses the values can be used as sent in the request); NOTE: in general outside a dialog (e.g. registration) requests get a new From tag and the corresponding reponses contain a new To tag */ SipUrl RemoteURI optional, /* URI of the remote side in a dialog created by INVITE @sic R5s160451: SipUrl instead of charstring sic@ */ integer RemoteCseq optional, /* @sic R5s140235: changed to optional sic@ */ charstring LocalTag optional, SipUrl LocalURI optional, /* URI of the UE in a dialog created by INVITE @sic R5s160451: SipUrl instead of charstring sic@ */ integer LocalCseq, charstring CallId optional, SipUrl ContactUrl optional, /* Contact Url provided by the UE for this dialog (especially in case GRUU this is different to the (non-GRUU) URI used for registration); @sic R5s150908 sic@ */ charstring SdpSessionIdTX, /* @sic R5-150707 sic@ */ integer SdpSessionVersionTX, /* current SDP session version for the DL @sic R5-150707 sic@ */ SDP_Message SdpMessagePrevRX optional, /* previous SDP message sent by the UE @sic R5-150702 sic@ */ integer RemoteRSeq optional /* RemoteRSeq for ongoing transaction initiated by MT INVITE; shall be incremented for every reliable 1XX response (RFC 3262) @sic R5-181190 sic@ */ };

After Change

type record IMS_DialogInfo_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS) @sic R5s140092 change 1.5: RemoteURI, RemoteCseq, LocalURI, LocalCseq stored in this structure sic@ @sic R5-174544: SessionId removed sic@ */ boolean MobileOriginated optional, /* needed e.g. to get the right route-set for re-INVITE */ charstring RemoteTag optional, /* acc. to RFC 3261 when creating a dialog in first request there is a From tag but no To tag; the To tag is chosen by the other end and sent in a response; in all further messages within the dialog the same From and To tags shall be used (=> especially in request messages (e.g. BYE) it may be necessary to use this stored value whereas in responses the values can be used as sent in the request); NOTE: in general outside a dialog (e.g. registration) requests get a new From tag and the corresponding reponses contain a new To tag */ SipUrl RemoteURI optional, /* URI of the remote side in a dialog created by INVITE @sic R5s160451: SipUrl instead of charstring sic@ */ integer RemoteCseq optional, /* @sic R5s140235: changed to optional sic@ */ charstring LocalTag optional, SipUrl LocalURI optional, /* URI of the UE in a dialog created by INVITE @sic R5s160451: SipUrl instead of charstring sic@ */ integer LocalCseq, charstring CallId optional, SipUrl ContactUrl optional, /* Contact Url provided by the UE for this dialog (especially in case GRUU this is different to the (non-GRUU) URI used for registration); @sic R5s150908 sic@ */ charstring SdpSessionIdTX, /* @sic R5-150707 sic@ */ integer SdpSessionVersionTX, /* current SDP session version for the DL @sic R5-150707 sic@ */ SDP_Message SdpMessagePrevRX optional, /* previous SDP message sent by the UE @sic R5-150702 sic@ */ integer RemoteRSeq optional, /* RemoteRSeq for ongoing transaction initiated by MT INVITE; shall be incremented for every reliable 1XX response (RFC 3262) @sic R5-181190 sic@ */ boolean reINVITE optional };

## Change 2

| Template name | cs_IMS_DialogInfo_Init |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | New global variable is initialised to false value by default |
| TTCN module | IMS_Component.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

template (value) IMS_DialogInfo_Type cs_IMS_DialogInfo_Init := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS) */ /* @sic R5-174544: SessionId removed sic@ */ MobileOriginated := omit, RemoteTag := omit, RemoteURI := omit, RemoteCseq := omit, /* @sic R5s140235: changed to omit sic@ */ LocalTag := omit, LocalURI := omit, LocalCseq := 0, CallId := omit, ContactUrl := omit, /* @sic R5s150908 sic@ */ SdpSessionIdTX := "1111111111", /* @sic R5-150707: new variable replacing tsc_SDP_SessionId sic@ */ SdpSessionVersionTX := 1111111110, /* @sic R5-150702: new variable; intialised with 1111111110 as being incremented firstly (see f_IMS_PTC_ImsInfo_DialogIncrAndGetSessVersionTX) sic@ */ SdpMessagePrevRX := omit, RemoteRSeq := omit };

After Change

template (value) IMS_DialogInfo_Type cs_IMS_DialogInfo_Init := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS) */ /* @sic R5-174544: SessionId removed sic@ */ MobileOriginated := omit, RemoteTag := omit, RemoteURI := omit, RemoteCseq := omit, /* @sic R5s140235: changed to omit sic@ */ LocalTag := omit, LocalURI := omit, LocalCseq := 0, CallId := omit, ContactUrl := omit, /* @sic R5s150908 sic@ */ SdpSessionIdTX := "1111111111", /* @sic R5-150707: new variable replacing tsc_SDP_SessionId sic@ */ SdpSessionVersionTX := 1111111110, /* @sic R5-150702: new variable; intialised with 1111111110 as being incremented firstly (see f_IMS_PTC_ImsInfo_DialogIncrAndGetSessVersionTX) sic@ */ SdpMessagePrevRX := omit, RemoteRSeq := omit, reINVITE := false };

## Change 3

| Function name | fl_IMS_PTC_ImsInfo_GetreINVITE fl_IMS_PTC_ImsInfo_SetreINVITE |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | New functions to get/set the new global variable. |
| TTCN module | IMS_Component.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

After Change

function fl_IMS_PTC_ImsInfo_GetreINVITE(template (omit) IMS_PTC_DialogIndex_Type p_DialogIndex := omit) runs on IMS_PTC return boolean { var integer i := fl_IMS_PTC_ImsInfo_GetDialogIndexValue(p_DialogIndex); return vc_IMS_Global.ImsInfo.Dialog.Info[i].reINVITE; } function fl_IMS_PTC_ImsInfo_SetreINVITE(template (omit) IMS_PTC_DialogIndex_Type p_DialogIndex := omit) runs on IMS_PTC { var integer i := fl_IMS_PTC_ImsInfo_GetDialogIndexValue(p_DialogIndex); vc_IMS_Global.ImsInfo.Dialog.Info[i].reINVITE := true; }

## Change 4

| Function name | f_IMS_InviteResponse_183_MessageHeaderTX |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Instead of “DialogsEstablished” it is now used “reINVITE” global information to check the condition for applying R5-223453 |
| TTCN module | IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | Accepted in principle using f_IMS_SipRequest_MessageHeaderIsReINVITE instead of fl_IMS_PTC_ImsInfo_GetreINVITE (see MCC160 Comments for R5s220772) |

Before Change

function f_IMS_InviteResponse_183_MessageHeaderTX(INVITE_Request p_InviteRequest, charstring p_ContactUri := px_IMS_CalleeContactUri, template (omit) SemicolonParam_List p_AditionalContactParams := omit, OptionTag_List p_OptionTagsForRequire := tsc_OptionTagList_100rel_precondition, IMS_CallType_Type p_CallType := NORMAL_CALL, boolean p_A6 := pc_IMS_Video_FeatureTag, template (omit) ContentType p_ContentType := cs_ContentTypeSDP) runs on IMS_PTC return template (value) MessageHeader { … var boolean v_A15 := f_IMS_PTC_ImsInfo_DialogIsEstablished(); /* 183 sent for re-INVITE within an established dialog */

After Change

function f_IMS_InviteResponse_183_MessageHeaderTX(INVITE_Request p_InviteRequest, charstring p_ContactUri := px_IMS_CalleeContactUri, template (omit) SemicolonParam_List p_AditionalContactParams := omit, OptionTag_List p_OptionTagsForRequire := tsc_OptionTagList_100rel_precondition, IMS_CallType_Type p_CallType := NORMAL_CALL, boolean p_A6 := pc_IMS_Video_FeatureTag, template (omit) ContentType p_ContentType := cs_ContentTypeSDP) runs on IMS_PTC return template (value) MessageHeader { … var boolean v_A15 := fl_IMS_PTC_ImsInfo_GetreINVITE();

## Change 5

| Function name | f_IMS_OtherResponse_200_MessageHeaderTX |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Instead of “DialogsEstablished” it is now used “reINVITE” global information to check the condition for applying R5-223453 |
| TTCN module | IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | Accepted in principle using f_IMS_SipRequest_MessageHeaderIsReINVITE instead of fl_IMS_PTC_ImsInfo_GetreINVITE (see MCC160 Comments for R5s220772) |

Before Change

f_IMS_OtherResponse_200_MessageHeaderTX(MessageHeader p_MessageHeader_Request, template (omit) ContentType p_ContentType := omit, template (omit) charstring p_EmergencyTelURI := omit, charstring p_ContactUri := px_IMS_CalleeContactUri, boolean p_PreconditionsRequired := false, IMS_CallType_Type p_CallType := NORMAL_CALL) runs on IMS_PTC return template (value) MessageHeader { … select (v_Method) { case ("INVITE") { v_A1A3 := true; v_A19 := true; v_A23 := f_IMS_PTC_ImsInfo_DialogIsEstablished(); /* Response sent for re-INVITE within an established dialog */ } case ("UPDATE") { v_A1A3 := true; } }

After Change

f_IMS_OtherResponse_200_MessageHeaderTX(MessageHeader p_MessageHeader_Request, template (omit) ContentType p_ContentType := omit, template (omit) charstring p_EmergencyTelURI := omit, charstring p_ContactUri := px_IMS_CalleeContactUri, boolean p_PreconditionsRequired := false, IMS_CallType_Type p_CallType := NORMAL_CALL) runs on IMS_PTC return template (value) MessageHeader { … select (v_Method) { case ("INVITE") { v_A1A3 := true; v_A19 := true; v_A23 := fl_IMS_PTC_ImsInfo_GetreINVITE(); /* Response sent for re-INVITE within an established dialog */ } case ("UPDATE") { v_A1A3 := true; } }

## Change 6

| Function name | f_IMS_InviteResponse_183_MessageHeaderRX |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Instead of “DialogsEstablished” it is now used “reINVITE” global information to check the condition for applying R5-223453 |
| TTCN module | IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | Accepted in principle using f_IMS_SipRequest_MessageHeaderIsReINVITE instead of fl_IMS_PTC_ImsInfo_GetreINVITE (see MCC160 Comments for R5s220772) |

Before Change

function f_IMS_InviteResponse_183_MessageHeaderRX(template (value) INVITE_Request p_InviteRequest, template ContentType p_ContentType := cr_ContentTypeSDP, OptionTag_List p_MandatoryOptionTagsForRequire := tsc_OptionTagList_100rel_precondition, boolean p_A6 := pc_IMS_Video_FeatureTag) runs on IMS_PTC return template (present) MessageHeader { /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5-176371: p_A6 renamed to p_A6 sic@ */ /* @sic R5s190882: "p_AdditionalMandatoryOptionTagsForRequire := tsc_OptionTagList_Empty" replaced by "p_MandatoryOptionTagsForRequire := tsc_OptionTagList_100rel_precondition" sic@ */ /* @sic R5s190882: p_MandatoryContactParams added sic@ */ /* @sic R5s200000: p_MandatoryContactParams removed sic@ */ var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (present) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderRX(v_MessageHeader_Invite); /* @sic R5s200000: f_IMS_BaseResponse_MessageHeaderRX sic@ */ var integer v_Port_us := f_IMS_PTC_GetPort_us(); var template (present) SemicolonParam_List v_MandatoryContactParams := {}; var template (present) SipUrl v_ContactURI; var template RecordRoute v_RecordRoute; var boolean v_A15 := f_IMS_PTC_ImsInfo_DialogIsEstablished(); /* 183 sent for re-INVITE within an established dialog */ …

After Change

function f_IMS_InviteResponse_183_MessageHeaderRX(template (value) INVITE_Request p_InviteRequest, template ContentType p_ContentType := cr_ContentTypeSDP, OptionTag_List p_MandatoryOptionTagsForRequire := tsc_OptionTagList_100rel_precondition, boolean p_A6 := pc_IMS_Video_FeatureTag) runs on IMS_PTC return template (present) MessageHeader { /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5-176371: p_A6 renamed to p_A6 sic@ */ /* @sic R5s190882: "p_AdditionalMandatoryOptionTagsForRequire := tsc_OptionTagList_Empty" replaced by "p_MandatoryOptionTagsForRequire := tsc_OptionTagList_100rel_precondition" sic@ */ /* @sic R5s190882: p_MandatoryContactParams added sic@ */ /* @sic R5s200000: p_MandatoryContactParams removed sic@ */ var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (present) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderRX(v_MessageHeader_Invite); /* @sic R5s200000: f_IMS_BaseResponse_MessageHeaderRX sic@ */ var integer v_Port_us := f_IMS_PTC_GetPort_us(); var template (present) SemicolonParam_List v_MandatoryContactParams := {}; var template (present) SipUrl v_ContactURI; var template RecordRoute v_RecordRoute; var boolean v_A15 := fl_IMS_PTC_ImsInfo_GetreINVITE(); /* 183 sent for re-INVITE within an established dialog */ …

## Change 7

| Function name | f_TC_7_20_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | MS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_7_20_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS */ var IMS_CC_MO_CallInfo_Type v_CallInfo; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var charstring v_CallIdentification; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, -, normalCall_ReturnId); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_CallIdentification := f_UT_ReceivedIMSCallId(MMI); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); …

After Change

function f_TC_7_20_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS */ var IMS_CC_MO_CallInfo_Type v_CallInfo; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var charstring v_CallIdentification; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, -, normalCall_ReturnId); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_CallIdentification := f_UT_ReceivedIMSCallId(MMI); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 8

| Function name | f_TC_7_21_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | MS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_7_21_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS var template (omit) IMS_DATA_RSP v_IMS_DATA_RSP; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdpAudio; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdpVideo; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (value) SDP_Message v_SdpOffer; var template (present) SDP_Message v_ExpectedSdpAnswer; var template (present) IMS_Response_Type v_Response183; var IMS_Response_Type v_Response; var SDP_Message v_SdpAnswer; var charstring v_CallIdentification; timer t_Timer := 2.0; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, -, normalCall_ReturnId); v_InviteRequestWithSdpAudio := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(); f_IMS_MOCallSetup_NR5GC_A_4_2_Step2_8(v_InviteRequestWithSdpAudio); v_CallIdentification := f_UT_ReceivedIMSCallId(MMI); f_IMS_TestBody_Set(true); v_SdpOffer := f_IMS_BuildSDP_InitialOfferAddVideo_TX(EVS_16000, tsc_IMS_SDP_VideoCodecList_H265_only, v_InviteRequestWithSdpAudio.SdpOffer, tsc_SDP_WithoutPreconditions); v_ExpectedSdpAnswer := f_IMS_BuildSDP_MTCallAudioVideo_RX(AddVideo, InitialAnswer, EVS_16000, H265_90000, tsc_SDP_WithoutPreconditions); …

After Change

function f_TC_7_21_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS var template (omit) IMS_DATA_RSP v_IMS_DATA_RSP; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdpAudio; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdpVideo; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (value) SDP_Message v_SdpOffer; var template (present) SDP_Message v_ExpectedSdpAnswer; var template (present) IMS_Response_Type v_Response183; var IMS_Response_Type v_Response; var SDP_Message v_SdpAnswer; var charstring v_CallIdentification; timer t_Timer := 2.0; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, -, normalCall_ReturnId); v_InviteRequestWithSdpAudio := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(); f_IMS_MOCallSetup_NR5GC_A_4_2_Step2_8(v_InviteRequestWithSdpAudio); v_CallIdentification := f_UT_ReceivedIMSCallId(MMI); f_IMS_TestBody_Set(true); v_SdpOffer := f_IMS_BuildSDP_InitialOfferAddVideo_TX(EVS_16000, tsc_IMS_SDP_VideoCodecList_H265_only, v_InviteRequestWithSdpAudio.SdpOffer, tsc_SDP_WithoutPreconditions); v_ExpectedSdpAnswer := f_IMS_BuildSDP_MTCallAudioVideo_RX(AddVideo, InitialAnswer, EVS_16000, H265_90000, tsc_SDP_WithoutPreconditions); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 9

| Function name | f_TC_7_22_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | MS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; var charstring v_ContactUrlStrSS; var SipUrl v_ContactSipUrlSS; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1(); v_ContactSipUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); /* @sic R5s220500 change 1, R5-222412: get contact from INVITE creating the dialog sic@ */ v_ContactUrlStrSS := f_SIP_SipUrl2Str(v_ContactSipUrlSS); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); …

After Change

function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; var charstring v_ContactUrlStrSS; var SipUrl v_ContactSipUrlSS; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1(); v_ContactSipUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); /* @sic R5s220500 change 1, R5-222412: get contact from INVITE creating the dialog sic@ */ v_ContactUrlStrSS := f_SIP_SipUrl2Str(v_ContactSipUrlSS); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 10

| Function name | f_TC_7_23_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | MS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_7_23_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var charstring v_CallIdentification; var charstring v_ContactUrlStrSS; var SipUrl v_ContactSipUrlSS; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(tsc_SDP_WithoutPreconditions); v_ContactSipUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); v_ContactUrlStrSS := f_SIP_SipUrl2Str(v_ContactSipUrlSS); f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_10(v_InviteRequestWithSdp); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); …

After Change

function f_TC_7_23_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var charstring v_CallIdentification; var charstring v_ContactUrlStrSS; var SipUrl v_ContactSipUrlSS; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(tsc_SDP_WithoutPreconditions); v_ContactSipUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); v_ContactUrlStrSS := f_SIP_SipUrl2Str(v_ContactSipUrlSS); f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_10(v_InviteRequestWithSdp); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 11

| Function name | f_TC_7_33_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | MS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_7_33_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end chooses UE as refresher / 5GS var Method_List v_MethodList := {"INVITE", "ACK", "OPTIONS", "CANCEL", "BYE"}; var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var integer v_DeltaSec := 1800; var integer i; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9" siclog@ @sic R5-213545: A.5.2 instead of A.5.1 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithoutPreconditions); v_InviteRequestWithSdp.Invite.msgHeader.sessionExpires := cs_SessionExpires(v_DeltaSec, "uas"); v_InviteRequestWithSdp.Invite.msgHeader.allow := cs_Allow(v_MethodList); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); // @siclog "Step 10 - 16" IPCAN siclog@ f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_8(valueof(v_InviteRequestWithSdp)); // @sic R5-213545: A.5.2 instead of A.5.1 sic@ // @siclog "Steps 17" siclog@ f_UT_AnswerCall(MMI); // @siclog "Steps 18" Void siclog@ // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(valueof(v_InviteRequestWithSdp), -, -, v_DeltaSec, "uas"); /* @sic R5s210678 change 1: cr_Supported_timer instead of cr_Require_timer sic@ @sic R5-214450: cr_Supported_timer removed sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); …

After Change

function f_TC_7_33_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end chooses UE as refresher / 5GS var Method_List v_MethodList := {"INVITE", "ACK", "OPTIONS", "CANCEL", "BYE"}; var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var integer v_DeltaSec := 1800; var integer i; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9" siclog@ @sic R5-213545: A.5.2 instead of A.5.1 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithoutPreconditions); v_InviteRequestWithSdp.Invite.msgHeader.sessionExpires := cs_SessionExpires(v_DeltaSec, "uas"); v_InviteRequestWithSdp.Invite.msgHeader.allow := cs_Allow(v_MethodList); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); // @siclog "Step 10 - 16" IPCAN siclog@ f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_8(valueof(v_InviteRequestWithSdp)); // @sic R5-213545: A.5.2 instead of A.5.1 sic@ // @siclog "Steps 17" siclog@ f_UT_AnswerCall(MMI); // @siclog "Steps 18" Void siclog@ // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(valueof(v_InviteRequestWithSdp), -, -, v_DeltaSec, "uas"); /* @sic R5s210678 change 1: cr_Supported_timer instead of cr_Require_timer sic@ @sic R5-214450: cr_Supported_timer removed sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 12

| Function name | f_TC_8_26_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_8_26_NR5GC_IMS1() runs on IMS_PTC { // MO Voice Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var INVITE_Request v_InviteRequest; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_CallHold(MMI); …

After Change

function f_TC_8_26_NR5GC_IMS1() runs on IMS_PTC { // MO Voice Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var INVITE_Request v_InviteRequest; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_CallHold(MMI); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 13

| Function name | f_TC_8_27_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_8_27_NR5GC_IMS1() runs on IMS_PTC { // MO Video Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var INVITE_Request v_InviteRequest; f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_VideoCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_15_1(); v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_CallHold(MMI); …

After Change

function f_TC_8_27_NR5GC_IMS1() runs on IMS_PTC { // MO Video Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var INVITE_Request v_InviteRequest; f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_VideoCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_15_1(); v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_CallHold(MMI); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 14

| Function name | f_TC_8_28_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_8_28_NR5GC_IMS1() runs on IMS_PTC { // MT Voice Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var template (value) SDP_Message v_SDP_LatestMessageBySS; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var boolean v_DontIncrementSessVers := true; // @sic R5s211703: session version shall not be incremented sic@ f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_CallInfo.MessageHeader183); v_SDP_LatestMessageBySS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_CallInfo.LatestSdpOffer, v_DontIncrementSessVers); f_IMS_TestBody_Set(true); …

After Change

function f_TC_8_28_NR5GC_IMS1() runs on IMS_PTC { // MT Voice Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var template (value) SDP_Message v_SDP_LatestMessageBySS; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var boolean v_DontIncrementSessVers := true; // @sic R5s211703: session version shall not be incremented sic@ f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_CallInfo.MessageHeader183); v_SDP_LatestMessageBySS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_CallInfo.LatestSdpOffer, v_DontIncrementSessVers); f_IMS_TestBody_Set(true); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 15

| Function name | f_TC_8_29_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_8_29_NR5GC_IMS1() runs on IMS_PTC { // MT Video Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var template (value) SDP_Message v_SDP_LatestMessageBySS; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var boolean v_DontIncrementSessVers := true; // @sic R5s211703: session version shall not be incremented sic@ f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_VideoCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_15_1(); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_CallInfo.MessageHeader183); v_SDP_LatestMessageBySS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_CallInfo.LatestSdpOffer, v_DontIncrementSessVers); f_IMS_TestBody_Set(true); …

After Change

function f_TC_8_29_NR5GC_IMS1() runs on IMS_PTC { // MT Video Call Hold without announcement / 5GS var IMS_CC_MO_CallInfo_Type v_CallInfo; var template (value) SDP_Message v_SDP_LatestMessageBySS; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var boolean v_DontIncrementSessVers := true; // @sic R5s211703: session version shall not be incremented sic@ f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MO_VideoCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_15_1(); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_CallInfo.MessageHeader183); v_SDP_LatestMessageBySS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_CallInfo.LatestSdpOffer, v_DontIncrementSessVers); f_IMS_TestBody_Set(true); fl_IMS_PTC_ImsInfo_SetreINVITE(); …

## Change 16

| Function name | f_TC_10_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Correction of R5-223453 implementation |
| Summary of change | Specific test in specific step enables the “reINVITE” global variable to indicate the scenario for application of R5-223453 |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Not needed (see MCC160 Comments for R5s220772) |

Before Change

function f_TC_10_3_NR5GC_IMS1() runs on IMS_PTC { // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS var INVITE_Request v_InviteRequest; var IMS_CC_MO_CallInfo_Type v_CallInfo; var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_SpeechAndEmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { // @sic R5s220215 add. change sic@ f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ // UE is made to hold the ongoing call f_UT_CallHold(MMI); // @siclog "Step 2-5" siclog@ // Ongoing call is put on hold by UE (Steps 1-4 of Annex A.17) f_IMS_MOCallHoldResume(HOLD, v_InviteRequest, v_CallInfo.MessageHeader183, v_CallInfo.LatestSdpOffer); f_IMS_IMS_SendCoOrdMsg(OtherPDN); …

After Change

function f_TC_10_3_NR5GC_IMS1() runs on IMS_PTC { // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS var INVITE_Request v_InviteRequest; var IMS_CC_MO_CallInfo_Type v_CallInfo; var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_SpeechAndEmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { // @sic R5s220215 add. change sic@ f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1(); v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ // UE is made to hold the ongoing call f_UT_CallHold(MMI); // @siclog "Step 2-5" siclog@ // Ongoing call is put on hold by UE (Steps 1-4 of Annex A.17) f_IMS_MOCallHoldResume(HOLD, v_InviteRequest, v_CallInfo.MessageHeader183, v_CallInfo.LatestSdpOffer); f_IMS_IMS_SendCoOrdMsg(OtherPDN); fl_IMS_PTC_ImsInfo_SetreINVITE(); …