3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220767

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0837 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_IMS_CheckGeolocationAbsent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is logical error in f_IMS_CheckGeolocationAbsent that can also cause runtime error if mimePartList is not present. The second condition “or lengthof(v_MsgBody.mimeMessageBody.mimePartList) == 2)” doesn’t have to be included, because if the first condition is false then the second one can never be true. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed condition “or lengthof(v_MsgBody.mimeMessageBody.mimePartList) == 2)” from f_IMS_CheckGeolocationAbsent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Function can cause runtime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 and 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_IMS_CheckGeolocationAbsent |
| --- | --- |
| Reason for change | There is logical error in f_IMS_CheckGeolocationAbsent that can also cause runtime error if mimePartList is not present. The second condition “or lengthof(v_MsgBody.mimeMessageBody.mimePartList) == 2)” doesn’t have to be included, because if the first condition is false then the second one can never be true. |
| Summary of change | Removed condition “or lengthof(v_MsgBody.mimeMessageBody.mimePartList) == 2)” |
| TTCN module | IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_IMS_CheckGeolocationAbsent(INVITE_Request p_InviteRequest) runs on IMS_PTC { // @sic R5-222427, R5-223467 sic@ var MessageHeader v_MsgHeader := p_InviteRequest.msgHeader; var MessageBody v_MsgBody := p_InviteRequest.messageBody; // absence of geolocation in the message header is to be checked if (ispresent(v_MsgHeader.geolocation) or ispresent(v_MsgHeader.geolocationRouting)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Geolocation Information provided in the message header, but should be absent"); } // absence of geolocation in the message body is to be checked if (ispresent(v_MsgBody.mimeMessageBody.mimePartList) or lengthof(v_MsgBody.mimeMessageBody.mimePartList) == 2) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Geolocation Information provided in the message body, but should be absent"); } }

After Change

function f_IMS_CheckGeolocationAbsent(INVITE_Request p_InviteRequest) runs on IMS_PTC { // @sic R5-222427, R5-223467 sic@ var MessageHeader v_MsgHeader := p_InviteRequest.msgHeader; var MessageBody v_MsgBody := p_InviteRequest.messageBody; // absence of geolocation in the message header is to be checked if (ispresent(v_MsgHeader.geolocation) or ispresent(v_MsgHeader.geolocationRouting)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Geolocation Information provided in the message header, but should be absent"); } // absence of geolocation in the message body is to be checked if (ispresent(v_MsgBody.mimeMessageBody.mimePartList)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Geolocation Information provided in the message body, but should be absent"); } }