3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220779

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2596 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the test case will not execute on a device which only creates an IMS PDU session during preamble, a run-time error will occur at step 12A if there is no internet PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow the data path check of step 12A to be performed on any active PDU session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE supporting IMS may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_2_1_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Currently the test case will not execute on a device which only creates an IMS PDU session during preamble, a run-time error will occur at step 12A if there is no internet PDU session. |
| Summary of change | Allow the check of step 12A to be performed on any active PDU session. |
| TTCN module | RRC_Reconfiguration_NR5GC.ttcn |
| MCC160 Comment | Accepted as an improvement |

Before Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; ……………………. if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); ……………….. // @siclog "Step 12A" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, Internet_DNN))), dedicatedBearer, v_DRBId, "Step 12A"); // @sic R5-197057 R5s200341 Ch. 1 R5s200571 Ch. 4 sic@ ……………….. }

After Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; /*Skipping the code */ if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); /*Skipping the code */ // @siclog "Step 12A" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 12A"); /*Skipping the code */ }