3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220787

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0841 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in R5-222078, now is also checked that a test case run with no preconditinos does not include the tag in support header in 183 Session in Progress message. A UE conformant UE not containig “precondition” tag may not include Supported header. Current implementation is checking directly contents of “Supported” header when it should check before if “Supported” header is present itself. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented statement to check first if supported header is present. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | MT call test cases with no preconditions. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction to function f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_3	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_3

| Function name | f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_3 |
| --- | --- |
| Reason for change | A UE behaving as expected not containig “precondition” tag may not include Supported header. Current implementation is checking directly contents of “Supported” header when it should check before if “Supported” header is present itself. |
| Summary of change | Implemented statement to check first if supported header is present. |
| TTCN module | Common\IMS\IMS_Procedures_5G.ttcn |
| MCC160 Comment | Accepted in principle: Further changes are needed (see below) |

Before Change:

<<SKIPPED CODE>> function f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_3(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, boolean p_PreconditionsRequired, IMS_SDP_AudioCodec_Type p_AudioCodec := EVS_16000) runs on IMS_PTC return IMS_ResponseWithSdp_Type { var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var INVITE_Request v_InviteRequest := p_InviteRequestWithSdp.Invite; var InternetProtocol_Type v_Protocol := p_InviteRequestWithSdp.RoutingInfo.Protocol; var template (present) SDP_Message v_SdpAnswer := f_IMS_BuildSDP_AnswerAudio_RX(InitialAnswer, p_AudioCodec, p_PreconditionsRequired); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequest, v_SdpAnswer, v_Protocol, p_PreconditionsRequired); /* @sic R5s210334: p_PreconditionsRequired sic@ */ if (not p_PreconditionsRequired) { /* @sic R5-222078 sic@ */ f_IMS_SDP_CheckNoPreconditionAttributes(v_ResponseWithSdp.SdpMessage); f_IMS_CheckForbiddenOptionTags(v_ResponseWithSdp.Response.msgHeader.supported.optionsTags, c_tagPrecond); } return v_ResponseWithSdp; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_3(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, boolean p_PreconditionsRequired, IMS_SDP_AudioCodec_Type p_AudioCodec := EVS_16000) runs on IMS_PTC return IMS_ResponseWithSdp_Type { var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var INVITE_Request v_InviteRequest := p_InviteRequestWithSdp.Invite; var InternetProtocol_Type v_Protocol := p_InviteRequestWithSdp.RoutingInfo.Protocol; var template (present) SDP_Message v_SdpAnswer := f_IMS_BuildSDP_AnswerAudio_RX(InitialAnswer, p_AudioCodec, p_PreconditionsRequired); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequest, v_SdpAnswer, v_Protocol, p_PreconditionsRequired); /* @sic R5s210334: p_PreconditionsRequired sic@ */ if (not p_PreconditionsRequired) { /* @sic R5-222078 sic@ */ f_IMS_SDP_CheckNoPreconditionAttributes(v_ResponseWithSdp.SdpMessage); if (ispresent(v_ResponseWithSdp.Response.msgHeader.supported)){ f_IMS_CheckForbiddenOptionTags(v_ResponseWithSdp.Response.msgHeader.supported.optionsTags, c_tagPrecond); } } return v_ResponseWithSdp; } <<SKIPPED CODE>>

## MCC160 Implementation

New Function f_IMS_CheckNoPreconditions

function f_IMS_CheckNoPreconditions(boolean p_PreconditionsForbidden, MessageHeader p_MessageHeader, SDP_Message p_SDP_Message) runs on IMS_PTC { if (p_PreconditionsForbidden) { f_IMS_SDP_CheckNoPreconditionAttributes(p_SDP_Message); if (isvalue(p_MessageHeader.supported.optionsTags)) { f_IMS_CheckForbiddenOptionTags(p_MessageHeader.supported.optionsTags, c_tagPrecond); } } }

Use of f_IMS_CheckNoPreconditions

function f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(template SipUrl p_OtherCalleeUri := omit, boolean p_PreconditionsForbidden := true) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp := f_IMS_MOCallSetup_ReceiveINVITE_Common(p_OtherCalleeUri, -, tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithoutPreconditions); f_IMS_CheckNoPreconditions(p_PreconditionsForbidden, v_InviteRequestWithSdp.Invite.msgHeader, v_InviteRequestWithSdp.SdpOffer_Message); return v_InviteRequestWithSdp; }

function f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_3(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, boolean p_PreconditionsRequired, IMS_SDP_AudioCodec_Type p_AudioCodec := EVS_16000) runs on IMS_PTC return IMS_ResponseWithSdp_Type { var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var INVITE_Request v_InviteRequest := p_InviteRequestWithSdp.Invite; var InternetProtocol_Type v_Protocol := p_InviteRequestWithSdp.RoutingInfo.Protocol; var template (present) SDP_Message v_SdpAnswer := f_IMS_BuildSDP_AnswerAudio_RX(InitialAnswer, p_AudioCodec, p_PreconditionsRequired); var boolean v_PreconditionsForbidden := not p_PreconditionsRequired; v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequest, v_SdpAnswer, v_Protocol, p_PreconditionsRequired); /* @sic R5s210334: p_PreconditionsRequired sic@ */ f_IMS_CheckNoPreconditions(v_PreconditionsForbidden, v_ResponseWithSdp.Response.msgHeader, v_ResponseWithSdp.SdpMessage); return v_ResponseWithSdp; }

function f_IMS_MOCallSetup_Video_Step2(VideoCallTypeMO_Type p_VideoCallType, template SipUrl p_OtherCalleeUri := omit, IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR, IMS_SDP_VideoCodecList_Type p_VideoCodecList := tsc_IMS_SDP_VideoCodecList_H264_only, boolean p_WithPreconditions := true) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var template (present) SDP_Message v_SDP_Message_Step2; var INVITE_A_2_1_Context_Type v_Context; var IMS_DATA_REQ v_IMS_INVITE_REQ; var SDP_Message v_SDP_Message; var boolean v_A11 := true; var boolean v_CheckForbiddenCodecParameters := f_IMS_PTC_RanTypeIsNR5GC(); var boolean v_PreconditionsSupported := p_WithPreconditions; var boolean v_PreconditionsForbidden := not p_WithPreconditions; var OptionTag_List v_OptionTagsForSupported := f_IMS_Invite_OptionTagsSupported(v_PreconditionsSupported); var IMS_SDP_AudioCodec_Type v_AudioCodec; select (p_VideoCallType) { case (SetupVideoCall) { v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAudioVideo_RX(p_AudioCodecList, p_VideoCodecList, p_WithPreconditions); v_Context := A_2_1_A4; } case (AddVideo) { v_AudioCodec := p_AudioCodecList[0]; v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAddVideo_RX(v_AudioCodec, p_VideoCodecList, p_WithPreconditions); v_Context := A_2_1_A5; v_A11 := false; } } // Step 2. Receive INVITE Request v_IMS_INVITE_REQ := f_IMS_INVITE_ReceiveRequest(v_Context, v_OptionTagsForSupported, p_OtherCalleeUri, -, -, v_A11); v_SDP_Message := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_IMS_INVITE_REQ.Request.Invite.messageBody, v_SDP_Message_Step2); // check media description: if (p_VideoCallType == SetupVideoCall) { f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_SDP_Message.media_list[0], p_AudioCodecList, v_CheckForbiddenCodecParameters); } f_IMS_SDP_CheckMediaDescr_TcapPcfg(v_SDP_Message.media_list[1]); f_IMS_CheckNoPreconditions(v_PreconditionsForbidden, v_IMS_INVITE_REQ.Request.Invite.msgHeader, v_SDP_Message); return f_IMS_InviteRequestWithSdpMO(v_IMS_INVITE_REQ, v_SDP_Message); }