3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220799

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0164 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_TC_7_3_4_4_7s_EUTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 3GPP TS 37.571-2, Table 7.3.4.4.3.3-11, at Step 5b1, one of agnssProvideLocationInformation or otdoaProvideLocationInformation shall be present in the LPP Provide Location Information message. Current implementation for function f_TC_7_3_4_4_7s_EUTRA does not follow the behavior described above. Instead, it requires both agnssProvideLocationInformation and otdoaProvideLocationInformation to be present, otherwise it throws a fail verdict. Apart from that, the cases derived from steps 4a and 4b (LPP assistance data) described in Table 7.3.4.4.3.2-1 cannot be properly distingued in current implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the implementation of function f_TC_7_3_4_4_7s_EUTRA to be spec compliant as described above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4.4.7s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1 Correction to f_TC_7_3_4_4_7s_EUTRA	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## 1.1 Correction to f_TC_7_3_4_4_7s_EUTRA

| Function name | f_TC_7_3_4_4_7s_EUTRA |
| --- | --- |
| Reason for change | According to 3GPP TS 37.571-2, Table 7.3.4.4.3.3-11, at Step 5b1, one of agnssProvideLocationInformation or otdoaProvideLocationInformation shall be present in the LPP Provide Location Information message. Current implementation for function f_TC_7_3_4_4_7s_EUTRA does not follow the behavior described above. Instead, it requires both agnssProvideLocationInformation and otdoaProvideLocationInformation to be present, otherwise it throws a fail verdict. Apart from that, the cases derived from steps 4a and 4b (LPP assistance data) described in Table 7.3.4.4.3.2-1 cannot be properly distingued in current implementation. |
| Summary of change | Change the or to and in the if statement that checks the LPP Provide location information content. Apart form that, the cases 2 and 3 for the LPP request data use now an else if statement instead of an if statement. |
| TTCN module | EUTRA_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted. |

Before

<<SKIPPED CODE>>

//See what is in the decoded message...

//CASE 1: One single Request Assistance Data message

//@sic R5-160354 sic@

if (match(v_LppMessage, cr_LppReqAssistanceDataAgnssAndOtdoa_Ack(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1)))) { //UE requested A-GNSS and OTDOA

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 3");

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

}

//CASE 2: Two seperate Request Assistance Data messages - First A-GNSS, second OTDOA

//@sic R5-160354 sic@

if (match(v_LppMessage, cr_LppReqAssistanceDataAgnss_Ack)) { //UE requested A-GNSS only

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 3");

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with A-GNSS information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

omit);

//@siclog "Step 4a" siclog@

//Receive LPP Request Assistance Data for OTDOA

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, cr_LppReqAssistanceDataOtdoa_Ack(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a");

// Get Transaction number sent by the UE

v_TransactionNumber := v_LppMessage.transactionID.transactionNumber;

//@siclog "Step 4b" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with OTDOA information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

omit,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

}

//CASE 3: Two seperate Request Assistance Data messages - First OTDOA, second A-GNSS

//@sic R5-160354 sic@

if (match(v_LppMessage, cr_LppReqAssistanceDataOtdoa_Ack(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1)))) { //UE requested OTDOA only

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 3");

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with OTDOA information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

omit,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

//@siclog "Step 4a" siclog@

//Receive LPP Request Assistance Data for A-GNSS

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, cr_LppReqAssistanceDataAgnss_Ack);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a");

// Get Transaction number sent by the UE

v_TransactionNumber := v_LppMessage.transactionID.transactionNumber;

v_RequestAssistData := v_LppMessage.lpp_MessageBody.c1.requestAssistanceData.criticalExtensions.c1.requestAssistanceData_r9;

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4b" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with A-GNSS information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

omit);

}

else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected Assistance data request");

}

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber))); //@sic R5-152147 sic@

//@sic R5-152149 sic@

//@siclog "Step 5a1" siclog@

if (match(v_LppMessage, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber)))) { //@sic R5-152147 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList);//@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a1");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 5a2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1,

tsc_POS_RoutingId,

v_LppMessage);

}

//@siclog "Step 5b1" siclog@

else if (match (v_LppMessage, cr_LppProvideLocationInformation_NoError_EarlyFix(tsc_POS_TransactionNumber))) {

if (not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) or

not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Neither A-gnss Provide Location Information nor OTDOA Provide Location Information are present");

}

//@siclog "Step 5b2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

//@siclog "Step 5b3" siclog@

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo (tsc_POS_RoutingId))))) //@sic R5s140739 sic@

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber))); //@sic R5-152147 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList); //@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4b3");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 5b4" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

// Postamble

f_EUTRA_TestBody_Set(false);

f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED);//@sic R5-103875 sic@

f_POS_StopPositioningSimulatorEUTRA (v_GNSSList);

}<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

//See what is in the decoded message...

//CASE 1: One single Request Assistance Data message

//@sic R5-160354 sic@

if (match(v_LppMessage, cr_LppReqAssistanceDataAgnssAndOtdoa_Ack(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1)))) { //UE requested A-GNSS and OTDOA

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 3");

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

}

//CASE 2: Two seperate Request Assistance Data messages - First A-GNSS, second OTDOA

//@sic R5-160354 sic@

else if (match(v_LppMessage, cr_LppReqAssistanceDataAgnss_Ack)) { //UE requested A-GNSS only

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 3");

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with A-GNSS information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

omit);

//@siclog "Step 4a" siclog@

//Receive LPP Request Assistance Data for OTDOA

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, cr_LppReqAssistanceDataOtdoa_Ack(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a");

// Get Transaction number sent by the UE

v_TransactionNumber := v_LppMessage.transactionID.transactionNumber;

//@siclog "Step 4b" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with OTDOA information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

omit,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

}

//CASE 3: Two seperate Request Assistance Data messages - First OTDOA, second A-GNSS

//@sic R5-160354 sic@

else if (match(v_LppMessage, cr_LppReqAssistanceDataOtdoa_Ack(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1)))) { //UE requested OTDOA only

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 3");

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with OTDOA information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

omit,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

//@siclog "Step 4a" siclog@

//Receive LPP Request Assistance Data for A-GNSS

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, cr_LppReqAssistanceDataAgnss_Ack);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a");

// Get Transaction number sent by the UE

v_TransactionNumber := v_LppMessage.transactionID.transactionNumber;

v_RequestAssistData := v_LppMessage.lpp_MessageBody.c1.requestAssistanceData.criticalExtensions.c1.requestAssistanceData_r9;

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4b" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with A-GNSS information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

omit);

}

else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected Assistance data request");

}

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber))); //@sic R5-152147 sic@

//@sic R5-152149 sic@

//@siclog "Step 5a1" siclog@

if (match(v_LppMessage, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber)))) { //@sic R5-152147 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList);//@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a1");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 5a2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1,

tsc_POS_RoutingId,

v_LppMessage);

}

//@siclog "Step 5b1" siclog@

else if (match (v_LppMessage, cr_LppProvideLocationInformation_NoError_EarlyFix(tsc_POS_TransactionNumber))) {

if (not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Neither A-gnss Provide Location Information nor OTDOA Provide Location Information are present");

}

//@siclog "Step 5b2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

//@siclog "Step 5b3" siclog@

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo (tsc_POS_RoutingId))))) //@sic R5s140739 sic@

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber))); //@sic R5-152147 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList); //@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4b3");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 5b4" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

// Postamble

f_EUTRA_TestBody_Set(false);

f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED);//@sic R5-103875 sic@

f_POS_StopPositioningSimulatorEUTRA (v_GNSSList);

}

<<SKIPPED CODE>>