3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220801

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2613 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Inter-RAT MDT test case 8.1.6.2.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 SON MDT test case 8.1.6.2.4 to the NR5GC_IRAT ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Rel-16 SON MDT test case 8.1.6.2.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

2.4	Change 4	7

2.5	Change 5	10

3	Branches executed	13

4	Execution Log Files	13

4.1	Mediatek MT6983	13

4.2	Qualcomm SM8450, SDX65	13

5	References	13

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.2.4 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com
	Mohit, Kanchan
	Mohit.Kanchan@keysight.com

  - Verification Test Summary

Test Case: 	8.1.6.2.4

ATS Version:	iWD-TTCN3-B2020-09_D22wk23

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

Keysight 5G Protocol Conformance Toolset

UE used:	Mediatek MT6983

Qualcomm SM8450, SDX65 UE

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | templatecs_CoOrdEUTRASysInfo() |
| --- | --- |
| Reason for change | This test is required to pass information of PhysCellId to NR component as NR has to evaluate UE reported MeasResultNeighCellListEUTRA on UEInformationResponse at step15. So cs_CoOrdEUTRASysInfo template and associated its type record should be modified. |
| Summary of change | Check below for the changes Added PhysCellId on template for CoOrdEUTRASysinfo to be passed to NR Component |
| TTCN module | CommonIratDefs.ttcn |
| MCC160 Comment | Not needed. Used existing parameter CellId instead |

Before Change:

type record CoOrd_EUTRASysInfo_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ integer Arfcn, integer DL_Bandwidth, // @sic R5s100871 Need to pass this through the co-ord msg sic@ EUTRA_HOInfo_Type HO optional, // extended for HO test cases integer CA_Tested_Integer optional, O3_Type PLMN optional, // the PLMN of the cell, defined in octets B28_Type CellId optional, // the EUTRA cell identity B16_Type TAC optional, integer freqBandIndicator optional, boolean UL_CA optional, // @sic R5-183277 sic@ integer UL_Arfcn optional // @sic R5-217955 sic@ }; template (value) CoOrd_EUTRASysInfo_Type cs_CoOrdEUTRASysInfo(integer p_Arfcn, integer p_DLBW, template (omit) EUTRA_HOInfo_Type p_HOInfo := omit, template (omit) integer p_CATested := omit, template (omit) O3_Type p_PLMN := omit, template (omit) B28_Type p_CellId := omit, template (omit) B16_Type p_TAC := omit, template (omit) integer p_FreqBandIndicator := omit, template (omit) boolean p_UL_CA := omit, template (omit) integer p_UL_Arfcn := omit) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ Arfcn := p_Arfcn, DL_Bandwidth := p_DLBW, HO := p_HOInfo, CA_Tested_Integer := p_CATested, PLMN := p_PLMN, CellId := p_CellId, TAC := p_TAC, freqBandIndicator := p_FreqBandIndicator, UL_CA := p_UL_CA, UL_Arfcn := p_UL_Arfcn // @sic R5-217955 sic@ //added PhysCellId here };

After Change:

type record CoOrd_EUTRASysInfo_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ integer Arfcn, integer DL_Bandwidth, // @sic R5s100871 Need to pass this through the co-ord msg sic@ EUTRA_HOInfo_Type HO optional, // extended for HO test cases integer CA_Tested_Integer optional, O3_Type PLMN optional, // the PLMN of the cell, defined in octets B28_Type CellId optional, // the EUTRA cell identity B16_Type TAC optional, integer freqBandIndicator optional, boolean UL_CA optional, // @sic R5-183277 sic@ integer UL_Arfcn optional, // @sic R5-217955 sic@ integer PhysCellId optional }; template (value) CoOrd_EUTRASysInfo_Type cs_CoOrdEUTRASysInfo(integer p_Arfcn, integer p_DLBW, template (omit) EUTRA_HOInfo_Type p_HOInfo := omit, template (omit) integer p_CATested := omit, template (omit) O3_Type p_PLMN := omit, template (omit) B28_Type p_CellId := omit, template (omit) B16_Type p_TAC := omit, template (omit) integer p_FreqBandIndicator := omit, template (omit) boolean p_UL_CA := omit, template (omit) integer p_UL_Arfcn := omit, template (omit) integer p_PhysCellId := omit) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ Arfcn := p_Arfcn, DL_Bandwidth := p_DLBW, HO := p_HOInfo, CA_Tested_Integer := p_CATested, PLMN := p_PLMN, CellId := p_CellId, TAC := p_TAC, freqBandIndicator := p_FreqBandIndicator, UL_CA := p_UL_CA, UL_Arfcn := p_UL_Arfcn, // @sic R5-217955 sic@ PhysCellId := p_PhysCellId };

  - Change 2

| Function name | Funtion _TC_8_1_6_2_4_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | - Prose is expecting System information combination NR-6 and EUTRA SystemInformationBlockType24 so Scheduling for combination 31 should be used for it. - Max reference power can not be same as SS power for EUTRA cell. - NR component needed EUTRA frequency information to enable SIB5 so added co-ordination system information to send EUTRA cell information to NR. - EUTRA component need to be activated once NR component has finished premable steps. |
| Summary of change | Check below for the changes 3. Added co-ordinationinfo to be sent to NR component to pass EUTRA frequency information from EUTRA component. Change 1,2 and 4 is not required and looks unnecessary now. And it happened Due to refinment of CR during joint submission |
| TTCN module | RRC_MDT_InterRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | 1,2,4. Not needed 3. Done differently, see implementation below |

Before Change:

function f_TC_8_1_6_2_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT MDT / Radio Link Failure / Reporting at E-UTRA Inter-RAT handover f_EUTRA38_IRAT_Init(c1); f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell1, -90); // Send EUTRA frequency to NR //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_TestBody_Set(true); fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

After Change:

import from EUTRA_CellInfoInit all; function f_TC_8_1_6_2_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT MDT / Radio Link Failure / Reporting at E-UTRA Inter-RAT handover f_EUTRA38_IRAT_Init(c1); f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell1, -90); // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_TestBody_Set(true); fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

  - Change 3

| Function name | function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | System Co-ordination required to send EUTRA cell information to NR test body. |
| Summary of change | Check below for the changes Added co-ordinationinfo to be sent to NR component to pass EUTRA CellID information from EUTRA component. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted in principle, but used existing parameter CellId |

Before Change:

function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //added System coordination here f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, -90, -90); //FR2 is FFS }

After Change:

function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; v_CoOrd_SysInfo := cs_EUTRA_NR_CoOrdSysInfo_Eutra({f_EUTRA_CoOrdSysInfo(eutra_Cell1)}); v_CoOrd_SysInfo.Eutra[0].PhysCellId := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1); //adds the phycellID to the coorData f_EUTRA_NR_SendSysinfoCoOrd(NR, v_CoOrd_SysInfo); f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, -90, -90); //FR2 is FFS }

MCC160 Implementation

function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var template (value) B28_Type v_EUTRA_CellId;

var template (value) CoOrd_EUTRASysInfo_Type v_CoOrd_SysInfo := f_EUTRA_CoOrdSysInfo(eutra_Cell1);

v_EUTRA_CellId := int2bit(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1),28);

v_CoOrd_SysInfo.CellId := v_EUTRA_CellId;

f_EUTRA_NR_SendSysinfoCoOrd(NR, cs_EUTRA_NR_CoOrdSysInfo_Eutra({v_CoOrd_SysInfo}));

f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Power Level");

f_EUTRA38_SetCellPower(eutra_Cell1, -90, -90);  //FR2 is FFS

}

  - Change 4

| Function name | function f_TC_8_1_6_2_4_TestBody () |
| --- | --- |
| Reason for change | - No need to bit to integer conversion on v_CoOrd_SysInfo.Eutra[0].CellId as it will return EUTRA_PhysCellId type. - Parameters to be passed on got interchnaged on MeasResultNeighCellListEUTRA at step 15 |
| Summary of change | Check below for the changes - Bit2int conversion not required due to chagne 1. - Swapped entry of v_EUTRA_CellId and v_ARFCN_ValueEUTRA on cr_MeasResultNeighCellListEUTRA_1Entry template |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment | - Rejected – this is required for existing field 2.Agreed |

Before Change:

function f_TC_8_1_6_2_4_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var CellIdentity v_CellIdentity_Cell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var PLMN_Identity v_PLMN_Identity_Cell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); //@siclog "Steps 1" siclog@ //The SS changes NR Cell 1 and E-UTRA Cell 1 parameter according to the row "T1" in Table 8.1.6.2.4.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Power Level")); //@siclog "Steps 2" siclog@ //The SS waits for [30s] to ensure that the UE detects E-UTRA cell. f_Delay(30.0); //@siclog "Step 3" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 4" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 5" siclog@ //The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. f_Delay(2.0); //@siclog "Step 6-8" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit, true_, omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Steps 10-13" siclog@ //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 14" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, true_))); //@siclog "Step 15" siclog@ //Check: Check: Does the UE send a UEInformationResponse message with connEstFailReport containing the measurement result for E-UTRA Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def, -, -, -, cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1)), omit,cr_MeasResultNeighCellListEUTRA_1Entry(v_EUTRA_CellId,v_ARFCN_ValueEUTRA, f_EUTRA_Get_MeasQuantityResults()))))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); }// End of f_TC_8_1_6_2_4__TestBody()function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { return false; }

After Change:

function f_TC_8_1_6_2_4_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var CellIdentity v_CellIdentity_Cell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var PLMN_Identity v_PLMN_Identity_Cell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := v_CoOrd_SysInfo.Eutra[0].PhysCellId; // removed bit2int from here //@siclog "Steps 1" siclog@ //The SS changes NR Cell 1 and E-UTRA Cell 1 parameter according to the row "T1" in Table 8.1.6.2.4.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Power Level")); //@siclog "Steps 2" siclog@ //The SS waits for [30s] to ensure that the UE detects E-UTRA cell. f_Delay(30.0); //@siclog "Step 3" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 4" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 5" siclog@ //The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. f_Delay(2.0); //@siclog "Step 6-8" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit, true_, omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Steps 10-13" siclog@ //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 14" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, true_))); //@siclog "Step 15" siclog@ //Check: Check: Does the UE send a UEInformationResponse message with connEstFailReport containing the measurement result for E-UTRA Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def, -, -, -, cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1)), omit,cr_MeasResultNeighCellListEUTRA_1Entry(v_ARFCN_ValueEUTRA,v_EUTRA_CellId, f_EUTRA_Get_MeasQuantityResults()))))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); }// End of f_TC_8_1_6_2_4__TestBody

  - Change 5

| Function name | function f_GetTestcaseAttrib_EarlyContentionResolution() |
| --- | --- |
| Reason for change | At step 6-8 rrcsetup is scheduled as not part of rach procedure so test should be added on f_GetTestcaseAttrib_EarlyContentionResolution |
| Summary of change | Check below for the changes Please provide details of changes made Added TC_8_1_6_2_4_NR5GC on f_GetTestcaseAttrib_EarlyContentionResolution for NR5GC_IRAT TC. |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment | Agreed |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { return false; }

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_8_1_6_2_4_NR5GC") { return true; } } return false; }

- Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

On Keysight, the test case was executed on NR band n78 using NR ciphering nea1 and integrity algorithm nia1.

- Execution Log Files

  - Mediatek MT6983

The Mediatek MT6983UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_6_2_4_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_6_2_4_PIXIT.txt

  - Qualcomm SM8450, SDX65

The Mediatek MT6983UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight\ TC_8_1_6_2_4_LOG.html

- PICS/PIXIT parameter file:

Keysight\ TC_8_1_6_2_4_PIXIT.xml

- References

| [1] | R5s220802 - Supporting information for addition of NR5GC Logged MDT test case 8.1.6.2.4 in FR1 |
| --- | --- |