3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220816

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2626 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the testcases 8.1.4.1.7.x and 8.1.4.1.8.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of function : f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common - v_DrbIdList is initialized at the beginning of the function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common and the initialized value is getting used in Steps 17a.1 and 18.1. It is proposed to update v_DrbIdList value before making use of it in steps 17a.1 and 18.1 - At step 18.1, SRB DRB release is expected to happen on Source Scell instead of Source Pcell - In case of 8.1.4.1.7.1, during the test steps 7- 8 , NR Cell 3 NR Cell 23 handover happens with NR Cell 2 gets released as Scell . and scell (NR cell 2 ) was born as scell inactive ( only SRB0 was configured ). It is proposed to add extra checks before doing f_NR_SS_SRBs_DRBs_Release at Step 18.1 - In case of 8.1.4.1.7.1, during the test steps 4-5, NR Cell 1 NR Cell 3 handover happens with NR Cell 2 added as Scell ; when target Pcell is same as Source Scell , SRB DRB reset on Source Pcell is happening twice in the function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common - 1st time SRB DRB RESET :: inside f_NR_ReconfigurePCellToNormalCell which is called from 4.1,5th steps of f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common - 2nd time SRB DRB RESET :: as part of 17.1 step of f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common ) this can be improvised by blocking the redundant RB reset and release inside f_NR_ReconfigurePCellToNormalCell based on the condition - This way we can also make sure RB reset is done on source Cell only after the PDCP HO competition in align with other HO cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | - v_DrbIdList is updated every time before it gets used in the function calls in Steps 17a.1 and 18.1 - At step 18.1, SRB DRB release is expected to happen on Source Scell - RB Count check is added before doing f_NR_SS_SRBs_DRBs_Release at Step 18.1 - SRB DRB reset in the function f_NR_ReconfigurePCellToNormalCell is guarded with the condition so that it avoids the RB RESET on Source Pcell happening twice when source Scell is becoming target Pcell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.7.1.NR5GC, 8.1.4.1.7.2.NR5GC, 8.1.4.1.7.3.NR5GC, 8.1.4.1.8.1.NR5GC, 8.1.4.1.8.2.NR5GC, 8.1.4.1.8.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for NR testcases 8.1.4.1.7.x and 8.1.4.1.8.x	3

2.1	Correction to the function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common	3

2.2	Correction to the function f_NR_ReconfigurePCellToNormalCell	14

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcases 8.1.4.1.7.x and 8.1.4.1.8.x

## Correction to the function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common

| Function name | f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common |
| --- | --- |
| Reason for change | In the current TTCN implementation of function : f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common - v_DrbIdList is initialized at the beginning of the function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common and the initialized value is getting used in Steps 17a.1 and 18.1. It is proposed to update v_DrbIdList value before making use of it in steps 17a.1 and 18.1 - At step 18.1, SRB DRB release is expected to happen on Source Scell instead of Source Pcell - In case of 8.1.4.1.7.1, during the test steps 7- 8 , NR Cell 3 NR Cell 23 handover happens with NR Cell 2 gets released as Scell . and scell (NR cell 2 ) was born as scell inactive ( only SRB0 was configured ). It is proposed to add extra checks before doing f_NR_SS_SRBs_DRBs_Release at Step 18.1 - In case of 8.1.4.1.7.1, during the test steps 4-5, NR Cell 1 NR Cell 3 handover happens with NR Cell 2 added as Scell ; when target Pcell is same as Source Scell , SRB DRB reset on Source Pcell is happening twice in the function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common - 1st time SRB DRB RESET inside f_NR_ReconfigurePCellToNormalCell which is called from 4.1,5th steps of f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common ( with activation time ) - 2nd time SRB DRB RESET as part of 17.1 step of f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common ) this can be improvised by blocking the redundant RB reset and release inside f_NR_ReconfigurePCellToNormalCell based on the condition ( with immediate activation ) - This way we can also make sure RB reset is done on source Cell only after the PDCP HO competition in align with other HO cases |
| Summary of change | - v_DrbIdList is updated every time before it gets used in the function calls in Steps 17a.1 and 18.1 - At step 18.1, SRB DRB release is expected to happen on Source Scell - RB Count check is added before doing f_NR_SS_SRBs_DRBs_Release at Step 18.1 - SRB DRB reset in the function f_NR_ReconfigurePCellToNormalCell is guarded with the condition so that it avoids the RB RESET on Source Pcell happening twice when source Scell is becoming target Pcell |
| TTCN module | NR5GC_Handover.ttcn |
| MCC160 Comment | - Accepted - Accepted - Accepted - Accepted |

Before Change:

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, template(omit) CellGroupConfig p_SCGConfig_SCell, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (value) DL_DCCH_Message p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) NR_DRBInfoList_Type p_DrbInfoList, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR_BASE_PTC { //@sic R5s201249: added p_SCGConfig_SCell R5-213516: added MacBearerRouting sic@ var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(p_DrbInfoList); var IntegerList_Type v_DrbIdList:= f_NR_GetActiveDRBs(p_SourcePCellId); var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s201387 sic@ v_Timing := f_NR_GetNextSendOccasion_DL(p_SourcePCellId, 700); // 700 ms // Time for schedule of DL RRC message v_Timing1 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 10); // time for Target SCell-> PCell configuration SS configuration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 20); // time for Target PCell SS configuration // 1. Source PCell: Stop periodic TA. // NOTE 1: Unless explicitly specified UL grant configuration keeps configured as per default at the source cell. f_NR_ULGrantConfiguration_Common(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), -, cs_NR_UplinkTimeAlignment_Stop, p_UL_GrantConfig_SourceCell); // 2. Target PCell: Configure target PCell for no RACH response transmission. //Calculate T in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), p_TargetPCellId); f_NR_SS_RachProcedureConfig(p_TargetPCellId, cs_NR_RachProcedureConfig_NoResponse, v_TimingInfo); // 3. Source PCell: Schedule the transmission of RRCReconfiguration message to UE requesting Handover to target PCell and SCell. SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), p_RRCReconfiguration, crs_MacBearerRouting(p_SourcePCellId))); //@sic R5-213516 sic@ // At time T1 if target PCell is the same as source SCell, else at time "Now", steps 4-6: if (p_TargetPCellId == p_SourceSCellId) { //@sic R5-223443 R5s220590 sic@ //Calculate T1 in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing1), p_TargetPCellId); // 4.1 Source PCell: Configure from PCell to normal cell: remove source SCell from the SCell list in the source PCell // 5. Target PCell: Configuration of DRBs f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, v_TimingInfo, v_DrbIdList); // 4.2 Target PCell: Configure from SCell to PCell. f_NR_ReconfigSCellToNormalCell(p_TargetPCellId, v_RadioBearerList, v_TimingInfo); } else { // At time Now, steps 5-6: // 5. Target PCell: Configuration of DRBs f_NR_SS_CommonRadioBearerConfig(p_TargetPCellId, v_RadioBearerList); //@sic R5s201249 sic@ } //6. Transfer of the PDCP Count for AM DRBs and SRBs (if applied) from source to target PCell: // a) Source PCell: Get PDCP COUNT. // b) Target PCell: Set PDCP COUNT. // NOTE 2: No further sending/receiving of DRB data before the HO is done. // NOTE 3: For AM DRBs the PDCP count is maintained, for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. if (p_TargetPCellId == p_SourceSCellId) { f_NR_SS_PdcpCount_Handover(p_SourcePCellId, p_TargetPCellId, v_RadioBearerList, p_RBConfig_KeyChange, cs_TimingInfo_NR(v_Timing1)); } else { f_NR_SS_PdcpCount_Handover(p_SourcePCellId, p_TargetPCellId, v_RadioBearerList, p_RBConfig_KeyChange); //@sic R5s201249 sic@ } //At time T2, steps 7-12: //7. Target PCell: Inform the SS about the HO and about the source PCell id. f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverInit_REQ(p_SourcePCellId, p_TargetPCellId, cs_TimingInfo_NR(v_Timing2))); //8. Target PCell: Configure RACH procedure either dedicated or C-RNTI based. // NOTE 4: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI f_NR_CellInfo_SetRNTI(p_TargetPCellId, p_RNTI_Value); //Calculate T2 in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), p_TargetPCellId); if (isvalue(p_TargetSCellId)) { //@sic R5s210079 sic@ //@sic R5s201249 sic@ f_NR_CellInfo_SetRNTI(valueof(p_TargetSCellId), p_RNTI_Value); f_NR_SS_C_RNTI_Config(valueof(p_TargetSCellId), p_RNTI_Value, cs_TimingInfo_NR(v_Timing2)); //@sic R5s210389 sic@ } f_NR_SS_C_RNTI_Config(p_TargetPCellId, p_RNTI_Value, cs_TimingInfo_NR(v_Timing2), tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369, R5s210389 sic@ f_NR_SS_RachProcedureConfigHO(p_TargetPCellId, p_RachProcedureConfig, v_TimingInfo, p_RACH_ConfigDedicated); //9. Target PCell: Activate security. if ((p_NCC > tsc_NR_38508_NextHopChainingCount) or p_RBConfig_KeyChange) { // @sic R5s190795 R5s200159 sic@ f_NR_SS_AS_ActivateSecurity_Current(p_TargetPCellId, p_NCC, v_TimingInfo); } else { f_NR_SS_AS_ActivateSecurity_NoRefresh(p_TargetPCellId, v_TimingInfo); //@sic R5s200102 sic@ } //10. Target PCell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetPCellId, v_TimingInfo, -, -, p_UL_GrantConfig_TargetCell); //11. Target PCell: Configure Target PCell as PCell. //12. Target SCell: Configure Target SCell as SCell with new PCell association. //NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PCell. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ f_NR_ConfigPCell_AddRelSCells(p_TargetPCellId, {valueof(p_TargetSCellId)}, tsc_PCellIndex_0, v_TimingInfo, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ //Calculate T2 in the TargetSCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), valueof(p_TargetSCellId)); f_NR_ConfigSCell(valueof(p_TargetSCellId), p_SCGConfig_SCell, v_TimingInfo); } // After time T2 (without activation time): // 13. Target PCell: Receive RRCReconfigurationComplete. // when CA is not active , make sure there is no MAC Bearer Routing ( refer step 7-8 of the test case 8.1.4.1.7.1 ) if(not isvalue(p_TargetSCellId)) { //@sic R5s220590 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetPCellId, p_RRCConnReconfigComplete)); } else { SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetPCellId, p_RRCConnReconfigComplete, -, crs_MacBearerRouting(p_TargetPCellId))); //@sic R5-213516 sic@ } //14. Target Pell: Start periodic TA. f_NR_ULGrantConfiguration_Common(p_TargetPCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -); //15. Target Pell: Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPCellId)); //16. Target PCell: Re-configure RACH procedure as for initial access. f_NR_SS_RachProcedureConfig_Def(p_TargetPCellId); //17. Source PCell: If source PCell is not target SCell //17.1 Reset SRBs and release DRBs. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ if (p_SourcePCellId != valueof(p_TargetSCellId)) { //17.1 Reset SRBs and release DRBs. f_NR_SS_SRBs_DRBs_Release(p_SourcePCellId, cs_TimingInfo_Now, v_DrbIdList); f_NR_SS_SRBs_DRBs_Config(p_SourcePCellId, cs_TimingInfo_Now, omit); } } //17a. Source PCell: If target PCell is not the same as source SCell: //17a.1 Configure from PCell to normal cell: remove source SCell from the SCell list in the source PCell //@sic R5-223443 R5s220590 sic@ if (p_TargetPCellId != p_SourceSCellId) { f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, -, v_DrbIdList ); } //18. Source SCell: If source SCell is neither target PCell nor target SCell: //18.1 Release DRBs //@sic R5-223443 R5s220590 sic@ if (not(match(p_SourceSCellId, p_TargetPCellId)) and not match(p_SourceSCellId, p_TargetSCellId)) { f_NR_SS_SRBs_DRBs_Release(p_SourcePCellId, cs_TimingInfo_Now, v_DrbIdList); } }

After Change:

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, template(omit) CellGroupConfig p_SCGConfig_SCell, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (value) DL_DCCH_Message p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) NR_DRBInfoList_Type p_DrbInfoList, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR_BASE_PTC { //@sic R5s201249: added p_SCGConfig_SCell R5-213516: added MacBearerRouting sic@ var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(p_DrbInfoList); var IntegerList_Type v_DrbIdList:= f_NR_GetActiveDRBs(p_SourcePCellId); var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s201387 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList := {}; //WA#WI=973937 v_Timing := f_NR_GetNextSendOccasion_DL(p_SourcePCellId, 700); // 700 ms // Time for schedule of DL RRC message v_Timing1 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 10); // time for Target SCell-> PCell configuration SS configuration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 20); // time for Target PCell SS configuration // 1. Source PCell: Stop periodic TA. // NOTE 1: Unless explicitly specified UL grant configuration keeps configured as per default at the source cell. f_NR_ULGrantConfiguration_Common(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), -, cs_NR_UplinkTimeAlignment_Stop, p_UL_GrantConfig_SourceCell); // 2. Target PCell: Configure target PCell for no RACH response transmission. //Calculate T in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), p_TargetPCellId); f_NR_SS_RachProcedureConfig(p_TargetPCellId, cs_NR_RachProcedureConfig_NoResponse, v_TimingInfo); // 3. Source PCell: Schedule the transmission of RRCReconfiguration message to UE requesting Handover to target PCell and SCell. SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), p_RRCReconfiguration, crs_MacBearerRouting(p_SourcePCellId))); //@sic R5-213516 sic@ // At time T1 if target PCell is the same as source SCell, else at time "Now", steps 4-6: if (p_TargetPCellId == p_SourceSCellId) { //@sic R5-223443 R5s220590 sic@ //Calculate T1 in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing1), p_TargetPCellId); // 4.1 Source PCell: Configure from PCell to normal cell: remove source SCell from the SCell list in the source PCell // 5. Target PCell: Configuration of DRBs f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, v_TimingInfo, v_DrbIdList,true); //WA#WI=973937 // 4.2 Target PCell: Configure from SCell to PCell. f_NR_ReconfigSCellToNormalCell(p_TargetPCellId, v_RadioBearerList, v_TimingInfo); } else { // At time Now, steps 5-6: // 5. Target PCell: Configuration of DRBs f_NR_SS_CommonRadioBearerConfig(p_TargetPCellId, v_RadioBearerList); //@sic R5s201249 sic@ } //6. Transfer of the PDCP Count for AM DRBs and SRBs (if applied) from source to target PCell: // a) Source PCell: Get PDCP COUNT. // b) Target PCell: Set PDCP COUNT. // NOTE 2: No further sending/receiving of DRB data before the HO is done. // NOTE 3: For AM DRBs the PDCP count is maintained, for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. if (p_TargetPCellId == p_SourceSCellId) { f_NR_SS_PdcpCount_Handover(p_SourcePCellId, p_TargetPCellId, v_RadioBearerList, p_RBConfig_KeyChange, cs_TimingInfo_NR(v_Timing1)); } else { f_NR_SS_PdcpCount_Handover(p_SourcePCellId, p_TargetPCellId, v_RadioBearerList, p_RBConfig_KeyChange); //@sic R5s201249 sic@ } //At time T2, steps 7-12: //7. Target PCell: Inform the SS about the HO and about the source PCell id. f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverInit_REQ(p_SourcePCellId, p_TargetPCellId, cs_TimingInfo_NR(v_Timing2))); //8. Target PCell: Configure RACH procedure either dedicated or C-RNTI based. // NOTE 4: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI f_NR_CellInfo_SetRNTI(p_TargetPCellId, p_RNTI_Value); //Calculate T2 in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), p_TargetPCellId); if (isvalue(p_TargetSCellId)) { //@sic R5s210079 sic@ //@sic R5s201249 sic@ f_NR_CellInfo_SetRNTI(valueof(p_TargetSCellId), p_RNTI_Value); f_NR_SS_C_RNTI_Config(valueof(p_TargetSCellId), p_RNTI_Value, cs_TimingInfo_NR(v_Timing2)); //@sic R5s210389 sic@ } f_NR_SS_C_RNTI_Config(p_TargetPCellId, p_RNTI_Value, cs_TimingInfo_NR(v_Timing2), tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369, R5s210389 sic@ f_NR_SS_RachProcedureConfigHO(p_TargetPCellId, p_RachProcedureConfig, v_TimingInfo, p_RACH_ConfigDedicated); //9. Target PCell: Activate security. if ((p_NCC > tsc_NR_38508_NextHopChainingCount) or p_RBConfig_KeyChange) { // @sic R5s190795 R5s200159 sic@ f_NR_SS_AS_ActivateSecurity_Current(p_TargetPCellId, p_NCC, v_TimingInfo); } else { f_NR_SS_AS_ActivateSecurity_NoRefresh(p_TargetPCellId, v_TimingInfo); //@sic R5s200102 sic@ } //10. Target PCell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetPCellId, v_TimingInfo, -, -, p_UL_GrantConfig_TargetCell); //11. Target PCell: Configure Target PCell as PCell. //12. Target SCell: Configure Target SCell as SCell with new PCell association. //NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PCell. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ f_NR_ConfigPCell_AddRelSCells(p_TargetPCellId, {valueof(p_TargetSCellId)}, tsc_PCellIndex_0, v_TimingInfo, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ //Calculate T2 in the TargetSCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), valueof(p_TargetSCellId)); f_NR_ConfigSCell(valueof(p_TargetSCellId), p_SCGConfig_SCell, v_TimingInfo); } // After time T2 (without activation time): // 13. Target PCell: Receive RRCReconfigurationComplete. // when CA is not active , make sure there is no MAC Bearer Routing ( refer step 7-8 of the test case 8.1.4.1.7.1 ) if(not isvalue(p_TargetSCellId)) { //@sic R5s220590 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetPCellId, p_RRCConnReconfigComplete)); } else { SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetPCellId, p_RRCConnReconfigComplete, -, crs_MacBearerRouting(p_TargetPCellId))); //@sic R5-213516 sic@ } //14. Target Pell: Start periodic TA. f_NR_ULGrantConfiguration_Common(p_TargetPCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -); //15. Target Pell: Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPCellId)); //16. Target PCell: Re-configure RACH procedure as for initial access. f_NR_SS_RachProcedureConfig_Def(p_TargetPCellId); //17. Source PCell: If source PCell is not target SCell //17.1 Reset SRBs and release DRBs. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ if (p_SourcePCellId != valueof(p_TargetSCellId)) { //17.1 Reset SRBs and release DRBs. f_NR_SS_SRBs_DRBs_Release(p_SourcePCellId, cs_TimingInfo_Now, v_DrbIdList); f_NR_SS_SRBs_DRBs_Config(p_SourcePCellId, cs_TimingInfo_Now, omit); } } //17a. Source PCell: If target PCell is not the same as source SCell: //17a.1 Configure from PCell to normal cell: remove source SCell from the SCell list in the source PCell //@sic R5-223443 R5s220590 sic@ if (p_TargetPCellId != p_SourceSCellId) { v_DrbIdList:= f_NR_GetActiveDRBs(p_SourcePCellId); //WA#WI=973937 f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, -, v_DrbIdList, false ); //WA#WI=973937 } //18. Source SCell: If source SCell is neither target PCell nor target SCell: //18.1 Release DRBs //@sic R5-223443 R5s220590 sic@ if (not(match(p_SourceSCellId, p_TargetPCellId)) and not match(p_SourceSCellId, p_TargetSCellId)) { v_PdcpCountInfoList := f_NR_SS_PdcpCount_Get(p_SourceSCellId); //WA#WI=973937 if(lengthof(v_PdcpCountInfoList) > 0) //WA#WI=973937 { //WA#WI=973937 v_DrbIdList:= f_NR_GetActiveDRBs(p_SourceSCellId); //WA#WI=973937 f_NR_SS_SRBs_DRBs_Release(p_SourceSCellId, cs_TimingInfo_Now, v_DrbIdList); //WA#WI=973937 } } //WA#WI=973937 }

## Correction to the function f_NR_ReconfigurePCellToNormalCell

| Function name | f_NR_ReconfigurePCellToNormalCell |
| --- | --- |
| Reason for change | In case of 8.1.4.1.7.1, during the test steps 4-5, NR Cell 1 NR Cell 3 handover happens with NR Cell 2 added as Scell ; when target Pcell is same as Source Scell , SRB DRB reset is happening twice in the function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common - 1st time SRB DRB RESET inside f_NR_ReconfigurePCellToNormalCell which is called from 4.1,5th steps of f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common - 2nd time SRB DRB RESET as part of 17.1 step of f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common ) this can be improvised by blocking the redundant RB reset and release inside f_NR_ReconfigurePCellToNormalCell based on the condition |
| Summary of change | SRB DRB reset in the function f_NR_ReconfigurePCellToNormalCell is guarded with the condition so that it avoids the RB RESET on Source Pcell happening twice when source Scell is becoming target Pcell |
| TTCN module | NR_CommonFunctions_CA |
| MCC160 Comment | Accepted |

Before Change:

//------------------------------------------------------------------------------------------------------------------ /* * @desc Common function to reconfigure a PSCell to a normal cell * @param p_PCell * @param p_PServingCellIndex * @param p_TimingInfo (default value: cs_TimingInfo_Now) * @param p_DrbToReleassList * @status APPROVED (NR5GC) */ function f_NR_ReconfigurePCellToNormalCell( NR_CellId_Type p_PCell, NR_ServingCellIndex_Type p_PServingCellIndex, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, IntegerList_Type p_DrbToReleassList ) runs on NR_BASE_PTC { f_NR_ConfigPCell_AddRelSCells( p_PCell, {}, //p_SCellList p_PServingCellIndex, p_TimingInfo, omit, // p_RNTI_Value > no need to reconfigure C-RNTI omit // p_SS_RadioBearerList > DRB + SRBs to be released just after ); //Release DRBs f_NR_SS_SRBs_DRBs_Release(p_PCell, p_TimingInfo, p_DrbToReleassList); //@sic R5s201249 sic@ //Release and re-establish SRBs - if not ENDC if (not f_GetTestcaseAttrib_ENDC(testcasename())) { f_NR_SS_SRBs_DRBs_Config(p_PCell, p_TimingInfo, omit); //@sic R5s201249 sic@ } }

After Change:

//------------------------------------------------------------------------------------------------------------------ /* * @desc Common function to reconfigure a PSCell to a normal cell * @param p_PCell * @param p_PServingCellIndex * @param p_TimingInfo (default value: cs_TimingInfo_Now) * @param p_DrbToReleassList * @param ScellBecomingPcell //WA#WI=973937 * @status APPROVED (NR5GC) */ function f_NR_ReconfigurePCellToNormalCell( NR_CellId_Type p_PCell, NR_ServingCellIndex_Type p_PServingCellIndex, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, IntegerList_Type p_DrbToReleassList, boolean ScellBecomingPcell := false //WA#WI=973937 ) runs on NR_BASE_PTC { f_NR_ConfigPCell_AddRelSCells( p_PCell, {}, //p_SCellList p_PServingCellIndex, p_TimingInfo, omit, // p_RNTI_Value > no need to reconfigure C-RNTI omit // p_SS_RadioBearerList > DRB + SRBs to be released just after ); //Release DRBs if(not ScellBecomingPcell) //WA#WI=973937 { f_NR_SS_SRBs_DRBs_Release(p_PCell, p_TimingInfo, p_DrbToReleassList); //@sic R5s201249 sic@ //Release and re-establish SRBs - if not ENDC if (not f_GetTestcaseAttrib_ENDC(testcasename())) { f_NR_SS_SRBs_DRBs_Config(p_PCell, p_TimingInfo, omit); //@sic R5s201249 sic@ } } //WA#WI=973937 }