3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220821

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2629 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_EmergencyCallWithRegistration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of testcase 11.4.6, the function fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession is called which establishes a PDU Session for Emergency. After this the NR PTC sends a coordination message to IMS PTC so that procedure for IMS Emergency Voice call (A.6) is performed. Once this is done, the IMS PTC sends a coordination message to the NR PTC to add voice. After this, the IMS PTC waits for a coordination message from the NR PTC to release the call : f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_IPCAN_WaitForTrigger(IPCAN); Once the NR PTC receives the coordination message from IMS PTC, the function f_NR5GC_ModifyPDUSession_AddVoice is called to add voice (fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice), and then then a coordination message is expected from IMS PTC : fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); At this stage, both, the NR PTC and IMS PTC are waiting for a coordination message and the testcase gets stuck. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After the NR PTC has added voice by calling function f_NR5GC_ModifyPDUSession_AddVoice, it should send a coordination message to IMS PTC, rather than wait for a coordination message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Instead of calling fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); Called : fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.6.NR5GC and other NR5GC emergency call testcases. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for function f_NR5GC_EmergencyCallWithRegistration	3

2.1	Correction to f_NR5GC_EmergencyCallWithRegistration	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for function f_NR5GC_EmergencyCallWithRegistration

## Correction to f_NR5GC_EmergencyCallWithRegistration

| Template name | f_NR5GC_EmergencyCallWithRegistration |
| --- | --- |
| Reason for change | In the current TTCN implementation of testcase 11.4.6, the function fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession is called which establishes a PDU Session for Emergency. After this the NR PTC sends a coordination message to IMS PTC so that procedure for IMS Emergency Voice call (A.6) is performed. Once this is done, the IMS PTC sends a coordination message to the NR PTC to add voice. After this, the IMS PTC waits for a coordination message from the NR PTC to release the call : f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_IPCAN_WaitForTrigger(IPCAN); Once the NR PTC receives the coordination message from IMS PTC, the function f_NR5GC_ModifyPDUSession_AddVoice is called to add voice (fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice), and then then a coordination message is expected from IMS PTC : fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); At this stage, both, the NR PTC and IMS PTC are waiting for a coordination message and the testcase gets stuck. This needs to be addressed. |
| Summary of change | Instead of calling fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); Called : fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted, but added this to f_NR5GC_EmergencyCallPDUSession instead. Also removed the ‘same’ trigger from f_NR5GC_IPCAN_EmergencyCall as this is no longer needed |

Before Change:

<<SKIPPED CODE>> function f_NR5GC_EmergencyCallWithRegistration(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { // Split into 2 functions to accomodate IMS emergency TCs var integer v_PDUEstAlreadyRxd := f_NR5GC_EmergencyCallWithRegistration_Steps1_8 (p_NR_CellId); // @sic R5-217857 sic@ // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, -, v_PDUEstAlreadyRxd); // @sic R5-217286 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); // Emergency call now active } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_NR5GC_EmergencyCallWithRegistration(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { // Split into 2 functions to accomodate IMS emergency TCs var integer v_PDUEstAlreadyRxd := f_NR5GC_EmergencyCallWithRegistration_Steps1_8 (p_NR_CellId); // @sic R5-217857 sic@ // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, -, v_PDUEstAlreadyRxd); // @sic R5-217286 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //WA#11_4_6 Coordination message to tell the IMS PTC that Voice is added to the Emergency PDU session f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); // Emergency call now active } <<SKIPPED CODE>>

#### MCC160 Implementation

function f_NR5GC_IPCAN_EmergencyCall(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId,

boolean p_ConnectionEstablished) runs on NR5GC_PTC

{

var integer v_PDUEstAlreadyRxd := 0;

if (p_ConnectionEstablished) {

f_NR5GC_EmergencyCallPDUSession(p_NR_CellId);

} else { // @sic R5-217857 sic@

v_PDUEstAlreadyRxd := f_NR5GC_EmergencyCallWithRegistration_Steps1_8 (p_NR_CellId);

f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, -, v_PDUEstAlreadyRxd);

}

}

function f_NR5GC_EmergencyCallPDUSession(NR_CellId_Type p_NR_CellId,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2,

integer p_PDUEstAlreadyRxd := 0) runs on NR5GC_PTC

{

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo;

f_NR5GC_PDUSessionEstablishment(p_NR_CellId,

1,

p_SRB, -,                       // @sic R5s210518 sic@

omit,

omit,

crs_SSC_Mode(-, '001'B),        // @sic R5s210518 sic@

-,

f_BuildDefaultQoSRules("2"),

-, -, -, -, -, -,

crs_SSC_Mode('A'H, '001'B),        // @sic R5s210518, R5s210771 sic@

-, -, -, -, -, -, -,

cr_NG_Request_Type('011'B),

-, -, -, -, -, -, -, -,

"2",

-,

noRrcConnectionRelease,        // @sic R5s210515 sic@ don't need the extra delay here as there's further IMS signalling

p_PDUEstAlreadyRxd);           // @sic R5-217286 sic@

// Get the Info for the PDU Session just created

v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN);

// Wait until need to add voice QoS

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo));

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s210518, R5s220821 sic@

}