3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220828

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2636 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC PS data off test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Following implementation of change 2.3 of R5s220424, the function f_ExtdProtocolConfigOptions_CheckPSDataOff() is changed to check for status ‘02’ (=”activated”) instead of status ‘01’ (=”deactivated”), to check the PDU SESSION MODIFICATION REQUEST at step 2 of 11.6.2. But the same function is already used for checking all initial PDU SESSION ESTABLISHMENT REQUEST messages to check for status ‘01’ (for UEs supporting the feature), according to 38.508-1 table 4.7.2-1. Following this change all test cases will fail for UEs with pc_PS_data_off = TRUE due to this check. - Missing coordination trigger for 11.6.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Parameterise this function to be able to check for PS data status 01 (=”deactivated”) or 02 (=”activated”) - Add coordination trigger for 11.6.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6.1.NR5GC, 11.6.2.NR5GC, 11.6.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_ExtdProtocolConfigOptions_CheckPSDataOff() |
| --- | --- |
| Reason for change | Following implementation of change 2.3 of R5s220424, the function f_ExtdProtocolConfigOptions_CheckPSDataOff() is changed to check for status ‘02’ (=”activated”) instead of status ‘01’ (=”deactivated”), to check the PDU SESSION MODIFICATION REQUEST at step 2 of 11.6.2. But the same function is already used for checking all initial PDU SESSION ESTABLISHMENT REQUEST messages to check for status ‘01’ (for UEs supporting the feature), according to 38.508-1 table 4.7.2-1. |
| Summary of change | Parameterise this function to be able to check for PS data status 01 (=”deactivated”) or 02 (=”activated”) Note : Default value chosen as “01” to avoid change in f_Check_NG_PDUSessionEstablishmentReq() |
| TTCN module | NG_NASTemplateFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_ExtdProtocolConfigOptions_CheckPSDataOff(template (omit) ExtdProtocolConfigOptions p_Pco) return boolean { var template (omit) octetstring v_Content := f_ExtdProtocolConfigOptions_Get(p_Pco, '0017'O); var boolean v_Result := false; if (isvalue(v_Content)) { if (lengthof(v_Content) > 0) { v_Result := match ('02'O, v_Content); //@sic R5s220424 sic@ } } return v_Result; }

After Change

function f_ExtdProtocolConfigOptions_CheckPSDataOff(template (omit) ExtdProtocolConfigOptions p_Pco, O1_Type p_Status:=’01’O) return boolean { var template (omit) octetstring v_Content := f_ExtdProtocolConfigOptions_Get(p_Pco, '0017'O); var boolean v_Result := false; if (isvalue(v_Content)) { if (lengthof(v_Content) > 0) { v_Result := match (p_Status, v_Content); //@sic R5s220424 sic@ } } return v_Result; }

## Change 2

| Function name | f_TC_11_6_1_NR5GC_IMS1 |
| --- | --- |
| Reason for change | A coordination send trigger is missing to call the postamble function. |
| Summary of change | A coordination send trigger is added |
| TTCN module | PS_Data_OFF_NR5GC_IMS.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently |

Before Change

function f_TC_11_6_1_NR5GC_IMS1()runs on IMS_PTC { … … //@siclog "Steps 23-27" siclog@ //Steps 1-2 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.17.2.2-1 take place, f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_6_1_NR5GC_IMS1()runs on IMS_PTC

{

…

…

//@siclog "Steps 23-27" siclog@

//Steps 1-2 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.17.2.2-1 take place,

f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite);

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_IpcanReleaseWithOptionalDeregistration();

}

MCC160 Comments

function fl_TC_11_6_1_Body() runs on NR5GC_PTC

{

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo;

//Main test body

fl_TC_11_6_x_Body();

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8 - 12");

v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN);

//@siclog "Steps 13-15" siclog@

f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); // @sic R5-223441 sic@ // @sic R5s220718 sic@

//Receive coord from IMS PTC that call is setup

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

//Now release the call

f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall);

//Postamble

//Removed wait for Trigger  @sic R5s220828 sic@

} //End of test body of TC_11_6_1

## Change 3

| Function name | fl_TC_11_6_x_Body |
| --- | --- |
| Reason for change | Following implementation of change 2.3 of R5s220424, the function f_ExtdProtocolConfigOptions_CheckPSDataOff() is changed to check for status ‘02’ (=”activated”) instead of status ‘01’ (=”deactivated”), to check the PDU SESSION MODIFICATION REQUEST at step 2 of 11.6.2. But the same function is already used for checking all initial PDU SESSION ESTABLISHMENT REQUEST messages to check for status ‘01’ (for UEs supporting the feature), according to 38.508-1 table 4.7.2-1. |
| Summary of change | Pass status “02” (= “activated”) into the function f_ExtdProtocolConfigOptions_CheckPSDataOff(), see change 1 |
| TTCN module | PS_Data_OFF_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_11_6_x_Body() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) ExtdProtocolConfigOptions v_Pco := omit; var NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions; var integer i := 0; //@siclog "Steps 1" siclog@ //AT command to activate Data Off f_UT_TriggerPS_DataOff(UT, true); //@siclog "Steps 2" siclog@ // The UE transmits a PDU SESSION MODIFICATION REQ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu,v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session ID"); } v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; //Check if the UE has set the Data Off status to activated if (f_ExtdProtocolConfigOptions_CheckPSDataOff(v_Pco)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } if (isvalue(v_Pco.pco)) { v_ProtocolConfigOptions := valueof(v_Pco.pco); for (i := 0; i < lengthof(v_ProtocolConfigOptions); i := i + 1) { if (v_ProtocolConfigOptions[i].protocolID == '0017'O) { v_Pco.pco[i].protocolLength :='00'O; v_Pco.pco[i].content := omit; } } } //@siclog "Steps 3" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, v_GSM_MobilityInfo); v_MsgToSend.PiggybackedPduList[0].Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); } //End of test body of TC_11_6_x

After Change

function fl_TC_11_6_x_Body() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) ExtdProtocolConfigOptions v_Pco := omit; var NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions; var integer i := 0; //@siclog "Steps 1" siclog@ //AT command to activate Data Off f_UT_TriggerPS_DataOff(UT, true); //@siclog "Steps 2" siclog@ // The UE transmits a PDU SESSION MODIFICATION REQ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu,v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session ID"); } v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; //Check if the UE has set the Data Off status to activated if (f_ExtdProtocolConfigOptions_CheckPSDataOff(v_Pco, '02'O)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } if (isvalue(v_Pco.pco)) { v_ProtocolConfigOptions := valueof(v_Pco.pco); for (i := 0; i < lengthof(v_ProtocolConfigOptions); i := i + 1) { if (v_ProtocolConfigOptions[i].protocolID == '0017'O) { v_Pco.pco[i].protocolLength :='00'O; v_Pco.pco[i].content := omit; } } } //@siclog "Steps 3" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, v_GSM_MobilityInfo); v_MsgToSend.PiggybackedPduList[0].Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); } //End of test body of TC_11_6_x