3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220831

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2639 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Inter-RAT MDT test case 8.1.6.2.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 SON MDT test case 8.1.6.2.3 to the NR5GC_IRAT ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Rel-16 SON MDT test case 8.1.6.2.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-1, TS.38.523-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

2.4	Change 4	8

2.5	Change 5	10

2.6	Change 6	18

3	Branches executed	18

4	Execution Log Files	18

4.1	Mediatek MT6983	18

5	References	18

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.2.3 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	8.1.6.2.3

ATS Version:	iWD-TTCN3-B2020-09_D22wk23

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6983

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | templatecs_CoOrdEUTRASysInfo() |
| --- | --- |
| Reason for change | This test is required to pass information of PhysCellId to NR component as NR has to evaluate UE reported MeasResultNeighCellListEUTRA on UEInformationResponse at step15. So cs_CoOrdEUTRASysInfo template and associated its type record should be modified. |
| Summary of change | Added PhysCellID on template cs_CoOrdEUTRASysInfo |
| TTCN module | CommonIratDefs.ttcn |
| MCC160 Comment | Not needed. Used existing parameter CellId instead. |

Before Change:

type record CoOrd_EUTRASysInfo_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ integer Arfcn, integer DL_Bandwidth, // @sic R5s100871 Need to pass this through the co-ord msg sic@ EUTRA_HOInfo_Type HO optional, // extended for HO test cases integer CA_Tested_Integer optional, O3_Type PLMN optional, // the PLMN of the cell, defined in octets B28_Type CellId optional, // the EUTRA cell identity B16_Type TAC optional, integer freqBandIndicator optional, boolean UL_CA optional, // @sic R5-183277 sic@ integer UL_Arfcn optional // @sic R5-217955 sic@ }; template (value) CoOrd_EUTRASysInfo_Type cs_CoOrdEUTRASysInfo(integer p_Arfcn, integer p_DLBW, template (omit) EUTRA_HOInfo_Type p_HOInfo := omit, template (omit) integer p_CATested := omit, template (omit) O3_Type p_PLMN := omit, template (omit) B28_Type p_CellId := omit, template (omit) B16_Type p_TAC := omit, template (omit) integer p_FreqBandIndicator := omit, template (omit) boolean p_UL_CA := omit, template (omit) integer p_UL_Arfcn := omit) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ Arfcn := p_Arfcn, DL_Bandwidth := p_DLBW, HO := p_HOInfo, CA_Tested_Integer := p_CATested, PLMN := p_PLMN, CellId := p_CellId, TAC := p_TAC, freqBandIndicator := p_FreqBandIndicator, UL_CA := p_UL_CA, UL_Arfcn := p_UL_Arfcn // @sic R5-217955 sic@ //added PhysCellId here };

After Change:

type record CoOrd_EUTRASysInfo_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ integer Arfcn, integer DL_Bandwidth, // @sic R5s100871 Need to pass this through the co-ord msg sic@ EUTRA_HOInfo_Type HO optional, // extended for HO test cases integer CA_Tested_Integer optional, O3_Type PLMN optional, // the PLMN of the cell, defined in octets B28_Type CellId optional, // the EUTRA cell identity B16_Type TAC optional, integer freqBandIndicator optional, boolean UL_CA optional, // @sic R5-183277 sic@ integer UL_Arfcn optional, // @sic R5-217955 sic@ integer PhysCellId optional }; template (value) CoOrd_EUTRASysInfo_Type cs_CoOrdEUTRASysInfo(integer p_Arfcn, integer p_DLBW, template (omit) EUTRA_HOInfo_Type p_HOInfo := omit, template (omit) integer p_CATested := omit, template (omit) O3_Type p_PLMN := omit, template (omit) B28_Type p_CellId := omit, template (omit) B16_Type p_TAC := omit, template (omit) integer p_FreqBandIndicator := omit, template (omit) boolean p_UL_CA := omit, template (omit) integer p_UL_Arfcn := omit, template (omit) integer p_PhysCellId := omit) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ Arfcn := p_Arfcn, DL_Bandwidth := p_DLBW, HO := p_HOInfo, CA_Tested_Integer := p_CATested, PLMN := p_PLMN, CellId := p_CellId, TAC := p_TAC, freqBandIndicator := p_FreqBandIndicator, UL_CA := p_UL_CA, UL_Arfcn := p_UL_Arfcn, // @sic R5-217955 sic@ PhysCellId := p_PhysCellId };

  - Change 2

| Function name | Funtion f_TC_8_1_6_2_3_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | - Preamble on EUTRA not required, as prose expecting preamble on NR. - Need to power up EUTRA cell once UE is done with preamble on EUTRA cell, so wait for NR component is required. |
| Summary of change | - Commented out f_EUTRA38_NR5GC_PreambleAndSwitchOff - Added f_EUTRA_NR_WaitForCoOrd_Trigger |
| TTCN module | RRC_MDT_InterRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | - Accepted. - Accepted. |

Before Change:

function f_TC_8_1_6_2_3_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT MDT / Radio Link Failure / Reporting at E-UTRA Inter-RAT handover f_EUTRA38_IRAT_Init(c31); //System information combination 31 as defined in TS 36.508 [7] clause 4.4.3.1 is used in the E-UTRA cell. //initiate the power level f_EUTRA_CellInfo_InitMaxReferencePower( eutra_Cell1, -73); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA38_SetCellPower(eutra_Cell1, -73, tsc_NonSuitableOffCellRS_EPRE);// @sic R5-216499 R5-220052 sic@ f_EUTRA_TestBody_Set(true); fl_TC_8_1_6_2_3_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // UE now on NR f_EUTRA_ReleaseAllCells(); }

After Change:

import from EUTRA_CellInfoInit all; function f_TC_8_1_6_2_3_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT MDT / Radio Link Failure / Reporting at E-UTRA Inter-RAT handover f_EUTRA38_IRAT_Init(c31); //System information combination 31 as defined in TS 36.508 [7] clause 4.4.3.1 is used in the E-UTRA cell. //initiate the power level f_EUTRA_CellInfo_InitMaxReferencePower( eutra_Cell1, -73); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA38_SetCellPower(eutra_Cell1, -73, tsc_NonSuitableOffCellRS_EPRE);// @sic R5-216499 R5-220052 sic@ f_EUTRA_TestBody_Set(true); fl_TC_8_1_6_2_3_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // UE now on NR f_EUTRA_ReleaseAllCells(); }

  - Change 3

| Function name | function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | - System Co-ordination required to send EUTRA cell information like physcellID and frequency information to NR test body. - TAU process along with security activation required also need to Establishment of default and dedicated bearers. |
| Summary of change | - Added f_EUTRA_NR_SendSysinfoCoOrd - Added fl_TAU_N1ToS1 and commented out f_EUTRA38_TrackingAreaUpdateFromN1 |
| TTCN module | RRC_MDT_InterRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | The changes are made in function fl_TC_8_1_6_2_3_NR5GC_EUTRA_TestBody(). - Accepted in principle, but used existing parameter CellId. - Accepted conditional to prose CR agreement at RAN5#96e. |

Before Change:

function fl_TC_8_1_6_2_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var octetstring v_NR_HOCmd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1,tsc_RRC_TI_Def,-,-,noRrcConnectionRelease);// @sic R5-223273 sic@ f_EUTRA38_SetCellPower(eutra_Cell1, -85, tsc_NonSuitableOffCellRS_EPRE);// @sic R5-216499 R5-220052 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Power Level")); // @siclog "Step 7" siclog@ // the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4 v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF, v_SecurityParams.KASME, v_SecurityParams.NH); // 33.501 cl. 8.4.2 point 3: the target AMF shall also set the NCC to zero v_SecurityParams.NCC := 2; // @sic R5-212374 sic@ f_EUTRA_Security_Set (v_SecurityParams); // Send latest parameters to NR f_EUTRA38_Send_AuthParametersToNR(); f_EUTRA38_SendPduSessionInfoListToNR(); // Wait for NR to send back encoded RRC Reconfiguration message v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd"); SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def, false, nr, v_NR_HOCmd, omit))); // UE now moved to NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); }

After Change:

function fl_TC_8_1_6_2_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var octetstring v_NR_HOCmd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var NasCount_Type v_NasCountUL; // Send EUTRA frequency to NR //f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); v_CoOrd_SysInfo := cs_EUTRA_NR_CoOrdSysInfo_Eutra({f_EUTRA_CoOrdSysInfo(eutra_Cell1)}); //this one returns only dlarfcn and bw v_CoOrd_SysInfo.Eutra[0].PhysCellId := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1); //adds the phycellID to the coorData f_EUTRA_NR_SendSysinfoCoOrd(NR, v_CoOrd_SysInfo); // @siclog "Step 5" siclog@ //v_NasCountUL := f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1,tsc_RRC_TI_Def, omit, -, noRrcConnectionRelease); fl_TAU_N1ToS1(eutra_Cell1, tsc_RRC_TI_Def, "Step5", "Step6", omit); v_SecurityParams := f_EUTRA_Security_Get(); f_EUTRA38_SetCellPower(eutra_Cell1, -85, tsc_NonSuitableOffCellRS_EPRE);// @sic R5-216499 R5-220052 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Power Level")); // @siclog "Step 7" siclog@ // the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4 v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF, v_SecurityParams.KASME, v_SecurityParams.NH); // 33.501 cl. 8.4.2 point 3: the target AMF shall also set the NCC to zero v_SecurityParams.NCC := 2; // @sic R5-212374 sic@ f_EUTRA_Security_Set (v_SecurityParams); // Send latest parameters to NR f_EUTRA38_Send_AuthParametersToNR(); f_EUTRA38_SendPduSessionInfoListToNR(); // Wait for NR to send back encoded RRC Reconfiguration message v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd"); SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def, false, nr, v_NR_HOCmd, omit))); // UE now moved to NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); }

MCC160 Implementation

function fl_TC_8_1_6_2_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC

{

var octetstring v_NR_HOCmd;

var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get();

var template (value) B28_Type v_EUTRA_CellId;

var template (value) CoOrd_EUTRASysInfo_Type v_CoOrd_SysInfo := f_EUTRA_CoOrdSysInfo(eutra_Cell1);

// Send EUTRA frequency to NR

v_EUTRA_CellId := int2bit(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1),28);

v_CoOrd_SysInfo.CellId := v_EUTRA_CellId;

f_EUTRA_NR_SendSysinfoCoOrd(NR, cs_EUTRA_NR_CoOrdSysInfo_Eutra({v_CoOrd_SysInfo}));

…

}

  - Change 4

| Function name | function fl_TAU_N1ToS1 added |
| --- | --- |
| Reason for change | TAU process along with security activation added as prose expecting UE Tracking area updating / inter-system change from N1 mode to S1 mode in EMM-IDLE mode |
| Summary of change | Defined functionality of fl_TAU_N1ToS1. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#96e. |

Before Change:

function fl_TAU_N1ToS1 (EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, charstring p_StepNo1 := "", charstring p_StepNo2 := "", template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var template (value) RadioBearerList_Type v_DrbConfigList; var DRB_ToAddModList v_DRB_ToAddModList; var template (omit) SRB_ToAddModList v_SRB_ToAddModList := {cs_508_SRB2_ToAddMod_DEFAULT}; var integer i; v_NasInd := f_EUTRA38_TAUReqFromN1 (p_CellId, p_RRC_TI, -, -, p_CellIdPreviouslyCampedOn, -, p_FirstN1_S1Transition); // @sic R5s201062 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, p_StepNo1); // @sic R5-204399, R5s201465 sic@ get all DRBs transferred from NR v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); for (i:=0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DrbConfigList[i] := v_ExistingDRBs[i].SS_Config; } v_SecurityParams := f_EUTRA_Security_Get(); v_SecurityParams := f_EUTRA_RRC_ActivateSecurity(p_CellId, v_SecurityParams, v_NasInd.SecurityProtection.NasCount, v_DrbConfigList); // @sic R5s200878, R5-204399 sic@ f_EUTRA_Security_Set(v_SecurityParams); //Saving security parameters f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(p_CellId, v_NasInd, -, noRrcConnectionRelease, -, -, -, -,true); // @sic R5s220006 sic@ // @sic R5s201465 sic@ get all DRBs again in case more have been established v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); for (i:=0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := v_ExistingDRBs[i].DRB_ToAdd; } f_EUTRA_RRCConnectionReconfiguration_Send(p_CellId, // @sic R5s211690 sic@ p_RRC_TI, 0, v_SRB_ToAddModList, v_DRB_ToAddModList); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId, p_RRC_TI); }

  - Change 5

| Function name | function fl_TC_8_1_6_2_3_TestBody() |
| --- | --- |
| Reason for change | - Activated Security on EUTRA cell can be derived after establishing TAU on EUTRA cell so moved to step 7 . - DRB release and configuration is required once UE moved to EUTRA cell. - No need to bit to integer conversion on v_CoOrd_SysInfo.Eutra[0].CellId as it will return EUTRA_PhysCellId type. - PDu session modification not required - Parameters to be passed on got interchnaged on MeasResultNeighCellListEUTRA at step 14. As per proposed prose CR “set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;” and “set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the last radio link failure or handover failure;” and “set eutraReconnectCellId in reconnectCellId in VarRLF-Report of TS 38.331 [82] to the global cell identity and the tracking area code of the PCell;” and “timeConnFailure This field is used to indicate the time elapsed since the last HO initialization until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.” And “set the noSuitableCellFound in the VarRLF-Report to true;” - As procedure 4.5A.2C.2.2-1 is for “UE-requested PDU session modification after the first S1 to N1 mode change”, but in the preamble UE is already in state 3N-A. Therefore step 12 could be avoid. |
| Summary of change | - Commented f_EUTRA_NR_WaitForCoOrd_Security (EUTRA) while declaring variables and modified v_SecurityRx it with _EUTRA_NR_WaitForCoOrd_Security (EUTRA) at step 7. - added function for DRB release and config. - Removed bit2int. - Swaped v_ARFCN_ValueEUTRA and v_EUTRA_CellId. - Modified parameters for template cr_NR_Rlf_Report. - Commented f_NR5GC_IRAT_S1ToN1_SessionModification. |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment | The related module is RRC_MDT_InterRAT_NR5GC_NR.ttcn - Accepted. - Accepted. - Not needed. According to the CoOrd_EUTRASysInfo_Type definition, v_CoOrd_SysInfo.Eutra[0].CellId should be B28_Type, instead of int type. - Accepted conditional to prose CR agreement at RAN5#96e. - Accepted conditional to prose CR agreement at RAN5#96e.The usaged of v_C_RNTI_beforeRLF need to be clarified in the prose CR.The cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16 is inconsistent with the draft prose CR, since this will not handle the condition when the UE reported pci-arfcn-r16, please see the MCC160 implementation. - It seems a repetition of item 4. |

Before Change:

function fl_TC_8_1_6_2_3_TestBody() runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -90, tsc_NR_NonSuitableOffCellSSS_EPRE)};//@sic R5-216499 R5-220052 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1,tsc_NR_NonSuitableOffCellSSS_EPRE)};//@sic R5-216499 R5-220052 sic@ var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 0; //@sic R5-218335 sic@ var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1; var RSRP_Range v_B2_Threshold1 := 73; var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 58; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var default v_MyDefaultVar := null; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (omit) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) NR_RLF_Report_measResultNeighCells v_MeasResultNeighCells_r16; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (EUTRA); var PhysCellId v_PhysCellId_Cell1 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ARFCN_ValueNR v_SSB_Freq_Cell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); var template (present) UL_DCCH_Message v_RRCConnReconfigComplete; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer i; //Power levels of NR cell 1: Table 8.1.6.2.3.3.2-1, 8.1.6.2.3.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -90 ¦ FFS----//@sic R5-216499 R5-220052 sic@ * ------------------------------- * T1 ¦ off ¦ off * ------------------------------- * T2 ¦ -88 ¦ FFS //@sic R5-216499 R5-220052 sic@. * ------------------------------- */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for event B2 //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false))}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@siclog "Step 3" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1. f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId_Cell1, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)})); //@siclog "Step 4" siclog@ //The SS changes NR Cell1 and E-UTRA Cell 1 parameters according to the row "T1" in table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 5" siclog@ //Generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed on E-UTRA Cell 1. f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); // added function for DRB release and config //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //The SS transmits a MobilityFromEUTRACommand message on E-UTRA Cell 1. //get security context from EUTRAN f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, true); // this also sends octetstring to EUTRA and waits for the Complete message //@siclog "Step 8" siclog@ //Check: Does the UE transmit a RRCReconfigurationComplete message containing rlf-InfoAvailable on NR Cell 1? v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); v_RRCConnReconfigComplete:=cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-,cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF))); f_NR5GC_508RRC_InterRAT_HO_ToNR(nr_Cell1, v_PduSessionInfoList, v_SecurityRx,-,-,v_RRCConnReconfigComplete); //@siclog "Step 9" siclog@ //The UE transmits a ULInformationTransfer message and a REGISTRATION REQUEST message indicating "mobility registration updating"? is sent to update the registration of the actual tracking area. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 10-11" siclog@ //SS sends an DLInformationTransfer message and a REGISTRATION ACCEPT message containing a 5G-GUTI. //The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease, -, tsc_NR_RbId_SRB2); // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(nr_Cell1)); // make sure all the PDNs configured on EUTRA are changed in routing table for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { f_IP_ChangeDrbMappingNR(IP, nr_Cell1, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); } //@siclog "Step 12" siclog@ //The procedure in table 4.5A.2C.2.2-1 in TS 38.508-1 [4] is performed. f_NR5GC_IRAT_S1ToN1_SessionModification(nr_Cell1, v_PduSessionInfoList, -, tsc_NR_RbId_SRB2); deactivate(v_MyDefaultVar); //@siclog "Step 13" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Step 14" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-,?))) -> value v_ReceivedAsp; v_Nr_Rlf_Report_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; v_MeasResultNeighCells_r16:=cr_NR_RLF_Report_measResultNeighCells_Eutra_r16(cr_MeasResultNeighCellListEUTRA_1Entry(v_EUTRA_CellId,v_ARFCN_ValueEUTRA, f_EUTRA_Get_MeasQuantityResults())); if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(f_NR_CellInfo_GetRNTI(nr_Cell1), -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, cr_NR_RLF_Report_failedPCellId_PciArfcn_r16(v_PhysCellId_Cell1, v_SSB_Freq_Cell1), omit, omit, omit, ?, ?, rlf, t310_Expiry, -, omit)))// noSuitableCellFound-r16-omit? { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } }

After Change:

function fl_TC_8_1_6_2_3_TestBody() runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -90, tsc_NR_NonSuitableOffCellSSS_EPRE)};//@sic R5-216499 R5-220052 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1,tsc_NR_NonSuitableOffCellSSS_EPRE)};//@sic R5-216499 R5-220052 sic@ var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 0; //@sic R5-218335 sic@ var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1; var RSRP_Range v_B2_Threshold1 := 73; var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 58; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var default v_MyDefaultVar := null; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (omit) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) NR_RLF_Report_measResultNeighCells v_MeasResultNeighCells_r16; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx;// := f_EUTRA_NR_WaitForCoOrd_Security (EUTRA); var PhysCellId v_PhysCellId_Cell1 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ARFCN_ValueNR v_SSB_Freq_Cell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); var template (present) UL_DCCH_Message v_RRCConnReconfigComplete; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer i; var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); var RNTI_Value_Type v_C_RNTI_beforeRLF; var PLMN_Identity v_PLMN_Identity_Cell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_Cell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_Cell1 := f_NR_CellInfo_GetTAC (nr_Cell1); //Power levels of NR cell 1: Table 8.1.6.2.3.3.2-1, 8.1.6.2.3.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -90 ¦ FFS----//@sic R5-216499 R5-220052 sic@ * ------------------------------- * T1 ¦ off ¦ off * ------------------------------- * T2 ¦ -88 ¦ FFS //@sic R5-216499 R5-220052 sic@. * ------------------------------- */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := v_CoOrd_SysInfo.Eutra[0].PhysCellId; v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth); v_C_RNTI_beforeRLF := f_NR_CellInfo_GetRNTI(nr_Cell1); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for event B2 //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false))}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@siclog "Step 3" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1. f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId_Cell1, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)})); //@siclog "Step 4" siclog@ //The SS changes NR Cell1 and E-UTRA Cell 1 parameters according to the row "T1" in table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 5" siclog@ //Generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed on E-UTRA Cell 1. f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList); //let the SS know that there was OOS f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit); //recpn //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //The SS transmits a MobilityFromEUTRACommand message on E-UTRA Cell 1. v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (EUTRA); f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, true); // this also sends octetstring to EUTRA and waits for the Complete message //@siclog "Step 8" siclog@ //Check: Does the UE transmit a RRCReconfigurationComplete message containing rlf-InfoAvailable on NR Cell 1? v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); v_RRCConnReconfigComplete:=cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-,cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF))); f_NR5GC_508RRC_InterRAT_HO_ToNR(nr_Cell1, v_PduSessionInfoList, v_SecurityRx,-,-,v_RRCConnReconfigComplete); //@siclog "Step 9" siclog@ //The UE transmits a ULInformationTransfer message and a REGISTRATION REQUEST message indicating "mobility registration updating"? is sent to update the registration of the actual tracking area. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 10-11" siclog@ //SS sends an DLInformationTransfer message and a REGISTRATION ACCEPT message containing a 5G-GUTI. //The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease, -, tsc_NR_RbId_SRB2); // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(nr_Cell1)); // make sure all the PDNs configured on EUTRA are changed in routing table for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { f_IP_ChangeDrbMappingNR(IP, nr_Cell1, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); } //@siclog "Step 12" siclog@ //The procedure in table 4.5A.2C.2.2-1 in TS 38.508-1 [4] is performed. //f_NR5GC_IRAT_S1ToN1_SessionModification(nr_Cell1, v_PduSessionInfoList, -, tsc_NR_RbId_SRB2); deactivate(v_MyDefaultVar); //@siclog "Step 13" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Step 14" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-,?))) -> value v_ReceivedAsp; v_Nr_Rlf_Report_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; v_MeasResultNeighCells_r16:=cr_NR_RLF_Report_measResultNeighCells_Eutra_r16(cr_MeasResultNeighCellListEUTRA_1Entry(v_ARFCN_ValueEUTRA,v_EUTRA_CellId, f_EUTRA_Get_MeasQuantityResults())); if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_C_RNTI_beforeRLF, -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, (cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1, v_TAC_Cell1))), cr_eutraReconnectCellId_r16, ?, omit, -, ?, rlf, t310_Expiry, -, omit)))// noSuitableCellFound-r16-omit? { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } }

MCC160 Implementation:

function fl_TC_8_1_6_2_3_TestBody() runs on NR5GC_PTC { if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_C_RNTI_beforeRLF, -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1, v_TAC_Cell1)), cr_eutraReconnectCellId_r16, ?, omit, -, ?, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_C_RNTI_beforeRLF, -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, cr_NR_RLF_Report_failedPCellId_PciArfcn_r16(f_NR_CellInfo_GetPhysicalCellId(nr_Cell1), f_NR_CellInfo_GetFrequencySSB(nr_Cell1)), cr_eutraReconnectCellId_r16, ?, omit, -, ?, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } }

  - Change 6

| Function name | Added template cr_eutraReconnectCellId_r16 |
| --- | --- |
| Reason for change | Added Template to evaluate eutraReconnectCellId_r16 reprted by UE on reconnectCellId in UEInformationResponse. |
| Summary of change | Added cr_eutraReconnectCellId_r16 template with eutraReconnectCellId_r16 for any value |
| TTCN module | NR5GC_MDT_Specific_Templates.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

template NR_RLF_Report_reconnectCellId cr_eutraReconnectCellId_r16 := { /* @status APPROVED (NR5GC) */ eutraReconnectCellId_r16 := ? };

- Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

- Execution Log Files

  - Mediatek MT6983

The Mediatek MT6983UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_6_2_3_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_6_2_3_PIXIT.txt

- References

| [1] | R5s220832 - Supporting information for addition of NR5GC Inter-RAT MDT test case 8.1.6.2.3 in FR1 |
| --- | --- |