3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220840

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2645 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Prioritised bit rate is not set according to Table 7.1.1.3.5.3.1-1: Logical Channel Configuration Settings ( refer 38523-1 ) - The MAC testmode should be in Normal Mode for the UE to be brought to State 3A and then moved to Test Mode prior to the start of the Test Body. This will make ENDC and SA Test mode configurations in Line |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Priotised bit rate is set according to Table 7.1.1.3.5.3.1-1: Logical Channel Configuration Settings ( refer 38523-1 ) - MAC_TestModeInfo_NoHeaderManipulationDL_UL RB configuration is done before the testbody |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for NR testcase 7.1.1.3.5.NR5GC	2

2.1	Correction to the function function f_TC_7_1_1_3_5_NR5GC()	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 7.1.1.3.5.NR5GC

## Correction to the function function f_TC_7_1_1_3_5_NR5GC()

| Function name | function f_TC_7_1_1_3_5_NR5GC() |
| --- | --- |
| Reason for change | - Prioritised bit rate is not set according to Table 7.1.1.3.5.3.1-1: Logical Channel Configuration Settings ( refer 38523-1 ) - The MAC testmode should be in Normal Mode for the UE to be brought to State 3A and then moved to Test Mode prior to the start of the Test Body. This will make ENDC and SA Test mode configurations in Line |
| Summary of change | - Priotised bit rate is set according to Table 7.1.1.3.5.3.1-1: Logical Channel Configuration Settings ( refer 38523-1 ) - MAC_TestModeInfo_NoHeaderManipulationDL_UL RB configuration is done before the testbody |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_3_5_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR var DRB_Identity v_DRBId1 ; var DRB_Identity v_DRBId2 ; //@sic R5-201519 sic@ var UInt_Type v_LCID1; var UInt_Type v_LCID2; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList ; //@sic r5s190392 sic@ var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5s210862 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); //@sic R5s210862 sic@ //f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //@sic R5s210315 R5s210434 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(2); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; //@sic R5-201519 sic@ v_LCID1 := v_DRBId1 + 3; v_LCID2 := v_DRBId2 + 3; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID1,//@sic R5-201519 sic@ v_DRBId1, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(2, -, 7)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID2,//@sic R5-201519 sic@ v_DRBId2, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(1, -, 6)) ; v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config)}; f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, v_MAC_CellGroupConfig, //@sic R5s210862 sic@ v_DRBToAddModList, v_SS_Drb_ConfigList ); // f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); f_TC_7_1_1_3_5_NR_TestBody(v_DRBId1); f_NR_TestBody_Set(false); //Postamble // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_5_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR var DRB_Identity v_DRBId1 ; var DRB_Identity v_DRBId2 ; //@sic R5-201519 sic@ var UInt_Type v_LCID1; var UInt_Type v_LCID2; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList ; //@sic r5s190392 sic@ var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5s210862 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); //@sic R5s210862 sic@ //f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //@sic R5s210315 R5s210434 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(2); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; //@sic R5-201519 sic@ v_LCID1 := v_DRBId1 + 3; v_LCID2 := v_DRBId2 + 3; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID1,//@sic R5-201519 sic@ v_DRBId1, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(2, kBps0, 7)) ; //WA#WI=979664 v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID2,//@sic R5-201519 sic@ v_DRBId2, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(1, kBps0, 6)) ; //WA#WI=979664 v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; //WA#WI=979664 //@sic r5s190392 sic@ v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config)}; f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, v_MAC_CellGroupConfig, //@sic R5s210862 sic@ v_DRBToAddModList, v_SS_Drb_ConfigList ); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; //WA#WI=979664 f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //WA#WI=979664 f_NR_TestBody_Set(true); f_TC_7_1_1_3_5_NR_TestBody(v_DRBId1); f_NR_TestBody_Set(false); //Postamble // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };