3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220848

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2649 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC MAC test cases 7.1.1.3.8.1 and 7.1.1.3.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. In test case 7.1.1.3.8.1/2, for FR2 120Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 5, slot 0 for the UL grant scheduling to expect UL Data on subframe 5, slot 4 so that TTCN can control the subframe where the UL Grant is transmitted Similarly, for FR1 30Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 2, slot 0 for the UL grant scheduling to expect the UL Data on subframe 4, slot 0 so that TTCN can control the subframe where the UL Grant is transmitted 2. For call to f_NR_CellInfo_SetMAC_CellGroupConfig(), cell id parameter should be used instead of hardcoded cell 1 id. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed the subframe to 2 when SCS 30 is configured, and Changed the subframe to 5 when SCS 120 is configured Note : Similar change for 7.1.1.3.7 was agreed under R5s220122 - Use p_CellId for call to f_NR_CellInfo_SetMAC_CellGroupConfig |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.8.1.NR5GC, 7.1.1.3.8.1.ENDC, 7.1.1.3.8.2.NR5GC, 7.1.1.3.8.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_8_NR_Common |
| --- | --- |
| Reason for change | - In test case 7.1.1.3.8.1/2, for FR2 120Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 5, slot 0 for the UL grant scheduling to expect UL Data on subframe 5, slot 4 so that TTCN can control the subframe where the UL Grant is transmitted Similarly, for FR1 30Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 2, slot 0 for the UL grant scheduling to expect the UL Data on subframe 4, slot 0 so that TTCN can control the subframe where the UL Grant is transmitted - For call to f_NR_CellInfo_SetMAC_CellGroupConfig(), cell id parameter should be used |
| Summary of change | - Changed the subframe to 2 when SCS 30 is configured, and Changed the subframe to 5 when SCS 120 is configured Note : Similar change for 7.1.1.3.7 was agreed under R5s220122 - Use p_CellId for call to f_NR_CellInfo_SetMAC_CellGroupConfig |
| TTCN module | f_TC_7_1_1_3_8_NR_Common |
| MCC160 Comment | - Accepted as improvement - Accepted |

Before Change

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//UE power headroom reporting / SCell activation / DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; // Multi PR initialisation is taken care in Test body. f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //@sic R5s211385 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTMode_OFF);//@sic R5s211385 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(p_PCell, -,//@sic R5s211385 sic@ -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? //@sic R5-215675 sic@ f_NR5GC_AddMod_1SCell(p_PCell, p_SCell,-, true,-, false); // NR Cell1, NR Cell3/10 v_RRC_MsgRcvd:= false; v_PHR_Rcvd := false; v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP); //@siclog "Steps 4 preconfig" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); //@siclog "Steps 3" siclog@ //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10); ......

After Change

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//UE power headroom reporting / SCell activation / DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; var SubcarrierSpacing v_SCS;//KS R5s22xxx var SubFrameNumber_Type v_SF := 0;//KS R5s22xxx // Multi PR initialisation is taken care in Test body. f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //@sic R5s211385 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTMode_OFF);//@sic R5s211385 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(p_PCell, -,//@sic R5s211385 sic@ -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? //@sic R5-215675 sic@ f_NR5GC_AddMod_1SCell(p_PCell, p_SCell,-, true,-, false); // NR Cell1, NR Cell3/10 v_RRC_MsgRcvd:= false; v_PHR_Rcvd := false; v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP); //@siclog "Steps 4 preconfig" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138); f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_MAC_CellGroupConfig);//KS R5s22xxx v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); //@siclog "Steps 3" siclog@ //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. v_SCS := f_NR_CellInfo_GetSCS(p_PCell);//KS R5s22xxx if (v_SCS == kHz30) { f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_SubFrame(2), v_NR_ResourceAllocation, 10); } else { f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_SubFrame(5), v_NR_ResourceAllocation, 10); }//KS R5s22xxx