3GPP TSG-RAN5 Meeting #96-e	R5-2xxxxx

# Online, , 15th Aug 2022 – 26th Aug 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR |  | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MDT TC 8.1.6.1.4.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When Contention resolution is not successful, the UE will re attempt the procedure by performing another RACH procedure, in order to avoid this, its proposed to ignore the RACH procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the procedure in Step 1 to not sent contention resolution and ignore subsequent RACH procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 8	RRC

## 8.1	NR RRC

### 8.1.1	RRC connection management procedures

###### 8.1.6.1.4.8	Connection Establishment Failure / Logging and reporting / RACH failure report

8.1.6.1.4.8.1	Test Purpose (TP)

(1)

with { UE having previously failed the random access procedure where contention resolution was not successful for any of the transmitted preambles }

ensure that {

when { UE receives UEInformationRequest message with connEstFailReportReq set to true }

then { UE transmits UEInformationResponse message with a connEstFailReport including the perRAInfoList }

}

8.1.6.1.4.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.7, 5.7.10.3, 5.7.10.4, 5.7.10.5 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.10.3]

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; [TS 38.331, clause 5.7.10.5]

The UE shall set the content in ra-InformationCommon as follows:

…

1>	set the parameters associated to individual random-access attempt in the chronological order of attempts in the perRAInfoList as follows:

2>	if the random-access resource used is associated to a SS/PBCH block, set the associated random-access parameters for the successive random-access attempts associated to the same SS/PBCH block for one or more random-access attempts as follows:

3>	set the ssb-Index to include the SS/PBCH block index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnSSB to indicate the number of successive random-access attempts associated to the SS/PBCH block;

3>	for each random-access attempt performed on the random-access resource, include the following parameters in the chronological order of the random-access attempt:

4>	if the random-access attempt is performed on the contention based random-access resource and if raPurpose is not equal to 'requestForOtherSI', include contentionDetected as follows:

5>	if contention resolution was not successful as specified in TS 38.321 [6] for the transmitted preamble:

6>	set the contentionDetected to true;

5>	else:

6>	set the contentionDetected to false;

4>	if the random-access attempt is performed on the contention based random-access resource; or

4>	if the random-access attempt is performed on the contention free random-access resource and if the random-access procedure was initiated due to the PDCCH ordering:

5>	if the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above rsrp-ThresholdSSB:

6>	set the dlRSRPAboveThreshold to true;

5>	else:

6>	set the dlRSRPAboveThreshold to false;

2>	else if the random-access resource used is associated to a CSI-RS, set the associated random-access parameters for the successive random-access attempts associated to the same CSI-RS for one or more random-access attempts as follows:

3>	set the csi-RS-Index to include the CSI-RS index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnCSI-RS to indicate the number of successive random-access attempts associated to the CSI-RS.

[TS 38.331, clause 6.2.2]

–	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

…

UEInformationResponse-IEs field descriptions

logMeasReport This field is used to provide the measurement results stored by the UE associated to logged MDT.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE or RRC_IDLE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE or RRC_IDLE.

ra-Report This field is used to provide the list of RA reports that is stored by the UE for the past up to maxRAReport-r16 number of successful random access procedures.

rlf-Report This field is used to indicate the RLF report related contents.

…

RA-Report field descriptions

absoluteFrequencyPointA This field indicates the absolute frequency position of the reference resource block (Common RB 0).

cellID This field indicates the CGI of the cell in which the associated random access procedure was performed.

contentionDetected This field is used to indicate that contention was detected for the transmitted preamble in the given random access attempt or not. This field is not included when the UE performs random access attempt is using contention free random-access resources or when the raPurpose is set to requestForOtherSI.

csi-RS-Index This field is used to indicate the CSI-RS index corresponding to the random access attempt.

dlRSRPAboveThreshold This field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold (rsrp-ThresholdSSB in beamFailureRecoveryConfig in UL BWP configuration of UL BWP selected for random access procedure initiated for beam failure recovery; Otherwise, rsrp-ThresholdSSB in rach-ConfigCommon in UL BWP configuration of UL BWP selected for random access procedure.

locationAndBandwidth Frequency domain location and bandwidth of the bandwidth part associated to the random-access resources used by the UE.

numberOfPreamblesSentOnCSI-RS This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding CSI-RS.

numberOfPreamblesSentOnSSB This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding SS/PBCH block.

perRAAttemptInfoList This field provides detailed information about a random access attempt.

perRAInfoList This field provides detailed information about each of the random access attempts in the chronological order of the random access attempts.

perRACSI-RSInfoList This field provides detailed information about the successive random access attempts associated to the same CSI-RS.

perRASSBInfoList This field provides detailed information about the successive random access attempts associated to the same SS/PBCH block.

raPurpose This field is used to indicate the RA scenario for which the RA report entry is triggered. The RA accesses associated to Initial access from RRC_IDLE, transition from RRC-INACTIVE and the MSG3 based SI request are indicated using the indicator 'accessRelated'. The indicator beamFailureRecovery is used in case of beam failure recovery failure in the SpCell [3]. The indicator reconfigurationWithSync is used if the UE executes a reconfiguration with sync. The indicator ulUnSynchronized is used if the random access procedure is initiated in a SpCell by DL or UL data arrival during RRC_CONNECTED when the timeAlignmentTimer is not running in the PTAG or in a serving cell by a PDCCH order [3]. The indicator schedulingRequestFailure is used in case of SR failures [3]. The indicator noPUCCHResourceAvailable is used when the UE has no valid SR PUCCH resources configured [3]. The indicator requestForOtherSI is used for MSG1 based on demand SI request.

ra-InformationCommon This field is used to indicate the common random-access related information between RA-report and RLF-report. For RA report, this field is mandatory presented. For RLF-report, this field is optionally included when connectionFailureType is set to 'hof' or when connectionFailureType is set to 'rlf' and the rlf-Cause equals to 'randomAccessProblem' or 'beamRecoveryFailure'; otherwise this field is absent.

ssb-Index This field is used to indicate the SS/PBCH index of the SS/PBCH block corresponding to the random access attempt.

subcarrierSpacing Subcarrier spacing used in the BWP associated to the random-access resources used by the UE.

…

8.1.6.1.4.8.3	Test description

8.1.6.1.4.8.3.1	Pre-test conditions

System Simulator:

- NR Cell 1

- System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

- None.

Preamble:

- The UE is in state 1N-A on Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.8.3.2	Test procedure sequence

Table 8.1.6.1.4.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS is configured to not transmit MAC control element and ignore subsequent RACH attempts | - | - | - | - |
| 1A | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 1B | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 1C | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 1D | SS is configured to transmit MAC control element with matched UE Contention Resolution Identity | - | - | - | - |
| 2 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | NR RRC: Paging | - | - |
| 3-9 | Steps 2-8 of generic test procedure in TS 38.508-1 Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 10 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 11 | Check: Does the UE send a UEInformationResponse message with connEstFailReport including the perRAInfoList on NR Cell 1? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.4.8.3.3	Specific message contents

Table 8.1.6.1.4.8.3.3-1: UEInformationRequest (step 10, Table 8.1.6.1.4.8.3.2-1)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-32A with condition FAIL

Table 8.1.6.1.4.8.3.3-2: UEInformationResponse (step 11, Table 8.1.6.1.4.8.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Any allowed value |  |  |
| rsIndexResults-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16{ | 1 entry |  |  |
| PerRAInfo-r16[1] CHOICE { |  | entry 1 |  |
| perRASSBInfoList-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | 1 |  |  |
| numberOfPreamblesSentOnSSB-r16 | Any allowed value | total number of successive RA preambles sent by the UE before T300 expiry |  |
| perRAAttemptInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16{ | 1 entry |  |  |
| PerRAAttemptInfo-r16[x] SEQUENCE { |  | x ranges from 1 to numberOfPreamblesSentOnSSB-r16 |  |
| contentionDetected-r16 | true |  |  |
| dlRSRPAboveThreshold-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |