3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220868

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2658 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 7.1.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no specific requirement for the retxBSR-Timer to be configured as sf10240 for this test case as configuring such a large value would lead to un-necessary delay for a UE to trigger an SR based on expiry of the retxBSR-Timer. NOTE: A Prose CR R5-223422 is already raised for this testcase for NR5GC version. The NR5GC version of this testcase has this implemented, but ENDC was missed in TTCN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the retxBSR-Timer as per Table 7.1.3.4.1.3.1-4: MAC-CellGroupConfig for both step 8 and step 23 call flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 7.1.3.4.1	2

2.1	Correction to f_NR_PDCP_PSCellChange_InterCell_Common	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test case 7.1.3.4.1

## Correction to f_NR_PDCP_PSCellChange_InterCell_Common

| Function name | f_NR_PDCP_PSCellChange_InterCell_Common |
| --- | --- |
| Reason for change | There is no specific requirement for the retxBSR-Timer to be configured as sf10240 for this test case as configuring such a large value would lead to un-necessary delay for a UE to trigger an SR based on expiry of the retxBSR-Timer. NOTE: A Prose CR R5-223422 is already raised for this. |
| Summary of change | Set the retxBSR-Timer as per Table 7.1.3.4.1.3.1-4: MAC-CellGroupConfig for both step 8 and step 23 call flow. |
| TTCN module | 7_1_3\PDCP_ENDC_NR.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_NR_PDCP_PSCellChange_InterCell_Common(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, NR_PDCP_SS_State_Type p_NR_PDCP_State, template(value) PDCP_Config p_PDCP_Config, ServCellIndex p_TargetServCellIndex, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, //@sic R5s190844 sic@ template(omit) RLC_Config.am.ul_AM_RLC.t_PollRetransmit p_T_PollRetransmit:=omit //@sic R5-201219 sic@ ) runs on NR_BASE_PTC { var template (value)CellGroupConfig v_CellGroupConfig ; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var template (value) NR_RadioBearerList_Type v_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(p_NR_PDCP_State.DRB_Id)}; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( nr_Cell2, -); var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(p_SourceCellId)); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; if (isvalue(p_T_PollRetransmit)){ v_T_PollRetransmit := valueof(p_T_PollRetransmit); } v_RLC_BearerConfigList:= f_NR_ReconfigureRLC_AM(p_NR_PDCP_State.DRB_Id, -, v_T_PollRetransmit, -, -, -, -, f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(p_SourceCellId)), -, true_); //@sic R5-195338 sic@ //@siclog "Step 8 or 23 " siclog@ //The SS requests UE to make a handover to target cell v_RadioBearerList := {f_NR_GetRadioBearerConfig_FullAM(p_TargetCellId, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode)}; v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.Rlc.Rb.AM.Tx.R15.t_PollRetransmit := v_T_PollRetransmit; //@sic R5-201219 sic@ v_RLC_BearerConfigList[0].rlc_Config.am.ul_AM_RLC.t_PollRetransmit := v_T_PollRetransmit; v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(p_TargetCellId, v_MAC_CellGroupConfig); f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList); v_CellGroupConfig := f_NR_GetCellGroupConfigHO (p_TargetCellId, v_RLC_BearerConfigList,p_RNTI_Value); //@sic R5-200063 sic@ v_CellGroupConfig.spCellConfig.servCellIndex := p_TargetServCellIndex; v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, false, -, p_PDCP_Config);//@sic R5-200063 sic@ // Save new C-RNTI in cell configuration for p_TargetCellId f_NR_CellInfo_SetRNTI(p_TargetCellId, p_RNTI_Value);//@sic R5s190844 sic@ f_NR_SS_C_RNTI_Config(p_TargetCellId, p_RNTI_Value); f_NR_SS_RachProcedureConfigHO(p_TargetCellId, v_RachProcedureConfig, -, v_CellGroupConfig.spCellConfig.reconfigurationWithSync.rach_ConfigDedicated); //@siclog "Step 9 or 24" siclog@ //The UE transmitsNR RRCReconfigurationComplete message 2. f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_CellGroupConfig, omit, -, -, reconfigurationWithSync_CheckRACH, p_TargetCellId); //@sic R5s190541,R5-200063 sic@ //Release DRB f_NR_SS_CommonRadioBearerConfig(p_SourceCellId, v_RadioBearerToReleaseList); }

After Change:

function f_NR_PDCP_PSCellChange_InterCell_Common(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, NR_PDCP_SS_State_Type p_NR_PDCP_State, template(value) PDCP_Config p_PDCP_Config, ServCellIndex p_TargetServCellIndex, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, //@sic R5s190844 sic@ template(omit) RLC_Config.am.ul_AM_RLC.t_PollRetransmit p_T_PollRetransmit:=omit //@sic R5-201219 sic@ ) runs on NR_BASE_PTC { var template (value)CellGroupConfig v_CellGroupConfig ; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var template (value) NR_RadioBearerList_Type v_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(p_NR_PDCP_State.DRB_Id)}; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( nr_Cell2, -); var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(p_SourceCellId)); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; if (isvalue(p_T_PollRetransmit)){ v_T_PollRetransmit := valueof(p_T_PollRetransmit); } v_RLC_BearerConfigList:= f_NR_ReconfigureRLC_AM(p_NR_PDCP_State.DRB_Id, -, v_T_PollRetransmit, -, -, -, -, f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(p_SourceCellId)), -, true_); //@sic R5-195338 sic@ //@siclog "Step 8 or 23 " siclog@ //The SS requests UE to make a handover to target cell v_RadioBearerList := {f_NR_GetRadioBearerConfig_FullAM(p_TargetCellId, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode)}; v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.Rlc.Rb.AM.Tx.R15.t_PollRetransmit := v_T_PollRetransmit; //@sic R5-201219 sic@ v_RLC_BearerConfigList[0].rlc_Config.am.ul_AM_RLC.t_PollRetransmit := v_T_PollRetransmit; v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; v_MAC_CellGroupConfig.bsr_Config := valueof(cs_NR_BSR_ConfigDef(infinity_, sf80)); //WA#WI= 982424 f_NR_CellInfo_SetMAC_CellGroupConfig(p_TargetCellId, v_MAC_CellGroupConfig); f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList); v_CellGroupConfig := f_NR_GetCellGroupConfigHO (p_TargetCellId, v_RLC_BearerConfigList,p_RNTI_Value); //@sic R5-200063 sic@ v_CellGroupConfig.spCellConfig.servCellIndex := p_TargetServCellIndex; v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, false, -, p_PDCP_Config);//@sic R5-200063 sic@ // Save new C-RNTI in cell configuration for p_TargetCellId f_NR_CellInfo_SetRNTI(p_TargetCellId, p_RNTI_Value);//@sic R5s190844 sic@ f_NR_SS_C_RNTI_Config(p_TargetCellId, p_RNTI_Value); f_NR_SS_RachProcedureConfigHO(p_TargetCellId, v_RachProcedureConfig, -, v_CellGroupConfig.spCellConfig.reconfigurationWithSync.rach_ConfigDedicated); //@siclog "Step 9 or 24" siclog@ //The UE transmitsNR RRCReconfigurationComplete message 2. f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_CellGroupConfig, omit, -, -, reconfigurationWithSync_CheckRACH, p_TargetCellId); //@sic R5s190541,R5-200063 sic@ //Release DRB f_NR_SS_CommonRadioBearerConfig(p_SourceCellId, v_RadioBearerToReleaseList); }