3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220890

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4678 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 8.3.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-07-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The reconfiguration (Step 9) message changes the measurement gap to gp30. However, According to the 36508.Table 4.6.6-1 MeasConfig-DEFAULT, if the UE supports the IntraFreq CE need for Gaps(pc_intraFreq-CE-NeedForGaps is TRUE), the measGapConfig should be set to MeasGapConfig-CE(gp20) Hence, a TTCN correction is required at Step 9 A Prose CR will be raised for this |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the pc_intraFreq-CE-NeedForGaps is TRUE, the measGapConfig should be set to MeasGapConfig-CE(gp20). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN will not be in alignment with Prose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	f_TC_8_3_1_10	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## f_TC_8_3_1_10

| Template name | f_TC_8_3_1_10 |
| --- | --- |
| Reason for change | The reconfiguration (Step 9) message changes the measurement gap to gp30. However, According to the 36508.Table 4.6.6-1 MeasConfig-DEFAULT, if the UE supports the IntraFreq CE need for Gaps(pc_intraFreq-CE-NeedForGaps is TRUE), the measGapConfig should be set to MeasGapConfig-CE(gp20) Hence, a TTCN correction is required at Step 9 |
| Summary of change | For the pc_intraFreq-CE-NeedForGaps is TRUE, the measGapConfig should be set to MeasGapConfig-CE(gp20). |
| TTCN module | RRC_Measurement.ttcn |
| MCC160 Comment | Accepted, but Prose CR needed |  |

Before Change:

function f_TC_8_3_1_10_EUTRA() runs on EUTRA_PTC { <<SKIPPED CODE>> //@siclog "Step 9-10" siclog@ //The SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 3 and //receives an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 3. //@sic R5s110743 sic@ f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell3, //@sic R5s100516 Change 1 sic@ cs_RRCConnectionReconfiguration_ResumeInterFreqMeas(v_RRC_TI)); //@siclog "Step 11" siclog@ //The SS changes the cell-specific reference signal levels of Cell 1 and Cell 3 and switches "On" Cell 12 according to the row "T3" in table 8.3.1.10.3.2-1. //@sic R5-106572, correct description sic@ v_CellPowerList := { cs_CellPower(eutra_Cell1, v_T3_RS_EPRE_Cell1), cs_CellPower(eutra_Cell3, v_T3_RS_EPRE_Cell3), cs_CellPower(eutra_Cell12, v_T3_RS_EPRE_Cell12) //@sic R5-106572 sic@ }; f_EUTRA_SetCellPowerList(v_CellPowerList); <<SKIPPED CODE>> }

After Change:

function f_TC_8_3_1_10_EUTRA() runs on EUTRA_PTC { <<SKIPPED CODE>> //@siclog "Step 9-10" siclog@ //The SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 3 and //receives an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 3. //@sic R5s110743 sic@ //WI#946495 if(pc_intraFreq_CE_NeedForGaps) { f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell3, //@sic R5s100516 Change 1 sic@ cs_RRCConnectionReconfiguration_ResumeInterFreqMeas(v_RRC_TI, cs_508_MeasGapConfig_CE)); }else { f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell3, //@sic R5s100516 Change 1 sic@ cs_RRCConnectionReconfiguration_ResumeInterFreqMeas(v_RRC_TI)); } //@siclog "Step 11" siclog@ //The SS changes the cell-specific reference signal levels of Cell 1 and Cell 3 and switches "On" Cell 12 according to the row "T3" in table 8.3.1.10.3.2-1. //@sic R5-106572, correct description sic@ v_CellPowerList := { cs_CellPower(eutra_Cell1, v_T3_RS_EPRE_Cell1), cs_CellPower(eutra_Cell3, v_T3_RS_EPRE_Cell3), cs_CellPower(eutra_Cell12, v_T3_RS_EPRE_Cell12) //@sic R5-106572 sic@ }; f_EUTRA_SetCellPowerList(v_CellPowerList); <<SKIPPED CODE>> }