3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220923

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2662 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS Fallback test case 11.1.1a in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC_IRAT TC 11.1.1a to the NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 11.1.1a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	Mediatek MT6879	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.1a which is part of the NR5GC_IRAT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	11.1.1a

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Mediatek MT6879

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_11_1_1a_NR5GC_IMS1() |
| --- | --- |
| Reason for change | - After redirection to LTE from NR UE will be sending all SIP msgs on EUTRA and it will include access type '3GPP-E-UTRAN' on pAccessNetworkInfo. So TTCN should update RAN type for E_UTRAN for upcoming SIPs.. - There is extra waiting command for EUTRA co-ordination. So TTCN is keep waitng to trigger MO call release. |
| Summary of change | - Added function to set RAN type as EUTRAN. - Commented f_IMS_IPCAN_WaitForTrigger. |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment | - Accepted - Rejected. This corresponds to the trigger sent from the EUTRA PTC to trigger the call release: f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(v_EPSBearerId, 1)); // @sic R5s220448 sic@ |

Before Change:

function f_TC_11_1_1a_NR5GC_IMS1()runs on IMS_PTC { // MO MMTEL enhanced voice service call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; fl_IMSFallback_PreambleRegistrations(); // @siclog "Step 10 - 14" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3 - 6" siclog@ 100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_1a_Step2_5(v_InviteRequestWithSdp); f_Delay(0.5); // short delay to ensure last msg (DL) has been transmitted & Ack'd f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // changed RAN type here f_IMS_MOCallSetup_NR5GC_A_4_1a_Step6_7(v_InviteRequestWithSdp); // @siclog "Step 9 - 11" siclog@ 180 Ringing, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_1_Step8_10(v_InviteRequestWithSdp); // @siclog "Step 12 - 13" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Table 11.1.1a.3.2-2 Step 7"); // @siclog "Step 29 - 33" siclog@ f_IMS_IPCAN_WaitForTrigger(OtherIPCAN); f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); // Wait for indication RAT has changed to NR IPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); // This is required as re-using the same TC body as 11.1.1 f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change:

function f_TC_11_1_1a_NR5GC_IMS1()runs on IMS_PTC { // MO MMTEL enhanced voice service call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; fl_IMSFallback_PreambleRegistrations(); // @siclog "Step 10 - 14" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3 - 6" siclog@ 100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_1a_Step2_5(v_InviteRequestWithSdp); f_Delay(0.5); // short delay to ensure last msg (DL) has been transmitted & Ack'd f_IMS_IPCAN_SendCoOrdMsg(IPCAN); OtherIPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); f_IMS_MOCallSetup_NR5GC_A_4_1a_Step6_7(v_InviteRequestWithSdp); // @siclog "Step 9 - 11" siclog@ 180 Ringing, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_1_Step8_10(v_InviteRequestWithSdp); // @siclog "Step 12 - 13" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Table 11.1.1a.3.2-2 Step 7"); // @siclog "Step 29 - 33" siclog@ //f_IMS_IPCAN_WaitForTrigger(OtherIPCAN); f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); // Wait for indication RAT has changed to NR IPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); // This is required as re-using the same TC body as 11.1.1 f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

# Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Mediatek MT6879

The Mediatek MT6879 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_11_1_1a_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_11_1_1a_PIXIT.txt

# References

| [1] | R5s220924- Addition of EPS Fallback test case 11.1.1a in FR1 |
| --- | --- |